3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2207230

# Online Meeting, 17th - 29th Augest, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0505 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on inter-RAT handover from E-UTRA to NR for RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sequans Communications, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2022-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An issue was discussed for inter-RAT handover from E-UTRA to NR of RedCap UEs, where the source eNB may not support RedCap, and therefore cannot identify RedCap UEs nor have information on whether neighbour cells support RedCap. The following agreements were achieved in previous RAN2 meetings. RAN2#116bis-e Agreements online: - For the LTE to NR handover, in case the target NR cell is a legacy cell, the RedCap UE should trigger RRC re-establishment procedure. FFS any specification impact or purely leave to implementation. RAN2#118e Agreements: - Network implementation should avoid handover attempts from source eNB to legacy gNB that does not support RedCap. FFS is specific UE behaviour should also be specified Proposal 16 UE behaviour is not specified to address the case for inter-RAT mobility from LTE to NR for RedCap UEs. - Huawei thinks this will not cause any NBC issue by some UE options, even it is introduced in next meeting. We suggest to postpone this issue to next meeting. And no agreement on this for now. - QC, Oppo, Apple think we need to take a decision - VDF thinks we need a sentence on the UE behaviour. - Come back in the next meeting to check whether a different note in Stage2 can be agreed. - No Stage-3 impact will be considered because of this. The above agreements should be captured in TS 38.300. |
| Summary of change: |  | Add a NOTE to clarify that the network implementation should avoid handover attempts from source eNB to legacy gNB not supporting RedCap and the RedCap UE triggers RRC re-establishment in case the target NR cell is a legacy cell. Impact analysis Impacted 5G architecture options: NR standalone Impacted functionality: Inter-RAT handover from E-UTRA to NR Inter-operability: If the UE is implemented according to this CR but the network is not, there is no inter-operability issue forseen. If the network is implemented according to this CR but the UE is not, there is no inter-operability issue forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap UEs may be handed over from a source eNB to a target gNB which does not support RedCap. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 16.13	Support of Reduced Capability (RedCap) NR devices

### 16.13.3	Identification, access and camping restrictions

A RedCap UE can be identified by the network during Random Access procedure via MSG3/MSGA from a RedCap specific LCID(s) and optionally via MSG1/MSGA (PRACH occasion or PRACH preamble). For RedCap UE identification via MSG1/MSGA, RedCap specific Random Access configuration may be configured by the network. For MSG3/MSGA, a RedCap UE is identified by the dedicated LCID(s) indicated for CCCH identification (CCCH or CCCH1) regardless whether RedCap specific Random Access configuration is configured by the network.

RedCap UEs with 1 Rx branch and 2 Rx branches can be allowed separately via system information. In addition, Half-Duplex FDD RedCap UEs can be allowed via system information. A RedCap specific IFRI can be provided in SIB1, when absent, RedCap UEs access is not allowed. Information on which frequencies RedCap UE access is allowed can be provided in system information.

NOTE:	It is up to the E-UTRA network implementation, if possible, to avoid handover attempts of a RedCap UE to a target NR cell not supporting RedCap. If a RedCap UE determines that the target NR cell does not support RedCap, by considering the above configuration in SIB1 of the target cell, the UE is expected to initiate the connection re-establishment procedure.

END OF CHANGE