3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2207240

Online, August 17 - August 26, 2022

Agenda item:	6.10.2.1

Source:	Samsung

Title:	Discussion on TA report

WID/SID:	NR_NTN_solutions-Core - Release 17

Document for:	Discussion and Decision

# Introduction

In this document, we discuss some remaining issues on TA report.

# Discussion

## Dedicated SR configuration

In the past meetings, RAN2 has agreed that SR for TA report is configurable, and timingAdvanceSR is the parameter to enable or disable SR for TA report. Then, in the last RAN2 meeting whether to support dedicated SR configuration for TA report when timingAdvanceSR is enabled was discussed but not concluded. The options are summarized as follows.

- Option 1: dedicated SR configuration for TA report is not supported, i.e., the UE can use any SR configuration; if no SR configuration is provided, the UE initiates random access.

- Option 2: dedicated SR configuration for TA report is supported

  - Option 2-1: the UE applies the dedicated SR configuration for TA report if the dedicated SR configuration for TA report is provided; if no dedicated SR configuration for TA report is provided, the UE initiates RA procedure.

  - Option 2-2: the UE applies the dedicated SR configuration for TA report if the dedicated SR configuration for TA report is provided; if no dedicated SR configuration for TA report is provided, the UE uses any SR configuration; if no SR configuration is provided, the UE initiates random access.

In our view, all options are feasible. Option 1 is slightly preferred with smallest specification impacts among all options. For Option 2, dedicated SR configuration has always to be provided to guarantee dedicated PUCCH resource for TA report triggered SR for a better performance than Option 1; otherwise, one can expect the same performance as compared to Option 1 or even worse performance by Option 2-1 when RA is triggered due to no dedicated SR configuration provided. From another perspective, currently only SR for BFR and SR for consistent LBT failure are provided with dedicated SR configuration. Delay in TA report may have impact on timely scheduling, but does not directly impact radio link availability as BFR and LBT. So dedicated SR configuration for TA report may not be so strongly motivated.

Proposal 1: For TA report triggered SR, any SR configuration can be used, dedicated SR configuration for TA report is not needed.

## Outdated TA at NW

In the last RAN2 meeting, the issue of outdate TA at NW due to unsuccessful TA report MAC CE transmission was raised. Currently, if TA report is enabled, the UE reports TA during Random Access due to RRC connection establishment, RRC connection reestablishment, RRC connection resume, and Handover. In RRC_CONNECTED state, the UE reports TA when the variation between the current full TA and the last reported value is equal or larger than a threshold. However, the TA information at NW side can be outdated due to transmission failure of TA report MAC CE in HARQ process with feedback disabled, or due to UE does not have valid assistance information e.g. common TA. The outdated TA may lead to scheduling failure if there is DL data to transmit. In this case, NW-requested TA report can be beneficial. There are several solutions to this issue.

- Option 1: UE always sends TA report MAC CE using UL HARQ processes with feedback enabled.

- Option 2: NW triggers TA report in RACH by intra-cell handover, i.e., handover to the current serving cell

- Option 3-1: NW requests TA report in RACH by PDCCH order, and ta-Report enabled in SIB19 is used to indicate TA report is requested in RACH by PDCCH order

- Option 3-2: NW requests TA report in RACH by PDCCH order, and an indication in DCI is used to indicate TA report is requested in RACH by PDCCH order

- Option 4: NW requests TA report by MAC CE.

- Option 5: NW requests TA report by an indication in DCI.

- Option 6: NW requests TA report by UE information procedure.

By Option 1, additional restriction is added to TA report MAC CE transmission. Option 2 reuses the RACH triggered by HO to request TA report, which have no specification impact. TA report in PDCCH ordered RACH mimics legacy operation of PDCCH ordered RACH for re-sync when timeAlignmentTimer is expired. Then for Option 3-1, UE needs to report TA in every PDCCH ordered RACH, while by Option 3-2 UE only reports TA in PDCCH ordered RACH when there is TA report indication in DCI. For Option 4, a new MAC CE needs to be introduced. Option 5 involves RAN1, so does Option 3-2. Option 6 is similar to coarse UE location report.

Proposal 2: RAN2 to discuss the listed solutions to outdated TA issue:

•	Option 1: UE always sends TA report MAC CE using UL HARQ processes with feedback enabled.

•	Option 2: NW triggers TA report in RACH by intra-cell handover, i.e., handover to the current serving cell

•	Option 3-1: NW requests TA report in RACH by PDCCH order, and ta-Report enabled in SIB19 is used to indicate TA report is requested in RACH by PDCCH order

•	Option 3-2: NW requests TA report in RACH by PDCCH order, and an indication in DCI is used to indicate TA report is requested in RACH by PDCCH order

•	Option 4: NW requests TA report by MAC CE.

•	Option 5: NW requests TA report by an indication in DCI.

•	Option 6: NW requests TA report by UE information procedure.

# Conclusion

Based on the discussion, we have the following proposal.

Proposal 1: For TA report triggered SR, any SR configuration can be used, dedicated SR configuration for TA report is not needed.

Proposal 2: RAN2 to discuss the listed solutions to outdated TA issue:

•	Option 1: UE always sends TA report MAC CE using UL HARQ processes with feedback enabled.

•	Option 2: NW triggers TA report in RACH by intra-cell handover, i.e., handover to the current serving cell

•	Option 3-1: NW requests TA report in RACH by PDCCH order, and ta-Report enabled in SIB19 is used to indicate TA report is requested in RACH by PDCCH order

•	Option 3-2: NW requests TA report in RACH by PDCCH order, and an indication in DCI is used to indicate TA report is requested in RACH by PDCCH order

•	Option 4: NW requests TA report by MAC CE.

•	Option 5: NW requests TA report by an indication in DCI.

•	Option 6: NW requests TA report by UE information procedure.