3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2207241

Online, August 17 - August 26, 2022

Agenda item:	6.10.2.1

Source:	Samsung

Title:	Discussion on remaining MAC issues

WID/SID:	NR_NTN_solutions-Core - Release 17

Document for:	Discussion and Decision

# Introduction

In this document, we discuss some remaining MAC issues.

# Discussion

## Blind Msg3 retransmission

Blind Msg3 retransmission is supported by specific operation of ra_ContentionResolutionTimer for NTN. The current MAC specifies that the UE starts or restarts the ra-ContentionResolutionTimer in the first symbol after the end of the Msg3 transmission plus the UE-gNB RTT. This means the UE cannot monitor the PDCCH for blind Msg3 retransmission grant for UE-gNB RTT after the first Msg3 transmission. The remaining issue from the last meeting is how to ensure blind Msg3 retransmission is possible for the first Msg3 transmission.

In order to monitor PDCCH for potential additional retransmission grants to increase Msg3 transmission reliability, one way is to start the ra-ContentionResolutionTimer in the first symbol after the end of the Msg3 transmission. However, in this case the ra-ContentionResolutionTimer could expire during UE-gNB RTT so that UE cannot receive Msg4 in the contention resolution window. In our view, it is reasonable that the ra-ContentionResolutionTimer is started or restarted after an offset of UE-gNB RTT, which is aligned for all Msg3 transmissions. It is inevitable that the UE cannot monitor PDCCH during the offset of UE-gNB RTT when ra-ContentionResolutionTimer is not running, which is not only for the first Msg3 but also the case for the later Msg3. The UE only need to monitor PDCCH for blind Msg3 retransmission grant in a best effort manner when the ra-ContentionResolutionTimer is running. Furthermore, in order to have high reliability of the Msg3 transmission, Msg3 repetition can be applied if it’s configured in SIB and the number of repetitions is indicated in Msg2.

Proposal 1: No change to the current MAC specification on ra-ContentionResolutionTimer for the first Msg3 transmission.

## Maintenance of UL Synchronization

In the current RRC and MAC specification, the following is specified in TS38.331 and T38.321.

TS 38.331

5.2.2.6 T430 expiry

The UE shall:

1> if in RRC_CONNECTED:

2> inform lower layers that UL synchronisation is lost;

2> acquire SIB19 as defined in clause 5.2.2.3.2;

2> upon successful acquisition of SIB19:

3> inform lower layers that UL synchronisation is obtained;

TS 38.321

5.2a     Maintenance of UL Synchronization

The MAC entity shall:

1> if an indication of Serving Cell uplink synchronization has been received from upper layers (see clause 5.2.2.6 of TS 38.331 [5]):

2> allow uplink transmission on the corresponding Serving Cell.

1> if an indication of Serving Cell uplink synchronization loss is received from upper layers:

2> flush all HARQ buffers;

2> not perform any uplink transmission on the corresponding Serving Cell

According to TS 38.331, the UL synchronization indication is per T430 timer, while according to TS 38.321, the UL synchronization indication obtained from the upper layer is per serving cell.

Observation 1: In TS 38.331, the UL synchronization indication is per T430 timer, while according to TS 38.321, the UL synchronization indication obtained from the upper layer is per serving cell.

In our view, one T430 timer can be used for multiple serving cells from the same satellite with the epoch time and validity duration aligned in SIB19. On the other hand, for the MAC entity, although DC/CA has not been discussed for NTN, the uplink synchronization indication obtained from the upper layers should be for a group of cells, e.g., all serving cells of the cell group for the MAC entity. For future-proof purpose, the UL synchronization indication informed by RRC and the UL synchronization indication obtained by MAC entity should be aligned in the way that the UL synchronization indication is per cell group. Therefore, the following text for MAC specification is suggested. TP is attached in the Appendix.

5.2a     Maintenance of UL Synchronization

The MAC entity shall for each Serving Cell:

1> if an indication of Serving Cell uplink synchronization has been received from upper layers (see clause 5.2.2.6 of TS 38.331 [5]):

2> allow uplink transmission on the corresponding Serving Cell.

1> if an indication of Serving Cell uplink synchronization loss is received from upper layers:

2> flush all HARQ buffers;

2> not perform any uplink transmission on the corresponding Serving Cell

Proposal 2: Update MAC 5.2a Maintenance of UL Synchronization with the proposed TP.

# Conclusion

Based on the discussion, we have the following proposal.

Proposal 1: No change to the current MAC specification on ra-ContentionResolutionTimer for the first Msg3 transmission.

Observation 1: In TS 38.331, the UL synchronization indication is per T430 timer, while according to TS 38.321, the UL synchronization indication obtained from the upper layer is per serving cell.

Proposal 2: Update MAC 5.2a Maintenance of UL Synchronization with the proposed TP.

# Appendix: Text proposal for TS38.321

Start of the change

5.2a     Maintenance of UL Synchronization

The MAC entity shall for each Serving Cell:

1> if an indication of Serving Cell uplink synchronization has been received from upper layers (see clause 5.2.2.6 of TS 38.331 [5]):

2> allow uplink transmission on the corresponding Serving Cell.

1> if an indication of Serving Cell uplink synchronization loss is received from upper layers:

2> flush all HARQ buffers;

2> not perform any uplink transmission on the corresponding Serving Cell.

End of the change