3GPP TSG-RAN WG2 Meeting #119-e                                                 R2-2207275

E-Meeting, Aug 17 – 29, 2022

Agenda item: 	8.6.3

Source:	Intel Corporation

Title: 	Discussion on neighbour cell measurements in IoT NTN

Document for:	Discussion and Decision

# Introduction

The Rel-18 IoT NTN WI was approved in [1], and one objective is to enhance mobility performance as shown below.

4.1.2 Mobility enhancements The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4].

In this paper, we discuss how to further optimize the neighbour cell measurements in IoT NTN.

# Discussion

In Rel-17 NB-IoT, the following agreements were made for neighbour cell measurements:

RAN2#112-e agreements: The solution includes reduction of the time between declaration of RLF and the start of the random-access procedure (points C and D) RAN2#114-e agreements: The criteria to start measurements is based on a combination of serving cell quality threshold (option b) and variance of the serving cell quality (option c) RAN2#115-e agreements: The conditions where the UE is required to perform measurements are specified. No requirement on when to stop measurements is needed. The configuration of the criteria for starting the measurements is provided via broadcast signalling.

The motivation of the support of neighbour cell measurements and corresponding measurement triggering before RLF is to shorten the time between declaration of RLF and the start of the random-access procedure, and this enhancement is to start neighbour cell measurements early before the potential RLF (e.g., when the NRSRP value is lower than a threshold). In this case when a RLF really happens, the UE can skip the cell search phase, and instead the UE can select a target cell based on the measurement results previously acquired in connected mode, and perform RRC re-establishment directly. This enhancement is used to achieve fast RRC re-establishment, and it could be reused in IoT NTN scenario.

Proposal 1: IoT NTN operation can also use the neighbour cell measurements specified in Rel-17 NB-IoT to achieve a fast RRC re-establishment after RLF.

But in Rel-17 NB-IoT, there was no time to discuss further optimization for neighbour cell measurements. For example, the following agreements were made:

RAN2#115-e agreements: Prioritisation of carriers/cells to measure is left to the UE implementation. Provision of information regarding which cells/carriers to be considered is not supported. It is up to UE implementation to choose and prioritize carrier/cell list for measurement.

Above agreement means that for the execution of intra/inter-frequency neighbour cell measurements, there is no prioritisation related assistance information provided for UE. But in IoT NTN, especially in case of discontinuous coverage, there may be only one or two satellites deployed for collecting data from NB-IoT UEs worldwide. And it is possible that network provides the information of next upcoming cells (maybe a smaller group of neighbour cells compared to the list of neighbour cells broadcast in system information) to connected UE for potential RRC re-establishment, so the UE may prioritize the measurements on these upcoming cells. In this way, it can save UE power consumption to focus on more likely neighbour cell measurements before RLF.

Proposal 2: For IoT NTN, network provides the information of next upcoming cells to connected UE, to make UE focus on more likely neighbour cells for RRC re-establishment and save UE power consumption.

# Conclusion

In this paper, we discuss how to further optimize the neighbour cell measurements in IoT NTN, and we have the following proposals:

Proposal 1: IoT NTN operation can also use the neighbour cell measurements specified in Rel-17 NB-IoT to achieve a fast RRC re-establishment after RLF.

Proposal 2: For IoT NTN, network provides the information of next upcoming cells to connected UE, to make UE focus on more likely neighbour cells for RRC re-establishment and save UE power consumption.

# References

[1] RP-221806		Revised WID on IoT NTN enhancements	MediaTek Inc.