3GPP TSG-RAN WG2 #119-e 	R2-2207291

Electronic meeting, 17th –26th August 2022

Agenda Item:	8.1.2 General

Source:	NEC

Title:	Overview of network-controlled repeaters

Document for:	Discussion, Decision

# Introduction

A new Rel-18 study on NR network-controlled repeaters was agreed at RAN#94e meeting. Objectives defined in [1] are

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions:

- Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios.

- For only single hop stationary network-controlled repeaters

- Network-controlled repeaters are transparent to UEs

- Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously

NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters.

Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1]

- Beamforming information

- Timing information to align transmission / reception boundaries of network-controlled repeater

- Information on UL-DL TDD configuration

- ON-OFF information for efficient interference management and improved energy efficiency

- Power control information for efficient interference management (as the 2nd priority)

Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]

Study the following aspects of network-controlled repeater management

- Identification and authorization of network-controlled repeaters [RAN2, RAN3]

NOTE2: Coordination with SA3 may be needed.

This contribution provides our initial considerations and views on network-controlled repeaters.

# Discussion

## General procedures

RAN1#109e meeting agreed to capture the following model of network-controlled repeater in TR38.867.

Fig.1 Network-controlled repeater (NCR)

- The NCR-MT is defined as a function entity to communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g. side control information). The C-link is based on NR Uu interface.

  - Note: Side control information is at least for the control of NCR-Fwd

- The NCR-Fwd is defined as a function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE via backhaul link and access link. The behavior of the NCR-Fwd will be controlled according to the received side control information from gNB.

Based on the agreed model of NCR shown in Fig.1, we assume that NCR-MT will act similarly to a mobile termination but with necessary signaling exchange specific to NCR-Fwd. Assuming that NCR has two phases, i.e., initiation phase and data forward phase, we will discuss them in details.

Initiation phase

During initiation phase, NCR-MT starts its registration/attach under the cell and NCR-FWD doesn’t start its work for forwarding data.

As shown in Fig.2, for NCR-MT initial access phase, legacy UE initial access procedure can be reused with following minor changes:

- a new NCR support indication should be included in system information, NCR should only access to an gNB supporting NCR.

- NCR indication may be added in RRC set up complete message as same as IAB case as discussed in section 2.2.

Fig.2 NCR procedures: initiation phase

For NCR-MT initial access phase, legacy UE initial access procedure can be reused as baseline with following minor changes:

- a new NCR support indication should be included in system information, NCR should only access to an gNB supporting NCR.

- NCR indication may be added in RRC setup complete message as same as IAB case

Data forward phase

After NCR-MT finishes the initial access phase (including necessary identification, authentication, security procedure), NCR-MT is in connected mode. To make NCR-Fwd work, NCR-Fwd should be configured when and how to amplify and forward the received signal from backhaul link to access link.

From higher layer point of view, in order to provide coverage extension, at least, NCR-Fwd should forward following signals :

- SSB

- SIB1 and SI(s)

- Paging

- PRACH in uplink

Above information is necessary for a UE to camp on the cell, monitor paging and initialize RACH procedure whenever needed. Also these information are semi-persistent and can be covered by the initial side control information and configured via RRC signaling. Before the initial side control information configuration, gNB may need to know the capability of NCR-Fwd at backhaul link and access link, e.g., antenna configuration or beam forming configuration. At the beginning of the data forward phase, we propose to have a kind of NCR (NCR-Fwd) setup procedure between NCR and gNB as shown in following figure:

Fig.3 NCR procedures: NCR-Fwd setup at the beginning of data forward phase

RAN2 to discuss to have a kind of NCR (NCR-Fwd) setup procedure between NCR and gNB as shown Fig.3 following the NCR-MT initial access phase.

## Identification and authorization

As we analyzed in section 2.1, it is assumed that NCR-MT terminates the Uu interface to the gNB using the procedures and behavior specified for UEs unless stated otherwise, therefore the existing UE authentication methods as defined in TS33.501 can be applied. Both USIM based methods and EAP based methods are allowed, and NAI based SUPIs can be used.

The existing UE authentication methods as defined in TS33.501 can be applied for NCR-MT. Both USIM based methods and EAP based methods are allowed, and NAI based SUPIs can be used.

Regarding NCR identification, there are two alternatives, one is IAB-like identification whereas another is RedCap-like identification as summarized below:

- IAB-like NCR identification: The NCR-MT set the content of RRCSetupComplete message with the NCR node indication when it is connecting to the gNB.

- RedCap-like NCR identification: The NCR-MT compiles and transfers the NCR node indication associated with its radio UE capability information upon receiving a UECapabilityEnquiry from the gNB.

Difference between IAB-like NCR identification and RedCap-like NCR identification is whether the identification is executed before the authentication and NAS/AS security activation. We propose RAN2 to agree to apply the IAB-like identification for NCR. If it is necessary, RAN2 can send an LS to SA3.

RAN2 to agree to apply the IAB-like identification for NCR. If it is necessary, RAN2 can send an LS to SA3.

Another issue is that whether early indication during random access procedure is necessary for NCR-MT. On the necessary configuration for receiving the L1/L2 signalling of the side control information, RAN1 #109e meeting achieved the following agreements:

Agreement

For the parameters in the necessary configurations for L1/L2 signaling, the existing parameters for PDCCH, PDSCH, PUCCH, PUSCH, DCI, UCI and MAC CE in Rel-17 are the baseline for further discussion.

• Note 1: This does not imply that all Rel-17 parameters will be supported for the NCR-MT.

• Note 2: This does not imply that PUCCH, PUSCH, UCI and MAC CE are currently agreed to be supported. Further consideration is needed.

Even though whether all Rel-17 parameters will be supported for the NCR-MT or not is still FFS, it is natural to assume that NCR-MT has almost the same coverage performance as a normal UE. Therefore, an early identification during random access procedure is not needed for NCR-MT.

RAN2 to agree that an early identification during random access procedure is not needed for NCR-MT.

# Summary

This contribution provides our initial considerations and views on network-controlled repeaters.

- Observation 1 For NCR-MT initial access phase, legacy UE initial access procedure can be reused as baseline with following minor changes:

- a new NCR support indication should be included in system information, NCR should only access to an gNB supporting NCR.

- NCR indication may be added in RRC set up complete message as same as IAB case.

- Observation 2  The existing UE authentication methods as defined in TS33.501 can be applied for NCR-MT. Both USIM based methods and EAP based methods are allowed, and NAI based SUPIs can be used.

- Proposal 1 RAN2 to discuss to have a kind of NCR (NCR-Fwd) setup procedure between NCR and gNB as shown Fig.3 following the NCR-MT initial access phase.

- Proposal 2 RAN2 to agree to apply the IAB-like identification for NCR. If it is necessary, RAN2 can send an LS to SA3.

- Proposal 3 RAN2 to agree that an early identification during random access procedure is not needed for NCR-MT.

# References

[1] RP-213700	New SI: Study on NR Network-controlled Repeaters