3GPP TSG-RAN WG2 #119-e 	R2-2207295

Electronic meeting, 17th –26th August 2022

Agenda Item:	8.5.4 General

Source:	NEC

Title:	XR-specific capacity improvements

Document for:	Discussion, Decision

# Introduction

A new Rel-18 study on XR Enhancements for NR was agreed at RAN#94e meeting [1]. One of the objectives is XR-specific capacity improvement:

Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

This contribution provides our initial consideration on the XR-specific capacity improvements aspect from RAN2 point of view.

# Discussion

## SPS and CG enhancements

For evaluating XR (Extended Reality) for NR, TR38.838 [2] provided DL and UL traffic models of VR, CG and AR application. Generic DL traffic models defined in [2] are categorized to single stream and multiple streams whereas generic UL traffic model is UL pose/control stream traffic model.

Single stream DL traffic model

In this model, the XR DL traffic is modelled as a sequence of video frames arriving at gNB according to the considered video frame rates and random jitter. The size of each frame is also random according to a certain distribution. According to [2], key parameters of this traffic model are packet size, packet arrival, packet delay budget, packet success rate requirement and dual-eye buffer (it is an optional modification of packet size and frame rates for separate packet arrival for dual-eye buffer).

As analyzed by many companies during the study item description discussion, variable packet arrival which mainly depends on non-integer periodicity (derived from frame generation rate, e.g., 60fps) and variable/ unpredictable jitter (due to the different delays caused by network transportation), may not be properly supported by current SPS-config periodicity and periodicityExt fields (i.e., integral multiple of slot duration with the maximum value of 640 ms). No other impacts on the resource allocation and scheduling are foreseen at this moment.

- Observation1: Variable packet arrival which mainly depends on non-integer periodicity and variable/ unpredictable jitter may not be properly supported by current SPS-config periodicity and periodicityExt fields. No other impacts on the resource allocation and scheduling are foreseen.

Multi-streams DL traffic model

This model has three options as followings [2]:

- Option 1: I-frame + P-frame

-	Option 1A: slice-based traffic model

-	Option 1B: Group-Of-Picture (GOP) based traffic model

- Option 2: video + audio/data

- Option 3: FOV + omnidirectional stream

Since Option 2 is just a simple mixture of a video stream (following the generic single stream model) and a periodic audio/data traffic, while the detailed modelling of Option 3 is up to company [2], here we only focus on Option 1.

For Option 1, two streams (I-stream and P-stream) are modelled:

-	Stream 1: I stream

-	Stream 2: P stream

Depending on the video encoding scheme, two additional sub models – slice based, and Group of Picture (GOP)-based models are defined.

-	Slice-based: In this encoding scheme, a single video frame is divided into N slices. Out of N, one slice is I slice and remaining N-1 slices are P slices. N packets (one I and N-1 P) packets corresponds to one video frame arriving at the same time.

-	GOP-based: In this encoding scheme, a single video frame is either I frame or P frame. I frame is transmitted every K frames, where K is the GOP size, i.e., every group of picture. One video frame arrives at a time as a packet.

How Option 1 impacts resource allocation depends on the QoS and policy, which is discussing at SA2 Rel-18 study on architecture enhancement for XR and media services [3]. Related work task in the study is defined as follows:

WT#3: Study whether and how the following QoS and policy enhancements for XR service and media service transmission are performed:

WT#3.1:  Study the traffic characteristics of media service enabling improved network resources usage and QoE.

WT#3.2:  Enhance QoS framework to support media units granularity (e.g., video/audio frame/tile, Application Data Unit, control information), where media units consist of PDUs that have the same QoS requirements.

WT#3.3: Support differentiated QoS handling considering different importance of media units. e.g., eligible drop packets belong to less important media units to reduce the resource wasting.

NOTE2: Coordination with RAN WGs may be needed for the above bullets.

WT#3.4: Whether and how to support uplink-downlink transmission coordination to meet RTT (Round-Trip Time) latency requirements between UE and N6 termination point at the UPF.

WT#3.5: Potential policy enhancements to minimize the jitter, focusing on i.e., requirement provisioning from AF, extension of PCC rule.

SA2 has agreed the definition of PDU Set (equivalent to the “media unit” in the work task description above), which may represent a “slice”, e.g., video/audio frame/tile, haptic application information, and send an LS [4] to SA4.

Wait for further SA2/SA4 input to study more efficient resource allocation and scheduling for multi-streams DL traffic model.

Generic UL pose/control traffic

This model is like single stream DL traffic model summarized in section 2.1. According to [2], key parameters of this traffic model are packet size, packet arrival, packet delay budget, packet success rate requirement. Different from single stream DL traffic, no jitter for the UL traffic and periodicity seems to be an integer ([2] gives an example of 4ms), which can be properly supported by ConfiguredGrantConfig periodicity and periodicityExt fields (i.e., integral multiple of slot duration with the maximum value of 640 ms). Therefore, no impacts on the resource allocation and scheduling are foreseen at this moment.

However, to enable gNB to configure type 1 or type 2 Configured Grant in UL, Input of the XR service characteristic would be useful for gNB e.g., periodicity and start of periodicity, potential packet size, potential jitter range and so on. UE is aware of this information better then network side entities. we can consider that UE report the XR traffic characteristic to gNB

RAN2 discuss to introduce UL assistance information on UL periodical XR traffic.

## Dynamic scheduling/grant enhancements

In our understanding, BSR is a very important input for gNB scheduler to make dynamic schedule decision. However, currently, regular BSR will not be triggered when more data arrive in buffer for the same LCH/LCG.

Padding BSR may be transmitted only if there is enough padding bits available, because padding BSR has lower priority then data. Periodical BSR may not be applicable for burst data arrival unless very short periodicity is configured which does not sound reasonable

The BSR report become less precise with higher BSR index/buffer size value. gNB scheduler may want to have a more precise BSR when it is about to finish the scheduling of data based on previous reported buffer size. e.g., for a packet with mean packet size of video 62500 bytes, according to following table copied from from TS38.321, with 8 bits buffer size field, UE reports BS index is 139 (<=62599 but > 58784 in bytes);

with 5 bits buffer size field, UE will report BS index 28 (≤ 77284 but > 55474),

Table: Buffer size levels (in bytes) for 5-bit Buffer Size field from 38.321

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 8 | ≤ 102 | 16 | ≤ 1446 | 24 | ≤ 20516 |
| 1 | ≤ 10 | 9 | ≤ 142 | 17 | ≤ 2014 | 25 | ≤ 28581 |
| 2 | ≤ 14 | 10 | ≤ 198 | 18 | ≤ 2806 | 26 | ≤ 39818 |
| 3 | ≤ 20 | 11 | ≤ 276 | 19 | ≤ 3909 | 27 | ≤ 55474 |
| 4 | ≤ 28 | 12 | ≤ 384 | 20 | ≤ 5446 | 28 | ≤ 77284 |
| 5 | ≤ 38 | 13 | ≤ 535 | 21 | ≤ 7587 | 29 | ≤ 107669 |
| 6 | ≤ 53 | 14 | ≤ 745 | 22 | ≤ 10570 | 30 | ≤ 150000 |
| 7 | ≤ 74 | 15 | ≤ 1038 | 23 | ≤ 14726 | 31 | > 150000 |

RAN2 agree to enhance BSR to improve the accuracy of dynamic scheduling

## Other aspects

DL and UL traffic are modelled separately in [2]. However, for XR services, interaction between upstream and downstream might also be a very important aspect. For example, with cloud gaming, each time a player inputs an action, the system sends it back upstream to the game engine, traversing the network along the way. The game must then update the view to reflect the action and send it back across the network to the players. Moreover, [5] also mentioned that XR services may have uplink and downlink traffic stream simultaneously, and the alignment of them in Uu interface is very helpful from power saving point of view. Therefore, it would be beneficial to study a more efficient resource allocation and scheduling mechanism for simultaneous/associated DL and UL traffic of XR services.

It would be beneficial to study a more efficient resource allocation and scheduling mechanism for simultaneous/associated DL and UL traffic of XR services.

# Summary

This contribution provided our initial observations and proposals as follows:

- Observation1: Variable packet arrival which mainly depends on non-integer periodicity and variable/ unpredictable jitter may not be properly supported by current SPS-config periodicity and periodicityExt fields. No other impacts on the resource allocation and scheduling are foreseen.

- Wait for further SA2/SA4 input to study more efficient resource allocation and scheduling for multi-streams DL traffic model.

- RAN2 discuss to introduce UL assistance information on UL periodical XR traffic.

- RAN2 agree to enhance BSR to improve the accuracy of dynamic scheduling

- It would be beneficial to study a more efficient resource allocation and scheduling mechanism for simultaneous/associated DL and UL traffic of XR services.

# References

[1] RP-213587	New SID “Study on XR Enhancements for NR”

[2] TR38.838 V17.0.0

[3] SP-211646 New SID on Study on architecture enhancement for XR and media services

[4] R2-2204439 LS on QoS support with PDU Set granularity

[5] RP-221446 “Discussion on XR enhancements for NR”, TCL Communication