3GPP TSG-RAN WG2 Meeting #119e	 Tdoc R2-2207300

Online, 15th Aug – 26th Aug 2022

Agenda Item:	8.6.2

Souce:	MediaTek Inc.

Title:	On Disabling HARQ Feedback in IoT-NTN

Document for:		Discussion and decision

# 1 Introduction

Rel-18 enhancements for IoT-NTN Work Item was approved at RAN Plenary #96e [1]. The corresponding work item has identified IoT-NTN Performance Enhancements to address remaining issues from Rel-17 as one key agenda. The work item objective (WID) has mentioned “Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates” [1]. Previous agreements in Rel-17 NR-NTN Work Item has agreed semi-statically disabling/enabling of downlink HARQ using RRC Signalling. In this contribution we discuss disabling of HARQ in IoT-NTN.

# 2 Discussion

2.1 Enabling and Disabling HARQ Feedback

HARQ is widely used to provide robustness. However, the high round trip delay (RTD) in NTN might result in HARQ staling thus impacting the data rates. Hence, disabling the HARQ feedback has the potential to increase data rates. However, disabling HARQ feedback compromises robustness, which is particularly important for Signalling Radio Bearers (SRBs) and msg3-msg4 transmissions.

Observation 1: Disabling HARQ feedback has the potential to increase data rates at the cost sacrificing robustness.

Hence, based on this motivation, in Rel-17 NR-NTN RAN2 has decided to semi-statically enable and disable HARQ feedback in per HARQ process basis using RRC signalling. Naturally, it is convenient for RAN2 to reuse the same solution strategy for selectively enabling/disabling HARQ feedback per process basis in IoT-NTN.

Proposal 1: RAN2 will reuse NR-NTN agreements to semi-statically enable/disable HARQ feedback using RRC signalling on per HARQ process.

It should be noted that RRC Reconfiguration message was used in NR-NTN for enabling/disabling HARQ feedback. However, NB-IoT CP solutions do not use RRC Reconfiguration. Hence, for NB-IoT, instead of using RRC Reconfiguration, msg4 could be used.

Proposal 2: As NB-IoT CP solutions do not use RRC Reconfiguration, msg4 could be used to semi-statically enable/disable HARQ feedback in NB-IoT based NTN.

2.2 Enhancements to Improve Performance with Disabled HARQ

Due to disabling HARQ feedback, RLC feedback for AM RLC entity become the last chance to guarantee the reliability of data transmission. It is predictable that the chance of RLC retransmission will increase. On the other hand, for IoT NTN, especially for NB-IoT, the RLC polling and STATUS report mechanism is weaker than regular LTE. The conditions to trigger polling, like condition based on unacknowledged RLC PDU number and condition based on unacknowledged byte size, are not applicable to NB-IoT. The rest of the conditions according to 3GPP TS 36.322 are: 1) “if both the transmission buffer and the retransmission buffer become empty (excluding transmitted RLC data PDU awaiting for acknowledgements) after the transmission of the RLC data PDU”, 2) “if no new RLC data PDU can be transmitted after the transmission of the RLC data PDU (e.g. due to window stalling)”. Hence, only when all the data has been transmitted or window stalling (all the unacknowledged RLC PDU has reached the sending window), can the polling be triggered. The chance of triggering polling is rare. The failure reception of RLC PDU will block the delivering data to upper layer. Considering the increasing chance of failure reception of RLC PDU, and the rare chance of polling which requests the peer AM RLC entity to retransmit the data, the chance of blockage will increase. As a result, the data transmission latency will suffer.

In the downlink transmission, UE can autonomously send STATUS report when the receiving side of AM RLC entity in UE detects a reception failure of an RLC data PDU by the SN gap. For NB-IoT which has no dedicated Physical Uplink Control Channel (PUCCH), UE needs to apply for the uplink grant by the random access procedure to send the UL STATUS report. The cell’s coverage in the NTN scenario is typically larger than in the TN scenario, and probably is not adjustable due to the limit satellite resource, so the number of UE covered by the cell could be much greater than in the TN scenario. The random access preamble resource is more likely to be deficient. Not only are the resources limited, the random access latency is much higher than in the TN scenario. A typical random access procedure takes 4 steps, each step takes 20ms to 90ms for propagation delay in LEO and takes 270ms for GEO, compared to less than 1ms taken in propagation delay in TN scenario. It takes valuable random access preamble resource and large transmission latency to send the status report when detecting a reception failure of an RLC PDU, which can be improved.

Observation 2: For NB-IoT, after disabling HARQ feedback, RLC can be enhanced to improve data transmission latency.

To add more chances to send polling to compensate for the loss of the data transmission latency, and to improve the situation of UE autonomously sending STATUS report (to save the valuable random access preamble resource and reduce the latency of sending STATUS report), we propose to enhance RLC in IoT NTN, especially for polling and STATUS report mechanism. More specifically, we propose to consider the following alternatives schemes for enhancing RLC in IoT NTN:

To improve the transmission latency, a more frequent polling can help.

In the condition of window stalling, polling can be triggered to request peer AM RLC entity to send STATUS report. Shortening the AM RLC window size can trigger polling more easily. It can help to reduce the transmission latency,

Proposal 3: For NB-IoT, shorten the AM RLC window size.

The condition based on unacknowledged RLC PDU number and condition based on unacknowledged byte size to trigger polling are not applicable to NB-IoT now. To allow more frequent polling, these two conditions can be added to NB-IoT.

Proposal 4: For NB-IoT, add condition based on unacknowledged RLC PDU number and condition based on unacknowledged byte size to trigger polling.

To save random access preamble resource and to improve the transmission latency for UE autonomously sending STATUS report, the pre-configured Semi-Persistent Scheduling (SPS) periodical UL resource can be utilized.  Since NPUSCH resource is preconfigured, the valuable Random Access preamble resource can be saved. There are no four steps of the random access procedure, so the long transmission latency can be saved.

Proposal 5: For NB-IoT, allow UE autonomously send STATUS report by SPS resource when detect a failure of an RLC PDU.

Alternatively, if the random access preamble resource is not limited, a CFRA resource can be configured to UE to send the STATUS report. Comparing to CBRA, the transmission latency is reduced. CFRA also use less resource than dedicated SPS.

Proposal 6: For NB-IoT, allow UE autonomously send STATUS report by CFRA resource when detect a failure of an RLC PDU.

# 3 Conclusions

Observation 1: Disabling HARQ feedback has the potential to increase data rates at the cost sacrificing robustness.

Proposal 1: RAN2 will reuse NR-NTN agreements to semi-statically enable/disable HARQ feedback using RRC signalling on per HARQ process.

Proposal 2: As NB-IoT CP solutions do not use RRC Reconfiguration, msg4 could be used to semi-statically enable/disable HARQ feedback in NB-IoT based NTN.

Observation 2: For NB-IoT, after disabling HARQ feedback, RLC can be enhanced to improve data transmission latency.

Proposal 3: For NB-IoT, shorten the AM RLC window size.

Proposal 4: For NB-IoT, add condition based on unacknowledged RLC PDU number and condition based on unacknowledged byte size to trigger polling.

Proposal 5: For NB-IoT, allow UE autonomously send STATUS report by SPS resource when detect a failure of an RLC PDU.

Proposal 6: For NB-IoT, allow UE autonomously send STATUS report by CFRA resource when detect a failure of an RLC PDU.

# 4 References

- RP-221554, MediaTek, WID on IoT NTN Enhancements, RAN#96e, June 2022.