3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2207323

# Elbonia, 17 – 26 August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0258 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 NTN related corrections to TS 38.304 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Various NTN-related corrections to IDLE mode specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ‘Otherwise’ is change to ‘else’ to match the if…else procedures The ambiguous term ‘valid UE location’ is removed. Editorial changes in 5.2.4.2 Editor notes are removed in 5.2.1 and 5.3.1 Impact analysis Impacted functionality: Rel-17 NTN Inter-operability: No issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IDLE mode description for NTN Rel-17 is erroneous and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.2, 5.2.1, 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 5.2.4.2	Measurement rules for cell re-selection

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-	If distanceThresh is broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its location information:

-	If the distance between UE and the serving cell reference location is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements;

-	Else, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	If distanceThresh is broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its location information:

-	If the distance between UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8];

-	Else, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8].

-	If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements, as specified in clause 5.2.4.9.

If the t-Service of the serving cell is present in SIB19, UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ.The exact time to start measurements before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service).

NOTE:	When evaluating the distance between UE and the serving cell reference location, it's up to UE implementation to obtain UE location information.

Next Modified Subclause

### 5.2.1	Introduction

UE shall perform measurements for cell selection and reselection purposes as specified in TS 38.133 [8].

When evaluating Srxlev and Squal of non-serving cells for reselection evaluation purposes, the UE shall use parameters provided by the serving cell and for the final check on cell selection criterion, the UE shall use parameters provided by the target cell for cell reselection.

The NAS can control the RAT(s) in which the cell selection should be performed, for instance by indicating RAT(s) associated with the selected PLMN, and by maintaining a list of forbidden registration area(s) and a list of equivalent PLMNs. The UE shall select a suitable cell based on RRC_IDLE or RRC_INACTIVE state measurements and cell selection criteria.

In order to expedite the cell selection process, stored information for several RATs, if available, may be used by the UE.

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected. The change of cell may imply a change of RAT. Details on performance requirements for cell reselection can be found in TS 38.133 [8].

The NAS is informed if the cell selection and reselection result in changes in the received system information relevant for NAS.

For normal service, the UE shall camp on a suitable cell, monitor control channel(s) of that cell so that the UE can:

-	receive system information from the PLMN or SNPN; and

-	receive registration area information from the PLMN or SNPN, e.g., tracking area information; and

-	receive other AS and NAS Information; and

-	if registered:

-	receive paging and notification messages from the PLMN or SNPN; and

-	initiate transfer to Connected mode.

For cell selection in multi-beam operations, measurement quantity of a cell is up to UE implementation.

For cell reselection in multi-beam operations, including inter-RAT reselection from E-UTRA to NR, the measurement quantity of this cell is derived amongst the beams corresponding to the same cell based on SS/PBCH block as follows:

-	if nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA):

-	derive a cell measurement quantity as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [11].

-	else:

-	derive a cell measurement quantity as the linear average of the power values of up to nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) of highest beam measurement quantity values above absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA).

Next Modified Subclause

### 5.3.1	Cell status and cell reservations

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of following fields:

-	cellBarred (IE type: "barred" or "not barred") 
Indicated in MIB message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is ignored by UEs supporting NTN while cellBarred-NTN is included in SIB1.

-	cellBarred-NTN (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is ignored if the UE does not support NTN connectivity.

-	cellBarredRedCap1Rx (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs.

-	cellBarredRedCap2Rx (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs.

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN.

-	cellReservedForOtherUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs.

-	cellReservedForFutureUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs.

NOTE 0:	IAB-MT ignores the cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse, and intraFreqReselection (i.e. treats intraFreqReselection as if it was set to allowed) as defined in TS 38.331 [3]. IAB-MT also ignores cellReservedForOtherUse for cell barring determination (i.e. NPN capable IAB-MT considers cellReservedForOtherUse for determination of an NPN-only cell) as defined in TS 38.331 [3].

-	iab-Support (IE type: "true")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN.

When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use and not "true" for future use,

-	UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell broadcasts any CAG-IDs or NIDs and the cell status is indicated as "not barred" and "not reserved" for operator use and "true" for other use, and not "true" for future use:

-	All NPN-capable UEs shall treat this cell as candidate during the cell selection and cell reselection procedures, other UEs shall treat this cell as if cell status is "barred".

When cell status is indicated as "true" for other use, and either cell does not broadcast any CAG-IDs or NIDs or does not broadcast any CAG-IDs and the UE is not operating in SNPN Access Mode,

-	The UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "true" for future use,

-	The UE shall treat this cell as if cell status is "barred".

When cellBarred-NTN is not broadcast in this cell,

-	For NTN access, the UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN/SNPN and not "true" for other use and not "true" for future use,

-	UEs assigned to Access Identity 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved".

-	UEs assigned to Access Identity 11 or 15 shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for selected/registered SNPN is set to "reserved".

-	UEs assigned to an Access Identity 0, 1, 2 and 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN/SNPN or the selected PLMN/SNPN.

-	UEs assigned to Access Identity 3 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN.

NOTE 1:	Access Identities 11, 15 are only valid for use in the HPLMN/ EHPLMN; Access Identities 12, 13, 14 are only valid for use in the home country as specified in TS 22.261 [12].

NOTE 1a:	Access Identity 3 is only valid for PLMNs that indicate to potential Disaster Inbound Roamers that the UEs can access the PLMN as specified in TS 22.261 [12].

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If the UE is a RedCap UE, the UE shall acquire SIB1 and, in the remainder of this procedure, consider 'intraFreqReselection in MIB' to be 'intraFreqReselectionRedCap in SIB1', if available.

-	If the UE is not a RedCap UE, or if the UE is a RedCap UE and intraFreqReselectionRedCap in SIB1 is available:

-	If the field intraFreqReselection in MIB message is set to "allowed":

-	the UE may select another cell on the same frequency if re-selection criteria are fulfilled;

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	else:

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "not allowed":

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	If the cell operates in licensed spectrum:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	else:

-	If the cell operates in licensed spectrum, or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN or the selected PLMN of the UE, or if this cell belongs to the registered SNPN or the selected SNPN of the UE:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

When cell status "barred" is indicated for RedCap UEs with 1Rx/2Rx or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If the field intraFreqReselectionRedCap in SIB1 message is set to "allowed"; or

-	If the cell is to be treated as if the cell status is "barred" due to not supporting RedCap UEs:

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	If the field intraFreqReselectionRedCap in SIB1 message is set to "not allowed":

-	If the cell operates in licensed spectrum, or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN or the selected PLMN of the UE, or if this cell belongs to the registered SNPN or the selected SNPN of the UE:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds.

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

NOTE 2:	If barring of a cell is triggered by the condition of trackingAreaCode and trackingAreaList not being provided, as specified in TS 38.331 [3], the barring only applies to this PLMN and the UE can re-evaluate the barring condition again due to selection of another PLMN.

End of Modified Subclause