3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2207326

Elbonia, 17 – 26 August 2022

Agenda item:	8.7.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on NW-verified UE location

WID/SID:	NR_NTN_enh – Rel-18

Document for:	Discussion and Decision

# 1	Introduction

In June 2022 RAN#96 has opened and closed a Study on requirements and use cases for network verified UE Location for Non-Terrestrial networks (NTN) [1]. It has the following recommendations:

In this study, we have identified the need to define a network based solution which aims at verifying the reported UE location information.

The verification should be performed independently from the location information reported by UE.

The UE location information for the study is considered verified if the reported UE location is consistent with the network based assessment to within 5-10 km (similar to terrestrial network macro cell size), enabling country discrimination and selection of an appropriate core network in order to support all the regulatory services (i.e. emergency call, lawful intercept, public warning, charging/billing).

The solution should not impact significantly the latency of the targeted services nor infringe privacy requirements that apply to the UE location.

The study in [RAN2,RAN1,RAN3], which will study and evaluate solutions for the network to verify UE reported location information, shall consider the following aspects:

-	The scenario of single satellite (or HAPS) in view by the UE at a time is considered with higher priority.

-	Multiple satellite (or HAPS) in view by the UE may be considered if time allows

-	Assume that the UE is attached to a network (so that its context has been set up in the network) for the purpose of positioning

-	Different solutions or positioning methods for NGSO, GSO or HAPS are not precluded

-	When considering solutions based on positioning methods, existing 3GPP defined RAT dependent positioning methods shall be considered as baseline. Other methods are not precluded.

-	Solutions using existing NG-RAN architecture and procedures shall be considered

In this paper we outline our initial thoughts on what questions need to be answered before RAN WGs proceed with specifying NW-based UE location verification.

# 2	Network Verified UE Location - Considerations

The recommendations from [1] contain several statements which deserve some more clarifications. First of all, the following statement:

The verification should be performed independently from the location information reported by UE.

The term ‘location information’ refers here to the GNSS based location which can be reported by the UE as part of the UEInformationResponse and was named coarseLocationInfo-r17. It is defined as:

coarseLocationInfo Parameter type Ellipsoid-Point defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit. The least significant bits of degreesLatitude and degreesLongitude are set to 0 to meet the accuracy requirement corresponds to a granularity of approximately 2 km. It is up to UE implementation how many LSBs are set to 0 to meet the accuracy requirement.

We suggest to begin with clarifying that the NW neither needs to wait for the UE’s reported coarseLocationInfo, nor is supposed to make use of what the UE has reported when performing NW-based verification.

Proposal 1: The verification method should be performed independently from the location information, reported by the UE (coarseLocationInfo). The NW does not need to wait for coarseLocationInfo reporting to begin its assessment.

As the purpose of the verification is to ensure the user is connected to the correct PLMN, for lawful interception and routing the emergency calls, it may be enough to estimate UE’s two-dimensional location (instead of 3-D position). A decision on this matter shall be taken:

Proposal 2: NW assesses two-dimensional UE location when it verifies UE’s geo-position.

Another interesting statement in [1] is formulated as follows:

The solution should not impact significantly the latency of the targeted services nor infringe privacy requirements that apply to the UE location.

One of the related questions could be: what latency is ‘insignificant’? As the goal of this procedure is to verify the UE location, the natural starting point would be the time when the network receives the coarseLocationInfo from the UE as mentioned above, while there should be no such formal restriction. Simultaneously, the time of stay (ToS) in a cell may be rather short (with a 50 km cell radius on average less than 5 seconds, as per [2]), so adding excessive delay may lead to an additional handover, while at the same time the setup of a call in NTN already is significantly slower than in terrestrial networks. Thus, the additional delay should be kept as low as possible. The fact that the UE position verification may lead to correction of the network selection in order to route emergency call to the correct country also points in the same direction.

Proposal 3: The extra latency introduced by the UE position verification should be kept as short as possible and should be below 1 s. More accurate value is FFS.

# 3	Network Verified UE Location - Method

Another requirement in [1] states that network verified UE location needs to be achieved with a single satellite. As most positioning methods require multiple reference points, several positions in time of the single satellite should be considered and methods like UL- and DL-TDOA can be taken into account.

Proposal 4: RAN2 considers UL-TDOA and DL-TDOA based methods to be applied with specific modifications for NTN.

Besides, methods like Multi-RTT and angle of arrival methods can be potentially used either.

Proposal 5: RAN2 considers also other relevant methods, like Multi-RTT and angle of arrival.

However all of these methods and their NTN-specific adaptations are more in the RAN1 competence area.

Proposal 6: RAN2 asks RAN1 to study how DL-TDOA, UL-TDOA, multi-RTT or angle of arrival methods can be used for network verification of the user location.

The abovementioned techniques can be suggested to RAN1, jointly with the questions and answers outlined in section 2 and 3, assuming RAN2 can find the answers to those doubts prior to liaising with RAN1.

Proposal 7: RAN2 provides RAN1 with the responses to the questions discussed within section 2 and 3 of this paper.

# 4	Conclusion

In this document the following observations and proposals have been made:

Proposal 1: The verification method should be performed independently from the location information, reported by the UE (coarseLocationInfo). The NW does not need to wait for coarseLocationInfo reporting to begin its assessment.

Proposal 2: NW assesses two-dimensional UE location when it verifies UE’s geo-position.

Proposal 3: The extra latency introduced by the UE position verification should be kept as short as possible and should be below 1 s. More accurate value is FFS.

Proposal 4: RAN2 considers UL-TDOA and DL-TDOA based methods to be applied with specific modifications for NTN.

Proposal 5: RAN2 considers also other relevant methods, like Multi-RTT and angle of arrival.

Proposal 6: RAN2 asks RAN1 to study how DL-TDOA, UL-TDOA, multi-RTT or angle of arrival methods can be used for network verification of the user location.

Proposal 7: RAN2 provides RAN1 with the responses to the questions discussed within section 2 and 3 of this paper.

# References

- 3GPP TR 38.882, “Study on requirements and use cases for network verified UE Location for Non-Terrestrial networks (NTN)” version 1.0.0, 2022.

- E. Juan, M. Lauridsen, J. Wigard and P. E. Mogensen, "5G New Radio Mobility Performance in LEO-based Non-Terrestrial Networks," 2020 IEEE Globecom Workshops