3GPP TSG-RAN WG2 Meeting #119-e	R2-2207351

# Electronic meeting, August 17-29, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1544 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the expiry of the contention resolution timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As shown in figure below, when Msg3 retransmission grant is received while the contention resolution timer is running, the contention resolution timer is not restarted immediately but after UE-eNB RTT. Therefore, it is possible the ongoing contention resolution timer expires before the scheduled contention resolution timer can be restarted. This will create unexpected problem as UE will declare contention resolution failure unnecessarily. This problem can be simply resolved by not declaring it as expiry of contention resolution timer (as it is going to be restarted soon in future). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification that if the contention resolution timer is going to be restarted soon then the UE should ignore the expiry of the contention resolution timer. Impact Analysis Impacted functionality: - Contention resolution Inter-operability: - If the UE implements this CR and network does not, the network may assume RACH failure assuming the contention resolution timer expired. - If the UE does not implement this CR and network implements, the UE may miss the Msg4 if it declares the contention resolution failure before restarting the contention resolution timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will declare contention resolution failure unnecessarily. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 5.1	Random Access procedure

### 5.1.1	Random Access Procedure initialization

The Random Access procedure described in this clause is initiated by a PDCCH order, by the MAC sublayer itself or by the RRC sublayer. Random Access procedure on an SCell shall only be initiated by a PDCCH order. If a MAC entity receives a PDCCH transmission consistent with a PDCCH order, as specified inTS 36.212 [5], masked with its C-RNTI, and for a specific Serving Cell, the MAC entity shall initiate a Random Access procedure on this Serving Cell. For Random Access on the SpCell a PDCCH order or RRC optionally indicate the ra-PreambleIndex and the ra-PRACH-MaskIndex, except for NB-IoT where the subcarrier index is indicated; and for Random Access on an SCell, the PDCCH order indicates the ra-PreambleIndex with a value different from 000000 and the ra-PRACH-MaskIndex. For the pTAG preamble transmission on PRACH and reception of a PDCCH order are only supported for SpCell. If the UE is an NB-IoT UE, the Random Access procedure is performed on the anchor carrier or one of the non-anchor carriers for which PRACH resource has been configured in system information.

Before the procedure can be initiated, the following information for related Serving Cell is assumed to be available for UEs other than NB-IoT UEs, BL UEs or UEs in enhanced coverage, as specified in TS 36.331 [8], unless explicitly stated otherwise:

-	the available set of PRACH resources for the transmission of the Random Access Preamble, prach-ConfigIndex.

-	the groups of Random Access Preambles and the set of available Random Access Preambles in each group (SpCell only):

The preambles that are contained in Random Access Preambles group A and Random Access Preambles group B are calculated from the parameters numberOfRA-Preambles and sizeOfRA-PreamblesGroupA:

If sizeOfRA-PreamblesGroupA is equal to numberOfRA-Preambles then there is no Random Access Preambles group B. The preambles in Random Access Preamble group A are the preambles 0 to sizeOfRA-PreamblesGroupA – 1 and, if it exists, the preambles in Random Access Preamble group B are the preambles sizeOfRA-PreamblesGroupA to numberOfRA-Preambles – 1 from the set of 64 preambles as defined in TS 36.211 [7].

-	if Random Access Preambles group B exists, the thresholds, messagePowerOffsetGroupB and messageSizeGroupA, the configured UE transmitted power of the Serving Cell performing the Random Access Procedure, PCMAX, c, as specified in TS 36.101 [10], and the offset between the preamble and Msg3, deltaPreambleMsg3, that are required for selecting one of the two groups of Random Access Preambles (SpCell only).

-	the RA response window size ra-ResponseWindowSize.

-	the power-ramping factor powerRampingStep.

-	the maximum number of preamble transmission preambleTransMax.

-	the initial preamble power preambleInitialReceivedTargetPower.

-	the preamble format based offset DELTA_PREAMBLE (see clause 7.6).

-	the maximum number of Msg3 HARQ transmissions maxHARQ-Msg3Tx (SpCell only).

-	the Contention Resolution Timer mac-ContentionResolutionTimer (SpCell only).

NOTE 1:	The above parameters may be updated from upper layers before each Random Access procedure is initiated.

The following information for related Serving Cell is assumed to be available before the procedure can be initiated for NB-IoT UEs, BL UEs or UEs in enhanced coverage, as specified in TS 36.331 [8]:

-	if the UE is a BL UE or a UE in enhanced coverage:

-	the available set of PRACH resources associated with each enhanced coverage level supported in the Serving Cell for the transmission of the Random Access Preamble, prach-ConfigIndex.

-	for EDT, the available set of PRACH resources associated with EDT for each enhanced coverage level supported in the Serving Cell for the transmission of the Random Access Preamble, prach-ConfigIndex.

-	the groups of Random Access Preambles and the set of available Random Access Preambles in each group(SpCell only):

-	except for EDT:

-	if sizeOfRA-PreamblesGroupA is not equal to numberOfRA-Preambles:

-	Random Access Preambles group A and B exist and are calculated as above;

-	else:

-	the preambles that are contained in Random Access Preamble groups for each enhanced coverage level, if it exists, are the preambles firstPreamble to lastPreamble.

-	for EDT, the preambles that are contained in Random Access Preamble groups for each enhanced coverage level, if it exists, are the preambles firstPreamble to edt-LastPreamble if PRACH resources configured by edt-PRACH-ParametersCE are different from the PRACH resources configured by PRACH-ParametersCE for all enhanced coverage levels and edt-PRACH-ParametersCE for all other enhanced coverage levels, otherwise the preambles for EDT are the preambles lastPreamble+1 to edt-LastPreamble.

NOTE 2:	When a PRACH resource is shared for multiple enhanced coverage levels, and enhanced coverage levels are differentiated by different preamble indices, Group A and Group B is not used for this PRACH resource.

-	if the UE is an NB-IoT UE:

-	the available set of PRACH resources supported in the Serving Cell on the anchor carrier, nprach-ParametersList, and on the non-anchor carriers, in ul-ConfigList.

-	for EDT, the available set of PRACH resources associated with EDT on anchor carrier, nprach-ParametersList-EDT, and on the non-anchor carriers, in ul-ConfigList.

-	for random access resource selection and preamble transmission:

-	a PRACH resource is mapped into an enhanced coverage level.

-	each PRACH resource contains a set of nprach-NumSubcarriers subcarriers which can be partitioned into one or two groups for single/multi-tone Msg3 transmission by nprach-SubcarrierMSG3-RangeStart and nprach-NumCBRA-StartSubcarriers as specified in TS 36.211 [7], clause 10.1.6.1. Each group is referred to as a Random Access Preamble group below in the procedure text.

-	a subcarrier is identified by the subcarrier index in the range:
[nprach-SubcarrierOffset, nprach-SubcarrierOffset + nprach-NumSubcarriers -1]

-	each subcarrier of a Random Access Preamble group corresponds to a Random Access Preamble.

-	when the subcarrier index is explicitly sent from the eNB as part of a PDCCH order ra-PreambleIndex shall be set to the signalled subcarrier index.

-	the mapping of the PRACH resources into enhanced coverage levels is determined according to the following:

-	the number of enhanced coverage levels is equal to one plus the number of RSRP thresholds present in rsrp-ThresholdsPrachInfoList.

-	each enhanced coverage level has one anchor carrier PRACH resource present in nprach-ParametersList and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

-	for EDT, each enhanced coverage level has zero or one anchor carrier PRACH resource present in nprach-ParametersList-EDT and zero or one PRACH resource for each non-anchor carrier signalled in ul-ConfigList.

-	enhanced coverage levels are numbered from 0 and the mapping of PRACH resources to enhanced coverage levels are done in increasing numRepetitionsPerPreambleAttempt order.

-	when multiple carriers provide PRACH resources for the same enhanced coverage level, the UE will randomly select one of them using the following selection probabilities:

-	the selection probability of the anchor carrier PRACH resource for the given enhanced coverage level, nprach-ProbabilityAnchor, is given by the corresponding entry in nprach-ProbabilityAnchorList

-	the selection probability is equal for all non-anchor carrier PRACH resources and the probability of selecting one PRACH resource on a given non-anchor carrier is (1- nprach-ProbabilityAnchor)/(number of non-anchor NPRACH resources)

-	the criteria to select PRACH resources based on RSRP measurement per enhanced coverage level supported in the Serving Cell rsrp-ThresholdsPrachInfoList.

-	the maximum number of preamble transmission attempts per enhanced coverage level supported in the Serving Cell maxNumPreambleAttemptCE.

-	the number of repetitions required for preamble transmission per attempt for each enhanced coverage level supported in the Serving Cell numRepetitionPerPreambleAttempt.

-	the configured UE transmitted power of the Serving Cell performing the Random Access Procedure, PCMAX, c, as specified in TS 36.101 [10].

-	the RA response window size ra-ResponseWindowSize and the Contention Resolution Timer mac-ContentionResolutionTimer (SpCell only) per enhanced coverage level supported in the Serving Cell.

-	for EDT, the Contention Resolution Timer mac-ContentionResolutionTimer configured for EDT (SpCell only) per enhanced coverage level supported in the Serving Cell.

-	the power-ramping factor powerRampingStep and optionally powerRampingStepCE1.

-	the maximum number of preamble transmission preambleTransMax-CE.

-	the initial preamble power preambleInitialReceivedTargetPower and optionally preambleInitialReceivedTargetPowerCE1.

-	the preamble format based offset DELTA_PREAMBLE (see clause 7.6).

-	for NB-IoT, the use of contention free random access ra-CFRA-Config.

The Random Access procedure shall be performed as follows:

-	flush the Msg3 buffer;

-	set the PREAMBLE_TRANSMISSION_COUNTER to 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	set the PREAMBLE_TRANSMISSION_COUNTER_CE to 1;

-	if the starting enhanced coverage level, or for NB-IoT the starting number of NPRACH repetitions, has been indicated in the PDCCH order which initiated the Random Access procedure, or if the starting enhanced coverage level has been provided by upper layers:

-	the MAC entity considers itself to be in that enhanced coverage level regardless of the measured RSRP;

-	else:

-	if the RSRP threshold of enhanced coverage level 3 is configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 3 and the UE is capable of enhanced coverage level 3 then:

-	the MAC entity considers to be in enhanced coverage level 3;

-	else if the RSRP threshold of enhanced coverage level 2 configured by upper layers in rsrp-ThresholdsPrachInfoList and the measured RSRP is less than the RSRP threshold of enhanced coverage level 2 and the UE is capable of enhanced coverage level 2 then:

-	the MAC entity considers to be in enhanced coverage level 2;

-	else if the measured RSRP is less than the RSRP threshold of enhanced coverage level 1 as configured by upper layers in rsrp-ThresholdsPrachInfoList then:

-	the MAC entity considers to be in enhanced coverage level 1;

-	else:

-	the MAC entity considers to be in enhanced coverage level 0;

-	set the backoff parameter value to 0 ms;

-	for the RN, suspend any RN subframe configuration;

-	proceed to the selection of the Random Access Resource (see clause 5.1.2).

NOTE 3:	There is only one Random Access procedure ongoing at any point in time in a MAC entity. If the MAC entity receives a request for a new Random Access procedure while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure.

NOTE 4:	An NB-IoT UE measures RSRP on the anchor carrier.

### 5.1.2	Random Access Resource selection

The Random Access Resource selection procedure shall be performed as follows:

-	for BL UEs or UEs in enhanced coverage or NB-IoT UEs, if EDT is initiated by the upper layers:

-	if the message size (UL data available for transmission plus MAC header and, where required, MAC control elements) is larger than the TB size signalled in edt-TBS for the selected enhanced coverage level for EDT; or

-	if the PRACH resource associated with EDT for the selected enhanced coverage level is not available:

-	indicate to upper layers that EDT is cancelled;

-	for BL UEs or UEs in enhanced coverage, select the PRACH resource set corresponding to the selected enhanced coverage level. For EDT, the PRACH resource set shall correspond to the set associated with EDT for the selected enhanced coverage level.

-	if, except for NB-IoT, ra-PreambleIndex (Random Access Preamble) and ra-PRACH-MaskIndex (PRACH Mask Index) have been explicitly signalled and ra-PreambleIndex is not 000000:

-	the Random Access Preamble and the PRACH Mask Index are those explicitly signalled;

-	else if, for NB-IoT, ra-PreambleIndex (Random Access Preamble) and PRACH resource have been explicitly signalled:

-	the PRACH resource is that explicitly signalled;

-	if the ra-PreambleIndex signalled is not 000000:

-	if ra-CFRA-Config is configured:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers + (ra-PreambleIndex modulo (nprach-NumSubcarriers - nprach-NumCBRA-StartSubcarriers)), where nprach-SubcarrierOffset, nprach-NumCBRA-StartSubcarriers and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + (ra-PreambleIndex modulo nprach-NumSubcarriers), where nprach-SubcarrierOffset and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	select the Random Access Preamble group according to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group.

-	randomly select a Random Access Preamble within the selected group.

-	else the Random Access Preamble shall be selected by the MAC entity as follows:

-	if the UE is a BL UE or UE in enhanced coverage and EDT is initiated:

-	select the Random Access Preambles group corresponding to PRACH resource for EDT for the selected enhanced coverage level.

-	else if the UE is a BL UE or UE in enhanced coverage and Random Access Preamble group B does not exist:

-	select the Random Access Preambles group corresponding to the selected enhanced coverage level.

-	else if the UE is an NB-IoT UE:

-	if the UE supports carrier specific NRSRP thresholds for NPRACH resource selection and rsrp-ThresholdsPrachInfoList-r16 is signalled for a carrier in ul-ConfigList:

-	if the measured RSRP is lower than the RSRP threshold corresponding to the selected enhanced coverage level in rsrp-ThresholdsPrachInfoList-r16:

-	do not consider the PRACH resource on this non-anchor carrier for PRACH resource selection.

-	randomly select one of the PRACH resources corresponding to the selected enhanced coverage level according to the configured probability distribution, and select the Random Access Preambles group corresponding to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group. For EDT, the PRACH resource shall correspond to resource associated with EDT for the selected enhanced coverage level.

-	else if Msg3 has not yet been transmitted, the MAC entity shall:

-	if Random Access Preambles group B exists and any of the following events occur:

-	the potential message size (UL data available for transmission plus MAC header and, where required, MAC control elements) is greater than messageSizeGroupA and the pathloss is less than PCMAX,c (of the Serving Cell performing the Random Access Procedure) – preambleInitialReceivedTargetPower – deltaPreambleMsg3 – messagePowerOffsetGroupB;

-	the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC header is greater than messageSizeGroupA;

-	select the Random Access Preambles group B;

-	else:

-	select the Random Access Preambles group A.

-	else, if Msg3 is being retransmitted, the MAC entity shall:

-	select the same group of Random Access Preambles as was used for the preamble transmission attempt corresponding to the first transmission of Msg3.

-	randomly select a Random Access Preamble within the selected group. The random function shall be such that each of the allowed selections can be chosen with equal probability;

-	except for NB-IoT, set PRACH Mask Index to 0.

-	determine the next available subframe containing PRACH permitted by the restrictions given by the prach-ConfigIndex (except for NB-IoT), the PRACH Mask Index (except for NB-IoT, see clause 7.3), physical layer timing requirements, as specified in TS 36.213 [2], and in case of NB-IoT, the subframes occupied by PRACH resources related to a higher enhanced coverage level (a MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH subframe);

-	except for NB-IoT:

-	if the transmission mode is TDD and the PRACH Mask Index is equal to zero:

-	if ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC):

-	randomly select, with equal probability, one PRACH from the PRACHs available in the determined subframe.

-	else:

-	randomly select, with equal probability, one PRACH from the PRACHs available in the determined subframe and the next two consecutive subframes.

-	else:

-	determine a PRACH within the determined subframe in accordance with the requirements of the PRACH Mask Index, if any.

-	for NB-IoT UEs, BL UEs or UEs in enhanced coverage, select the ra-ResponseWindowSize and mac-ContentionResolutionTimer corresponding to the selected enhanced coverage level and PRACH.

-	proceed to the transmission of the Random Access Preamble (see clause 5.1.3).

### 5.1.3	Random Access Preamble transmission

The random-access procedure shall be performed as follows:

-	set PREAMBLE_RECEIVED_TARGET_POWER to preambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep;

-	if the UE is a BL UE or a UE in enhanced coverage:

-	the PREAMBLE_RECEIVED_TARGET_POWER is set to:
PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt);

-	if the UE is an NB-IoT UE:

-	for enhanced coverage level 0, the PREAMBLE_RECEIVED_TARGET_POWER is set to:
 PREAMBLE_RECEIVED_TARGET_POWER - 10 * log10(numRepetitionPerPreambleAttempt)

-	for FDD if the UE supports enhanced random access power control and PowerRampingParameters-NB-v1450 is configured by upper layers, or for TDD:

-	the MSG3_RECEIVED_TARGET_POWER is set to preambleInitialReceivedTargetPower + (PREAMBLE_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

-	for other enhanced coverage levels:

-	for FDD if the UE supports enhanced random access power control and PowerRampingParameters-NB-v1450 is configured by upper layers, or for TDD; and

-	if the starting enhanced coverage level was enhanced coverage level 0 or enhanced coverage level 1:

-	if the MAC entity considers itself to be in enhanced coverage level 1 and if powerRampingStepCE1 and preambleInitialReceivedTargetPowerCE1 have been configured by upper layers:

-	the PREAMBLE_RECEIVED_TARGET_POWER is set to preambleInitialReceivedTargetPowerCE1 + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER_CE – 1) * powerRampingStepCE1 - 10 * log10(numRepetitionPerPreambleAttempt);

-	the MSG3_RECEIVED_TARGET_POWER is set to preambleInitialReceivedTargetPowerCE1 + (PREAMBLE_TRANSMISSION_COUNTER_CE – 1) * powerRampingStepCE1;

-	else:

-	the PREAMBLE_RECEIVED_TARGET_POWER is set to preambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep - 10 * log10(numRepetitionPerPreambleAttempt);

-	the MSG3_RECEIVED_TARGET_POWER is set to preambleInitialReceivedTargetPower + (PREAMBLE_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

-	else:

-	the PREAMBLE_RECEIVED_TARGET_POWER is set corresponding to the max UE output power;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	instruct the physical layer to transmit a preamble with the number of repetitions required for preamble transmission corresponding to the selected preamble group (i.e., numRepetitionPerPreambleAttempt) using the selected PRACH corresponding to the selected enhanced coverage level, corresponding RA-RNTI, preamble index or for NB-IoT subcarrier index, and PREAMBLE_RECEIVED_TARGET_POWER.

-	else:

-	instruct the physical layer to transmit a preamble using the selected PRACH, corresponding RA-RNTI, preamble index and PREAMBLE_RECEIVED_TARGET_POWER.

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception, and regardless of the prioritization of V2X sidelink communication described in clause 5.14.1.2.2, the MAC entity shall monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI defined below, in the RA Response window which starts at the subframe that contains the end of the preamble transmission,as specified in TS 36.211 [7], plus three subframes and has length ra-ResponseWindowSize.

If the UE is a BL UE or a UE in enhanced coverage:

-	if the random access preamble was transmitted in a non-terrestrial network:

-	RA Response window starts at the subframe that contains the end of the last preamble repetition plus 3 + UE-eNB RTT subframes and has length ra-ResponseWindowSize for the corresponding enhanced coverage level;

-	else:

-	RA Response window starts at the subframe that contains the end of the last preamble repetition plus three subframes and has length ra-ResponseWindowSize for the corresponding enhanced coverage level.

If the UE is an NB-IoT UE:

-	if the random access preamble was transmitted in a non-terrestrial network:

-	RA Response window starts at the subframe that contains the end of the last preamble repetition plus X + UE-eNB RTT subframes and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions;

-	else:

-	RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions.

Table 5.1.4-1: Subframes between preamble transmission and RA Response Window in NB-IoT

| TDD/FDD mode | Preamble format | Number of NPRACH repetitions | X |
| --- | --- | --- | --- |
| FDD | 0 or 1 | >= 64 | 41 |
| FDD | 0 or 1 | < 64 | 4 |
| FDD | 2 | >= 16 | 41 |
| FDD | 2 | < 16 | 4 |
| TDD | Any | Any | 4 |

The RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI= 1 + t_id + 10*f_id

where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), and f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6) except for NB-IoT UEs, BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where  is defined in clause 5.7.1 of TS 36.211 [7].

For BL UEs and UEs in enhanced coverage, RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI=1+t_id + 10*f_id + 60*(SFN_id mod (Wmax/10))

where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6), SFN_id is the index of the first radio frame of the specified PRACH, and Wmax is 400, maximum possible RAR window size in subframes for BL UEs or UEs in enhanced coverage. If the PRACH resource is on a TDD carrier, the f_id is set to , where  is defined in clause 5.7.1 of TS 36.211 [7].

For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id

where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0.

For NB-IoT UEs operating in TDD mode, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2)

where SFN_id is the index of the first radio frame of the specified PRACH and H-SFN is the index of the first hyper frame of the specified PRACH. The PDCCH transmission and the PRACH resource are on the same carrier.

The MAC entity may stop monitoring for Random Access Response(s) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted Random Access Preamble.

-	If a downlink assignment for this TTI has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded, the MAC entity shall regardless of the possible occurrence of a measurement gap or a Sidelink Discovery Gap for Transmission or a Sidelink Discovery Gap for Reception, and regardless of the prioritization of V2X sidelink communication described in clause 5.14.1.2.2:

-	if the Random Access Response contains a Backoff Indicator subheader:

-	set the backoff parameter value as indicated by the BI field of the Backoff Indicator subheader and Table 7.2-1, except for NB-IoT where the value from Table 7.2-2 is used.

-	else, set the backoff parameter value to 0 ms.

-	if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble (see clause 5.1.3), the MAC entity shall:

-	consider this Random Access Response reception successful and apply the following actions for the serving cell where the Random Access Preamble was transmitted:

-	process the received Timing Advance Command (see clause 5.2);

-	indicate the preambleInitialReceivedTargetPower and the amount of power ramping applied to the latest preamble transmission to lower layers (i.e., (PREAMBLE_TRANSMISSION_COUNTER – 1) * powerRampingStep);

-	if the SCell is configured with ul-Configuration-r14, ignore the received UL grant otherwise process the received UL grant value and indicate it to the lower layers;

-	if, except for NB-IoT, ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC):

-	consider the Random Access procedure successfully completed.

-	else if, the UE is an NB-IoT UE, ra-PreambleIndex was explicitly signalled and it was not 000000 (i.e., not selected by MAC) and ra-CFRA-Config is configured:

-	consider the Random Access procedure successfully completed.

-	the UL grant provided in the Random Access Response message is valid only for the configured carrier (i.e. UL carrier used prior to this Random Access procedure).

-	else:

-	if the Random Access Preamble was selected by the MAC entity; or

-	if the UE is an NB-IoT UE, the ra-PreambleIndex was explicitly signalled and it was not 000000 and ra-CFRA-Config is not configured:

-	set the Temporary C-RNTI to the value received in the Random Access Response message no later than at the time of the first transmission corresponding to the UL grant provided in the Random Access Response message;

-	if the Random Access Preamble associated with EDT was transmitted and UL grant provided in the Random Access Response message is not for EDT:

-	indicate to upper layers that EDT is cancelled due to UL grant not being for EDT;

-	for CP-EDT, flush the Msg3 buffer.

-	for UP-EDT, update the MAC PDU in the Msg3 buffer in accordance with the uplink grant received in the Random Access Response.

-	if the Random Access Preamble associated with EDT was transmitted, the UL grant was received in a Random Access Response for EDT, and there is a MAC PDU in the Msg3 buffer:

-	if the TB size according to edt-SmallTBS-Enabled and as described in clause 8.6.2 and 16.3.3 of TS 36.213 [2] does not match the size of the MAC PDU in the Msg3 buffer:

-	the MAC entity shall update the MAC PDU in the Msg3 buffer in accordance with the TB size.

-	if this is the first successfully received Random Access Response within this Random Access procedure; or

-	if CP-EDT is cancelled due to the UL grant provided in the Random Access Response message not being for EDT:

-	if the transmission is not being made for the CCCH logical channel, indicate to the Multiplexing and assembly entity to include a C-RNTI MAC control element in the subsequent uplink transmission;

-	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity and store it in the Msg3 buffer.

NOTE 1:	When an uplink transmission is required, e.g., for contention resolution, the eNB should not provide a grant smaller than 56 bits (or 88 bits for NB-IoT) in the Random Access Response.

NOTE 2:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behavior is not defined except for EDT.

If no Random Access Response or, for NB-IoT UEs, BL UEs or UEs in enhanced coverage for mode B operation, no PDCCH scheduling Random Access Response is received within the RA Response window, or if none of all received Random Access Responses contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, the Random Access Response reception is considered not successful and the MAC entity shall:

-	if the notification of power ramping suspension has not been received from lower layers:

-	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE + 1:

-	if the Random Access Preamble is transmitted on the SpCell:

-	indicate a Random Access problem to upper layers;

-	if NB-IoT:

-	consider the Random Access procedure unsuccessfully completed;

-	else:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

-	if the Random Access Preamble is transmitted on the SpCell:

-	indicate a Random Access problem to upper layers;

-	if the Random Access Preamble is transmitted on an SCell:

-	consider the Random Access procedure unsuccessfully completed.

-	if in this Random Access procedure, the Random Access Preamble was selected by MAC:

-	based on the backoff parameter, select a random backoff time according to a uniform distribution between 0 and the Backoff Parameter Value;

-	delay the subsequent Random Access transmission by the backoff time;

-	else if the SCell where the Random Access Preamble was transmitted is configured with ul-Configuration-r14:

-	delay the subsequent Random Access transmission until the Random Access Procedure is initiated by a PDCCH order with the same ra-PreambleIndex and ra-PRACH-MaskIndex;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	increment PREAMBLE_TRANSMISSION_COUNTER_CE by 1;

-	if PREAMBLE_TRANSMISSION_COUNTER_CE = maxNumPreambleAttemptCE for the corresponding enhanced coverage level + 1:

-	reset PREAMBLE_TRANSMISSION_COUNTER_CE;

-	consider to be in the next enhanced coverage level, if it is supported by the Serving Cell and the UE, otherwise stay in the current enhanced coverage level;

-	if the UE is an NB-IoT UE:

-	if the Random Access Procedure was initiated by a PDCCH order:

-	select the PRACH resource in the list of UL carriers providing a PRACH resource for the selected enhanced coverage level for which the carrier index is equal to ((Carrier Indication from the PDCCH order) modulo (Number of PRACH resources in the selected enhanced coverage));

-	consider the selected PRACH resource as explicitly signalled;

-	proceed to the selection of a Random Access Resource (see clause 5.1.2).

### 5.1.5	Contention Resolution

Contention Resolution is based on either C-RNTI on PDCCH of the SpCell or UE Contention Resolution Identity on DL-SCH.

Once Msg3 is transmitted, the MAC entity shall:

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if Msg3 is transmitted on a non-terrestrial network:

-	if, for EDT, edt-SmallTBS-Enabled is set to TRUE for the corresponding PRACH resource:

-	start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission of the bundle in the subframe corresponding to the last subframe of a PUSCH transmission corresponding to the largest TBS indicated by the UL grant plus UE-eNB RTT subframes.

-	else:

-	start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission of the bundle in the subframe containing the last repetition of the corresponding PUSCH transmission plus UE-eNB RTT subframes.

-	else:

-	if, for EDT, edt-SmallTBS-Enabled is set to TRUE for the corresponding PRACH resource:

-	start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission of the bundle in the subframe corresponding to the last subframe of a PUSCH transmission corresponding to the largest TBS indicated by the UL grant.

-	else:

-	start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission of the bundle in the subframe containing the last repetition of the corresponding PUSCH transmission.

-	else:

-	start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission.

-	regardless of the possible occurrence of a measurement gap or Sidelink Discovery Gap for Reception, monitor the PDCCH until mac-ContentionResolutionTimer expires or is stopped;

-	if notification of a reception of a PDCCH transmission is received from lower layers, the MAC entity shall:

-	if the C-RNTI MAC control element was included in Msg3:

-	if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains an UL grant for a new transmission; or

-	if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI:

-	consider this Contention Resolution successful;

-	stop mac-ContentionResolutionTimer;

-	discard the Temporary C-RNTI;

-	if the UE is an NB-IoT UE:

-	the UL grant or DL assignment contained in the PDCCH transmission is valid only for the configured carrier (i.e. UL/DL carrier used prior to this Random Access procedure).

-	consider this Random Access procedure successfully completed.

-	else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its Temporary C-RNTI:

-	if the MAC PDU is successfully decoded:

-	stop mac-ContentionResolutionTimer;

-	if the MAC PDU contains a UE Contention Resolution Identity MAC control element; and

-	if the UE Contention Resolution Identity included in the MAC control element matches the 48 first bits of the CCCH SDU transmitted in Msg3:

-	consider this Contention Resolution successful and finish the disassembly and demultiplexing of the MAC PDU;

-	set the C-RNTI to the value of the Temporary C-RNTI;

-	discard the Temporary C-RNTI;

-	consider this Random Access procedure successfully completed.

-	else:

-	discard the Temporary C-RNTI;

-	consider this Contention Resolution not successful and discard the successfully decoded MAC PDU.

-	if mac-ContentionResolutionTimer expires:

-	for BL UEs or UEs in CE or NB-IoT UEs:

-	if notification of a reception of a PDCCH transmission has been received from lower layers before mac-ContentionResolutionTimer expired; and

-	if the MAC PDU received until the subframe that contains the last repetition of the corresponding PDSCH transmission is successfully decoded; and

-	if the MAC PDU contains a UE Contention Resolution Identity MAC control element; and

-	if the UE Contention Resolution Identity included in the MAC control element matches the 48 first bits of the CCCH SDU transmitted in Msg3:

-	consider this Contention Resolution successful and finish the disassembly and demultiplexing of the MAC PDU;

-	set the C-RNTI to the value of the Temporary C-RNTI;

-	discard the Temporary C-RNTI;

-	consider this Random Access procedure successfully completed.

-	else if notification of a reception of a PDCCH transmission addressed to Temporary C-RNTI indicating uplink grant for a Msg3 retransmission has been received before mac-ContentionResolutionTimer expired:

-	not consider mac-ContentionResolutionTimer expired;

-	else:

-	discard the Temporary C-RNTI;

-	consider this Contention Resolution not successful.

-	except for BL UEs or UEs in CE or NB-IoT UEs:

-	discard the Temporary C-RNTI;

-	consider the Contention Resolution not successful.

-	if the Contention Resolution is considered not successful the MAC entity shall:

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

-	if the notification of power ramping suspension has not been received from lower layers:

-	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax-CE + 1:

-	indicate a Random Access problem to upper layers;

-	if NB-IoT:

-	consider the Random Access procedure unsuccessfully completed.

-	else:

-	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

-	indicate a Random Access problem to upper layers.

-	based on the backoff parameter, select a random backoff time according to a uniform distribution between 0 and the Backoff Parameter Value;

-	delay the subsequent Random Access transmission by the backoff time;

-	proceed to the selection of a Random Access Resource (see clause 5.1.2).

### 5.1.6	Completion of the Random Access procedure

At completion of the Random Access procedure, the MAC entity shall:

-	discard explicitly signalled ra-PreambleIndex and ra-PRACH-MaskIndex, if any;

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer.

Upon successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall:

-	indicate the successful completion of the Random Access Procedure to the upper layers.

In addition, the RN shall resume the suspended RN subframe configuration, if any.

END OF CHANGE