3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2207366

# Online, August, 2022

Source:	TCL Communication

Title:	Discussion on XR-awareness

Agenda item:	8.5.2

Document for:	Discussion and decision

- Introduction

In the Rel-18 Study Item on XR Enhancements for NR. One of the objectives is as follows:

Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling.

In this contribution we will discuss the XR-awareness and provide our considerations on it.

- Discussion

  - Framework for XR awareness in RAN

It is well known that XR service has some special characteristics, such as high data rate, low latency, and real-time interaction. Modelling of the XR service characteristics can facilitate identifying of XR awareness parameters for RAN and 5GC to support XR service more efficiently.

During the discussion of XR SI in RAN meetings, many models for XR service characteristics were proposed, some of them were included in the objective and the scope of the SI, and others such as non-integer periodicity with jitter, multiple flows with association (e.g. multiple flow synchronization), variable packet size, latency sensitive, high reliability but unequal importance inter/intra flows, etc were left for further study and discussion due to different views from companies,.

In the recent SA2 meeting, some models of XR service characteristics were also discussed and some conclusions were reached, such as frame and GOP, periodic and jitter, and frame importance, PDU set granularity, traffic correlation.

Based on modelling the characteristics of XR service, some XR awareness parameters could be defined. Generally, according to the means which are used to provide those parameters to RAN, the XR awareness parameters could be divided into two categories, i.e., control plane parameters and user plane parameters which are provided to RAN over control plane and user plane respectively. The control plane parameters could further comprise QoS parameters or service configuration parameters which are provide by means of QoS configuration or service configuration information in the XR service session establishment or reconfiguration procedure.

- XR awareness by control plane parameters

- QoS parameters

Obviously, the XR service characteristics of jitter requirement, latency requirement, reliability requirement and packet size should be expressed as QoS parameters. However, compared with 5QI QoS parameters which are based on QoS Flow granularity and packet, the QoS parameters for XR service should be based on PDU set granularity (e.g., video/audio frame/tile, Application Data Units etc.) and packet. As a result, some new QoS parameters may be needed.

- Service configuration parameters

The XR service characteristics of periodicity, number of flows, importance and correlation of inter flows are service specific and could be stable during the XR service session. All of these parameters should be configured in the XR service session establishment or reconfiguration procedure.

- XR awareness by user plane parameters

For XR services, the characteristic of PDU Set is a very important since a group of packets would be used to carry payloads of a PDU Set (e.g. a frame, video frame/slice/tile). In application layer, packets in such a PDU Set should be handled as a whole, i.e., the groups of packets within the PDU Set have inherent dependency on each other. For example, the frame/video/tile slice can only be decoded and decompressed in case that all of the packets carrying the frame/video slice are successfully received. P-frame can only be decoded and decompressed in case of the previous I-frame or P-fame are decoded and decompressed successfully.

Since XR service is characterized by high data rate, low latency, it’s more possible to cause congestion in RAN. Therefore, RAN should schedule radio resources for the packets transmission efficiently considering the inherent dependency of the packets of a PDU Set. For example, RAN may only drop the obsolete packets or the least important packet in congestion state. To meet this kind of requirement and to improve the resource efficiency, one potential solution is to provide the PDU Set SN and the packet SN within a PDU Set to RAN. Obviously, it is more suitable to provide those SN by user plane since the SN varies at every packet and PDU Set.

Proposal 1: The XR awareness could be divided into XR awareness by control plane parameters and XR awareness by user plane parameters, and the control plane parameters could further consist of QoS and Service configuration parameters

  - Service configuration

Regarding the service configuration, to make parameters more  predictable for more efficient gNB scheduling, based on the characteristic of XR traffic, especially for the video frame/slice/tile, the combinations of multiple parameters from {periodicity/frame rate, the number of packet, the size of each packet, packet interval, the total size of the whole frame} can be used for the service configuration.

For example, to support PDU Set for a frame of the video traffic flow periodically, one potential solution is to configure multiple SPS or CG per frame period for downlink and uplink respectively. In this case, As illustrated in figure 1, the periodicity/frame rate, the number of packet, packet interval of the traffic can be assumed stable relatively during the XR service session and be used for the service configuration parameters, while the size of packets which is variable cab be adaptable to the variable size of the video frame. By those configuration parameters, gNB can configure SPS/CG resources more convenient for the XR service.

Figure 1

Moreover, for different XR service requirement, a series of possible value combinations of the configuration parameter could be predefined with index for each of them. As a result, the index could be used to simplify the configuration and decrease the overhead of the signaling.

Static XR traffic awareness parameters

| index | periodicity/frame rate | the number of packet | packet interval |
| --- | --- | --- | --- |
| 0 | 33.33ms/30 | 3 | 1 |
| 1 | 33.33ms/30 | 5 | 2 |
| 2 | 16.67ms/60 | 2 | 0.5 |
| 3 | 16.67ms/60 | 4 | 1 |
| 4 | 11.11ms/90 | 3 | 0.25 |
| 5 | 8.33ms/120 | 3 | 0.25 |
| ... | ... | ... | ... |

Proposal 2: The combinations of multiple parameters from {periodicity/frame rate, the number of packet, the size of each packet, packet interval, the total size of the whole frame} can be used for XR service configuration

  - XR awareness by user plane parameters

Some of the characteristic information of XR service is variable in time, such as the correlation between packet and frame, the SN of the frame, the SN of the packet in a frame. These XR service parameters are very helpful for gNB efficient radio resource management, and the awareness of such parameters by user plane is also helpful.  The XR awareness of user plane parameters may include but not limited to: the start packet of a frame, the SN of a frame, the SN of a packet in a frame.

However, on one hand, although these parameters could be only useful for the packet transport but they are unnecessary for the XR service/application itself; on the other hand, the source point of those parameters usually locates in the application layer while the destination of them is located at the transport layer, so the delivery for these parameters is not peer to peer but is cross-layer type delivery. An example illustrated in figure 2, the XR awareness user plane parameters for the downlink could be generated at the PUD layer in the XR service server and used in PDCP layer in gNB.

Figure 2

Observation 1: the delivery of the XR awareness user plane parameters is not peer to peer and is cross-layer

Figure 3 is a potential protocol stack for the PDU Layer. The XR awareness user plane parameters could be generated at the Payload format sub-layer as the source point and the forwarded to the RTP sub-layer for delivery.

Figure 3

RTP protocol is designed for end-to-end, real-time transfer of streaming media and the general packet header of RTP is illustrates in Figure 4. The XR awareness user plane parameters could be piggybacked in the Extension header of the RTP header by setting the X (Extension) bit.

| Offsets | Octet | 0 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octet | Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 0 | 0 | Version |  | P | X | CC |  |  |  | M | PT |  |  |  |  |  |  | Sequence number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 32 | Timestamp |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 64 | SSRC identifier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | 96 | CSRC identifiers ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12+4×CC | 96+32×CC | Profile-specific extension header ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Extension header length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16+4×CC | 128+32×CC | Extension header ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 4

Propose 3: the XR awareness user plane parameters could be include but not limited to: the start packet of a frame, the SN of a frame, the SN of a packet in a frame, and be piggybacked in the Extension header of the RTP header

As the destination of the XR awareness user plane parameters, the XR awareness user plane parameters should be taken out and used to schedule radio resources to transmit related packets in gNB. As illustrated in figure 5, one potential solution is taken out the the XR awareness user plane parameters from RTP extension header at PDCP layer before the ROHC functionality.

Figure 5

Propose 4: the XR awareness user plane parameters could be taken out from the Extension header of the RTP header at PDCP layer in gNB

- Conclusion

In this contribution, we provide our considerations on XR awareness in RAN and then we have:

Proposal 1: the XR awareness could be divided into XR awareness by control plane parameters and XR awareness by user plane parameters, and the control plane parameters could further consist of QoS and Service configuration parameters

Proposal 2: the combinations of multiple parameters from {periodicity/frame rate, the number of packet, the size of each packet, packet interval, the total size of the whole frame} can be used for the service configuration

Observation 1: the delivery of the XR awareness user plane parameters is not peer to peer and is cross-layer

Propose 3: the XR awareness user plane parameters could be include but not limited to: the start packet of a frame, the SN of a frame, the SN of a packet in a frame, and be piggybacked in the Extension header of the RTP header

Propose 4: the XR awareness user plane parameters could be taken out from the Extension header of the RTP header at PDCP layer in gNB

- References

- RP-213587, “Study on XR Enhancements for NR”, Nokia, RAN#94-e

- S2-2201087, “Key issue on traffic characteristics of XR and media service”, Lenovo, Motorola Mobility, Huawei, HiSilicon, Vivo, CATT，SA2#149e

- S2-2201808, “New Key Issue: PDU Set integrated packet handling”，Huawei, HiSilicon, KDDI, CATT, Ericsson, Futurewei, Intel, InterDigital, Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell, OPPO, Tencent, Tencent Cloud, vivo, Deutsche Telekom, China Mobile, SA2#149e

- S2-2201809, “New Key Issue: The differentiated QoS handling considering different importance of media units”, CATT, Huawei, Hisilicon, Nokia, OPPO, Futurewei, Intel, Tencent, Lenovo, vivo, Ericsson, Interdigital Inc., SA2#149e

- S4-220220, 3GPP TR 26.926 V1.1.0 (2022-02)

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR V1.0.1 (2021-12)