3GPP TSG-RAN WG2 Meeting #119e                                             	 R2-2207381

E-Meeting, August 17th – August 26th, 2022

Agenda item: 	8.4.3

Source:	Intel Corporation

Title: 	Discussion on candidate solutions of L1 L2 mobility

Document for:	Discussion and Decision

# Introduction

New WI on “Further NR mobility enhancements” was approved in [1]. The RAN2 related objectives are as follow:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2]

  - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet

- Timing Advance management [RAN1, RAN2]

- CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3]

Note 2: FR2 specific enhancements are not precluded, if any.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

- To specify CHO including target MCG and target SCG [RAN3, RAN2].

Note 5: This is already being targeted for Rel-17, so this objective will be reviewed at RAN#95-e.

- To specify CHO including target MCG and candidate SCGs for CPC/CPA  [RAN3, RAN2]

- CHO including target MCG and target SCG is used as the baseline

In this contribution, we would like to discuss the protocol aspect and scenarios to support L1/L2 mobility to reduce handover latency.

# Discussion

## 2.1 Scenarios

As mentioned in the WID, following scenarios are considered in the WI:

“Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

”

We show our understanding on the different mobility scenarios to be considered to support L1/L2 mobility as following:

| (1) Intra-CU inter-DU mobility | (2a) Intra-DU mobility | (2b) Intra-DU handover to additional PCI |
| --- | --- | --- |

## 2.2 L2/L3 operation for L1/L2 mobility

Case (1): Intra-CU inter-DU mobility

In this case, the current serving cell and the candidate cells share the same CU, i.e., SDAP and PDCP protocol stacks are shared. Since PDCP is shared, if there is no change of the AS security algorithms associated to the master key, there is no need to update the master key (i.e., the NCC value is not changed). Then PDCP re-establishment is also not needed which would reduce interruption time. Since the source cell and the target cell are located in different DU, RLC layer should be re-established, and the MAC layer should be reset. Due to RLC re-establishment, the PDCP layer also needs to perform PDCP data recovery for AM bearers.

In legacy procedure, the PDCP recovery can be indicated by “recoverPDCP” and the RLC re-establishment can be indicated by “reestablishRLC” in TS38.331 specification. And currently “reset MAC” is a default UE behavior upon handover.

Observation 1: For intra-CU inter DU mobility case, PDCP recovery, RLC reestablishment and MAC reset are needed to support L1/L2 mobility.

Case (2a): Intra-DU mobility

In this case, the current serving cell and the candidate cells share the same CU and DU, i.e., the same SDAP, PDCP, RLC, and MAC protocol stacks are shared. Therefore, there is no need for PDCP, RLC to reestablished. As for MAC and PHY layer, the UE needs to access a new cell with new PCI and C-RNTI as well as reset some related procedures/ counters and timers such as RACH, HARQ and physical layer measurements (e.g., beam failure detection, and PHR (Power Headroom Reporting))). The partial MAC functions which include the logical channel Bj value, triggered BSR can be maintained.

Observation 2: For intra-DU case, PDCP and RLC reestablishment are not needed to support L1/L2 mobility.

Observation 3: For intra-DU mobility case, partial MAC reset is needed to support L1/L2 mobility.

Case (2b): Intra-DU case handover based on unified TCI state

Within intra-DU, there is a special case (2b) where the UE received PDSCH from a TRP associated with an additional PCI (different from current serving cell’s PCI) (i.e a unified TCI state associated to the target cell) is activated for UE. When the UE handovers to this target cell, TCI state change without change of serving cell is already supported in Rel-17. The TA timer can also be maintained as the UE doesn’t need to re-perform uplink synchronization, and the NDIs of uplink HARQ processes and the DL HARQ buffers can also be maintained. As a result, MAC reset may not be needed.

Observation 4: For intra-DU handover based on unified TCI state, MAC reset may not be needed.

Proposal 1: RAN2 to confirm that the following L2/L3 operations are required for the different cases for L1/L2 mobility in Rel-18 as:

  - Intra-CU inter-DU: PDCP recovery, RLC reestablishment and MAC reset are needed to support L1/L2 mobility

  - Intra-DU: partial MAC reset is needed to support L1/L2 mobility

  - Intra-DU based on unified TCI state: MAC reset may not be needed to support L1/L2 mobility

## 2.3 Trigger L1/L2 based handover

Another question is how L1/L2 mobility is triggered. Regarding the triggering of handover, the trigger could be L1/L2 signaling from the network (e.g., after a L3 measurement report or a L2 measurement event is notified by UE) or it can be CHO-like based on L1/L2/L3 measurement from the UE:

Option 1: L1/L2 signaling from network, i.e., based on DCI in physical layer or MAC CE in MAC layer

Figure 4: intra-DU handover based on L1/L2 signaling (RACH may be optional in some cases)

In this option, DCI can be used to trigger L1/L2 mobility. Then the UE can send a MAC CE to confirm handover is complete. If the handover is triggered by MAC CE (from network), a UE can send HARQ ACK to network for confirmation of successful reception. The candidate cell ID/PCI may need in be included in DCI/MAC CE payload when multiple candidate cells pre-configured is supported. If PCI is used, network may need to ensure that there is no PCI confusion in inter-frequency case.

Legacy measurement event trigger can be used for the network to determine when to trigger L1/L2 mobility, however there may be some delay. On the other hand, UE can reduce latency by sending L2 measurement report to the network.

Option 2: CHO-like based on L1/L2/L3 measurement initiate from the UE

In legacy CHO, an execution condition is configured for each candidate cell, and this execution condition (such as event A3/A5) is based on L3 measurement. When the entering condition of event A3 and/or A5 are met and time to trigger is expired, this means the execution condition of this candidate cell is met, then a CHO execution is triggered. Similarly, CHO can apply to L1/L2 mobility. However, the handover should be triggered based on L1/L2 measurement results. The network may configure execution condition for each candidate cell based on L1/L2 measurement. Figure 5 shows an example of the signaling flow for intra-DU handover based on L1/L2/L3 measurement. RACH may be options in some cases.

Figure 5: intra-DU handover based on L1/L2/L3 measurements

Network control mobility is necessary to allow the network to have the flexibility to signal UE to fast handover. We think this can be higher priority. If times allow, we can further discuss CHO-like based L1/L2/L3 mobility initiated from the UE.

Proposal 2: RAN2 to prioritize network-based switching for L1/L2 mobility via DCI/MAC signaling.

## 2.4 DL and UL synchronization

In terms of DL and UL synchronization, we can assume no latency is introduced for at least DL synchronization as the UE has performed measurements on target cell recently. However, as for UL synchronization, it depends if unified TCI state is configured:

- Case 1 (unified TCI state is configured): as the continuation of R17 FeMIMO, we can assume R17 UE is configured with a unified TCI state associated to target cell. In this case, the serving cell TA can be reused as target cell TA. In case of needing TA adjustment, the TA Command MAC CE should be sufficient. It means RACH procedure can be skipped.

- Case 2: If there is no unified TCI state associated to target cell, two step RACH, RACH-less handover or early RACH before the execution of handover can be considered.

In case 1, unified TCI state is simpler and further reduce latency because RACH is not needed. In case of unified TCI state cannot be assumed in case 2, different optimization to reduce latency such as two step RACH, RACH-less (however, it is only reused TA can be applied, no suitable for all cases) and early RACH can be considered. Due to the simplicity of unified TCI state and based on the guidance from Chairman “Focus first on intra-freq-intra-DU”, in the rest sections we mainly discuss scenario 2b.”

Proposal 3: RAN2 to prioritize unified TCI state based L1/L2 mobility over the non-unified TCI state based.

## 2.5 RRC modelling of scenario (2b)

Since the motivation of L1/L2 based inter-cell mobility is to reduce the latency, we would like to further study scenario (2b) as proposed in proposal 3 because it has the smallest latency as discussed in our paper [2]. The premise of scenario (2b) is that a unified TCI state associated to target cell has been activated for the UE. In this case, the RRC modelling issue needs to be addressed, i.e., the L1/L2 based inter-cell mobility is handled in which level.

During Rel-17 FeMIMO WI [3], there are 4 options heavily discussed for inter-cell beam management operation. They could be considered as the baseline for the RRC modelling of L1/L2 based handover, as RAN2 needs to consider how to model the target cell from RRC perspective when we consider the TCI state associated to target cell has been activated for the UE.

- Option 1: the target cell is considered as a Cell

  - Cell addition, modification and release are signaled via RRC

  - For intra-DU case, some part of the MAC configuration and operation are shared between source and target cell (e.g. beam/ TCI switching, TA, HARQ).

  - Pros:

    - This approach allows maximum flexibility which also future proof.

  - Cons:

    - Some L1 parameters may require limitation (some can be different from serving cell and some cannot). This can rely on network implementation or has to specify by referring to RAN1 specification.

    - Large specification effort/ complexity and signaling involve in this model for L1/L2 mobility. This mainly due to if it requires to capture what L1 parameters can be different and what can be the same. But network could solve this by implementation (gives the entire cell configuration to UE)

    - Need to support partially shared MAC operation.

    - Need to consider how to incorporate with exiting TCI state configuration associated with target cell but configured as a part of serving cell configuration.

- Option 2: the target cell is considered as an additional BWP

  - TRP activation/ deactivation can be achieved via BWP switching mechanism in current framework.

  - Pros:

    - Slightly simpler approach to support inter-cell mobility as part of BWP switching framework.

    - Support separate PHY configuration

  - Cons:

    - PHY parameters for dedicated BWP may be different. Similar to option 1, capture L1 parameters what is the same and different may be required.

    - Handle multiple active BWP operation as it is possible 2 BWPs active at the same time. It will involve RAN1 work.

    - BWP ID space may need to increase from current specification (currently BWP ID space is 4).

    - Cannot have cell-specific configuration as it is per-BWP configuration.

    - Need to consider how to incorporate with exiting TCI state configuration associated with target cell but configured as a part of serving cell configuration.

- Option 3: the target cell is considered as beam resource (e.g. TCI state, QCL-info)

  - Additional SSB set(s) from non-serving cell (TRP with different PCI) is configured.

  - Can associate TCI states, CSI measurement with index

  - Pros:

    - straightforward based on Rel-17 unified TCI framework

  - Cons:

    - how to separately configure without per cell or per BWP level configuration

Proposal 4: RAN2 to further discuss the identified options for RRC modelling of L1/L2 based handover (Option 1: Cell, Option 2: BWP, Option 3: beam resource (e.g. TCI state, QCL-info)).

# Conclusion

Proposal 1: RAN2 to confirm that the following L2/L3 operations are required for the different cases for L1/L2 mobility in Rel-18 as:

  - Intra-CU inter-DU: PDCP recovery, RLC reestablishment and MAC reset are needed to support L1/L2 mobility

  - Intra-DU: partial MAC reset is needed to support L1/L2 mobility

  - Intra-DU based on unified TCI state: MAC reset may not be needed to support L1/L2 mobility

Proposal 2: RAN2 to prioritize network-based switching for L1/L2 mobility via DCI/MAC signaling.

Proposal 3: RAN2 to prioritize unified TCI state based L1/L2 mobility over the non-unified TCI state based.

Proposal 4: RAN2 to further discuss the identified options for RRC modelling of L1/L2 based handover (Option 1: Cell, Option 2: BWP, Option 3: beam resource (e.g. TCI state, QCL-info)).

# Reference

[1] RP-213565		New WID on Further NR mobility enhancements		MediaTek

[2] R2-2207380		Discussion on latency model of L1 L2 mobility		Intel

[3] R2-2109206		Report of [AT115-e][052][feMIMO] RRC modelling 		Intel