3GPP TSG-RAN WG2 Meeting #119-e                                                                 R2-2207397

Online, Aug 17 –Aug 26, 2022

Source:	CATT

Title:	Discussion on CHO with SCG

Agenda Item:	6.2.3.2

Document for:	Discussion and Decision

Introduction

Following the previous discussions in the RAN plenary, there is one issue to check regarding whether the work related to the following objective point (see full document in [1]) has already been completed in Rel-17

To specify CHO including target MCG and target SCG [RAN3, RAN2].

Note 5: This is already being targeted for Rel-17, so this objective will be reviewed at RAN#97-e.

In this document we discuss this issue and we suggest the potential issue, if any, should be checked by RAN3. It is our preference that from RAN2 point of view we can consider this task already done in Rel-17.

Discussions

In the previous RAN2 meeting the topic was discussed [2], a set of CRs have been agreed by RAN2 [3-7].

Then in the RP#96 meeting [8], there were different views regarding whether the related work has already been properly completed in Rel-17, and as a result whether the bullet (as shown in section 1) could be removed from the Rel-18 work scope.

Based on the discussions so far, the main potential issue that has been mentioned is for the case when SN changes its configuration without informing the MN, after the configuration of CHO with SCG has already been provided to the UE. More specifically, the target SCG configuration, as a part of the CHO configuration, may use delta configuration based on the current SN. So when the mentioned case happened it is problematic for UE to apply the configuration.

In our understanding, there are two possible options to handle this issue.

- Up to network implementation: If SN knows whether CHO with SCG is configured for a UE (this should be possible), it avoids changing current SCG the configuration or requires MN to update UE’s configuration if needed.

- Consider possible enhancement to signalling between the SN and the MN, e.g., let MN know that intra-SN configuration changes happened (as proposed in [9])

It seems clear that no matter which option is selected, there is no much work to do in RAN2. RAN3 may further discuss if enhancements such as in b) is necessary.

Therefore the following proposal is made.

Proposal 1	From RAN2 point of view, CHO including target MCG and target SCG is supported and no remaining issue needs to be handled.

Conclusion

The following is proposed based on the discussions

Proposal 1	From RAN2 point of view, CHO including target MCG and target SCG is supported and no remaining issue needs to be handled.

Reference

[1] RP-221799, Revised WI: Mobility Enhancements

[2] R2-2206599	Report of [AT118-e][036][TEI17] CHO with SCG (CATT)	CATT

[3] R2-2206600   Support of CHO with SCG configuration - 36331 [CHOwithDCkept] CATT, Huawei, ZTE, China Unicom, China Telecommunications, CMCC, Ericsson CR   Rel-17  36.331  17.0.0   4823     -C        TEI17

[4] R2-2206601   Support of CHO with SCG configuration - 38331 [CHOwithDCkept]   CATT, Huawei, ZTE, China Unicom, China Telecommunications, CMCC, Ericsson  CR       Rel-17  38.331  3183     -17.0.0  C          TEI17

[5] R2-2206602   Support of CHO with SCG configuration - 37340 [CHOwithDCkept] CATT, Huawei, ZTE, China Unicom, China Telecommunications, Nokia, Nokia Shanghai Bell, CMCC, Ericsson    CR Rel-17  37.340  0329     -   17.0.0   C          TEI17

[6] R2-2206709   Introduction UE capability for CHO with SCG configuration             MediaTek Inc          CR       Rel-17  38.331  3179     -           17.0.0   C          TEI17

[7] R2-2206710   Introduction UE capability for CHO with SCG configuration             MediaTek Inc          CR       Rel-17  38.306  0746     -           17.0.0   C          TEI17

[8] RP#96 meeting notes, see https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_96/Report

[9] RP-221433 Rel-18 MobEnh scope