3GPP TSG-RAN WG2 #119 electronic	R2-2207410

17th – 26th Aug 2022

Online

Source:                    	DENSO CORPORATION

Title:  	Discussion on XR-specific capacity improvements

Document for:        	Discussion and decision

Agenda Item:         	8.5.4		XR-specific capacity improvements

- Introduction

In RAN #95, revised SID of XR enhancements was approved [1], and the following objectives are specified regarding XR-specific capacity improvements.

Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

In this contribution, we discuss the potential enhancements for XR-specific capacity improvements.

- Discussion

XR service has multiple traffic flows with different characteristics. According to TR 38.838 [2], for UL pose/control traffic model, fixed-size packets arrive at a UE periodically with no jitter. It is suitable to use configured grant (CG) for the UL traffic with such characteristics. Once activated, CG allows periodic resource allocations without having to transmit DCI, which can reduce the control signalling overhead. In addition, since the UE does not need to transmit a scheduling request (SR) and wait for new resource allocations, CG is useful for minimizing the transmission delay to meet the packet delay budget (PDB) of XR service.

Observation 1:	For UL pose/control traffic, CG can be used to reduce the DCI overhead and the transmission delay.

Proposal 1:	CG enhancements could be discussed for UL pose/control traffic.

And then, DL/UL video frame packets also arrive periodically, therefore semi-persistent scheduling (SPS) and CG based transmission can be considered. However, SPS/CG is configured to use fixed time/frequency domain resources and MCS, and these parameters cannot be dynamically changed depending on the channel condition and packet size. This mechanism might not be optimal for video traffic with variable packet size. Allocating more resources than necessary, or vice versa causes waste of radio resources and additional scheduling delay. Dynamic grant (DG) is beneficial in this aspect. Furthermore, DG allows to freely allocate resources in time domain when packets arrive, so it can also deal with quasi-periodic traffic with random jitter. For the above reasons, it seems that there are many advantages to using DG over SPS/CG for video traffic.

Observation 2:	For DL/UL video traffic, DG is suitable to deal with variable packet size and random jitter.

Proposal 2:	DG transmission could be enhanced for DL/UL video traffic.

In the case of UL DG, a UE requests resources for a new UL transmission by SR and buffer status reporting (BSR) mechanisms. Through BSR procedure, the UE provides the base station with the information regarding the UL data volume. This helps the base station allocate as much resources as the UE needs. However, there is a gap between the buffer size reported by BSR and the actual one because BSR reports the indexes corresponding to buffer sizes using the look-up table described in TS38.321 [3]. The base station may overestimate the UL data volume and allocate more resources than necessary because of this gap. It results in waste of radio resources and capacity loss. For XR video traffic characterized by large and variable packet sizes, the negative impact on capacity could be considerable. Therefore, in order to improve capacity, BSR enhancements could be studied so that it can report buffer sizes more precisely.

Observation 3:	There is a gap between the buffer size reported by BSR and the actual one, which can lead to waste of radio resources and capacity loss for XR video traffic.

Proposal 3:	It is beneficial to study BSR enhancements to improve capacity.

- Summary and proposal

In this contribution, we discussed the potential enhancements for XR-specific capacity improvements and provided the following observations and proposals.

Observation 1:	For UL pose/control traffic, CG can be used to reduce the DCI overhead and the transmission delay.

Proposal 1:	CG enhancements could be discussed for UL pose/control traffic.

Observation 2:	For DL/UL video traffic, DG is suitable to cope with variable packet size and random jitter.

Proposal 2:	DG transmission could be enhanced for DL/UL video traffic.

Observation 3:	There is a gap between the buffer size reported by BSR and the actual one, which can lead to waste of radio resources and capacity loss for XR video traffic.

Proposal 3:	It is beneficial to study BSR enhancement to improve capacity.

- References

[1]	RP-220285, Revised SID: Study on XR Enhancements for NR, Nokia

[2] 3GPP TR 38.838 V17.0.0 (2021-12)

[3] 3GPP TS 38.321 V17.1.0 (2022-06)