3GPP TSG-RAN WG2 Meeting #119-e                              R2-2207412

Electronic meeting, 18th – 26th August 2022

Agenda item:	8.11.2

Source:	TCL

Title:	State transition for UEs receiving Multicast in RRC_INACTIVE state

Document for:	Discussion and Decision

- Introduction

At RAN#94-e, a new RAN2 work item on ‘Enhancements of NR Multicast and Broadcast Services’ [1] was agreed. Thegoal of this work item is to further enhance the NR Multicast/Broadcast functions based on Rel -17 MBS. One of the objectives in the WID mentioned is as the following.

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]

In this contribution, we will provide initial discussion on the impact of state transition for UEs receiving multicast in RRC_INACTIVE.

- Discussion

In R17, the UE receiving the multicast service must remain in the connected state (i.e., RRC_CONNECTED state) while receiving MBS. However, in R18, UEs receiving multicast services are allowed to stay in an inactive state (i.e., RRC_INACTIVE) while receiving MBS. It means that UEs are allowed to transit from RRC connected state to the inactive state and vice versa while receiving multicast services. The way and the procedure of when and how the UE performs the state transition is an open issue in R18 that needs to be addressed.

For UEs receiving multicast services, the state transition includes two types:

- A state transition from the connected state to the inactive state; and

- A state transition from the inactive state to the connected state.

For the first type of transition (from the connected state to the inactive state), the network can fully configure and control UE in the connected state. It can be up to the network implementation to decide when UE needs to transition from a connected state to an inactive state. The state transition procedure could be like the following description.

A UE initially operates in the connected state to receive multicast services. And gNB provides a measurement configuration, including measurement objects, a reporting criterion, and other configurations, to the UE. When the reporting criterion is met, the UE performs measurement according to the measurement configuration and transmits measurement reports to the base station.  And gNB determines whether the UE is allowed to go to the inactive state based on the measurement reports or to be kept in RRC connected. When it is determined by the gNB that the UE is allowed to go to the inactive state, the gNB sends an RRC release with suspension configuration to the UE. In a response to the RRC release with suspension configuration, the UE performs a state transition from connected state to inactive state for multicast services reception.

Proposal 1: The UE, transition procedure from RRC connected state to inactive state for multicast services reception can be left to network implementation.

For the second type of transition (i.e., from the inactive state to the connected state), the network is not able to configure or control the UEs in the inactive state. Therefore, when a UE needs to transition from the inactive state to the connected state should be decided by the UE, which means UE should trigger a state transition procedure autonomously. Therefore the state transition procedure from inactive state to connected state for UEs receiving multicast should be discussed in RAN2.

Proposal 2: Recommend RAN2 to discuss the state transition procedure from inactive state to connected state for UEs receiving multicast.

- Conclusion

Based on the above discussion, we make the following observations and recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: The UE, transition procedure from RRC connected state to inactive state for multicast services reception can be left to network implementation..

Proposal 2: Recommend RAN2 to discuss the state transition procedure from inactive state to connected state for UEs receiving multicast.

Reference

- RP-213568, New WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting #94-e.