3GPP TSG-RAN2 Meeting #119	R2-2207450

Electronic, 17-26 August, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0510 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on user plane and control plan procedures for U2N relay in Stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - For the description of mapping from ingress RLC channel to egress RLC channel, we need clarify that L2 U2N relay UE need to determine the correct PC5 link first because PC5 RLC Channels are only meaningful for a particular PC5 link to a particular remote UE. - Remote UE cannot determine the right PDCP entity for received DL packets by just based on bearer ID in SRAP header, because it cannot differentiate SRB1/2/3 from DRBs. It is better to clarify how the differentiation of SRB/DRB is done. - “Double negative” grammar error in SRB/DRB separation description by using both “neither…nor…” and “cannot”. - The Note in RRC Connection establishment procedure uses the wrong message name “RRCSetup”. It should be “RRCSetupRequest”. - Also, the “due to remote UE” phrase is unclear, it is better to be clarified. - When Uu RLF is detected, the relay will need send multiple PC5-RRC messages respectively for multiple connected remote UEs, one per each remote UE, so it is misleading to say “a PC5-RRC message” to “remote UE(s)”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that relay UE need map the ingress traffic based on local ID to a particular PC5 link and then the specific egress RLC channel. - Clarify that egress PC5 RLC channels are “over one or more egress PC5 links towards different remote UEs “ - Explained that SRAP layer in remote UE need differentiate Uu SRB/Uu DRB for incoming traffic. - Rephrase the “SRB/DRB separation” requirements to fix the double negative issue. - Change “due to remote UE” to “due to remote UE’s connection establishment”. - Change “RRCSetup” to “RRCSetupRequest” - Add “(s)” in “PC5-RRC message” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect Stage 2 text will remain in the specifcaiton and misguide implementation for UE-to-NW relay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.12.2.1, 16.12.5.1, 16.12.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================Begin of Change==== ========================

#### 16.12.2.1	L2 UE-to-Network Relay

The protocol stacks for the user plane and control plane of L2 U2N Relay architecture are illustrated in Figure 16.12.2.1-1 and Figure 16.12.2.1-2. The SRAP sublayer is placed above the RLC sublayer for both CP and UP at both PC5 interface and Uu interface. The Uu SDAP, PDCP and RRC are terminated between L2 U2N Remote UE and gNB, while SRAP, RLC, MAC and PHY are terminated in each hop (i.e. the link between L2 U2N Remote UE and L2 U2N Relay UE and the link between L2 U2N Relay UE and the gNB).

For L2 U2N Relay, the SRAP sublayer over PC5 hop is only for the purpose of bearer mapping. The SRAP sublayer is not present over PC5 hop for relaying the L2 U2N Remote UE's message on BCCH and PCCH. For L2 U2N Remote UE's message on SRB0, the SRAP header is not present over PC5 hop, but the SRAP header is present over Uu hop for both DL and UL.

Figure 16.12.2.1-1: User plane protocol stack for L2 UE-to-Network Relay

Figure 16.12.2.1-2: Control plane protocol stack for L2 UE-to-Network Relay

For L2 U2N Relay, for uplink:

-	The Uu SRAP sublayer performs UL bearer mapping between ingress PC5 Relay RLC channels for relaying and egress Uu Relay RLC channels over the L2 U2N Relay UE Uu interface. For uplink relaying traffic, the different end-to-end Uu Radio Bearers (SRBs or DRBs) of the same L2 U2N Remote UE and/or different L2 U2N Remote UEs can be multiplexed over the same egress Uu Relay RLC channel;

-	The Uu SRAP sublayer supports L2 U2N Remote UE identification for the UL traffic. The identity information of L2 U2N Remote UE end-to-end Uu Radio Bearer and a local Remote UE ID are included in the Uu SRAP header at UL in order for gNB to correlate the received packets for the specific PDCP entity associated with the right end-to-end Uu Radio Bearer of the L2 U2N Remote UE;

-	The PC5 SRAP sublayer at the L2 U2N Remote UE supports UL bearer mapping between L2 U2N Remote UE end-to-end Uu Radio Bearers and egress PC5 Relay RLC channels.

For L2 U2N Relay, for downlink:

-	The Uu SRAP sublayer performs DL bearer mapping at gNB to map end-to-end Uu Radio Bearer (SRB, DRB) of L2 U2N Remote UE into Uu Relay RLC channel over L2 U2N Relay UE Uu interface. The Uu SRAP sublayer performs DL bearer mapping and data multiplexing between multiple end-to-end Radio Bearers (SRBs or DRBs) of a L2 U2N Remote UE and/or different L2 U2N Remote UEs and one Uu Relay RLC channel over the L2 U2N Relay UE Uu interface;

-	The Uu SRAP sublayer supports L2 U2N Remote UE identification for DL traffic. The identity information of L2 U2N Remote UE end-to-end Uu Radio Bearer and a local Remote UE ID are included into the Uu SRAP header by the gNB at DL for the L2 U2N Relay UE to map the received packets from L2 U2N Remote UE end-to-end Uu Radio Bearer to the corresponding egress PC5 link towards the remote UE and its associated PC5 Relay RLC channel;

-	The PC5 SRAP sublayer at the L2 U2N Relay UE performs DL bearer mapping between ingress Uu Relay RLC channels and egress PC5 Relay RLC channels over one or more egress PC5 links towards different remote UEs;

-	The PC5 SRAP sublayer at the L2 U2N Remote UE correlates the received packets for the specific PDCP entity associated with the right end-to-end Radio Bearer of the L2 U2N Remote UE based on the identity information included in the PC5 SRAP header and whether the ingress PC5 Relay RLC channel of the received packet is configured to multiplex end-to-end SRB(s) or end-to-end DRB(s).

A local Remote UE ID is included in both PC5 SRAP header and Uu SRAP header. L2 U2N Relay UE is configured by the gNB with the local Remote UE ID(s) to be used in SRAP header. Remote UE obtains the local Remote UE ID from the gNB via Uu RRC messages including RRCSetup, RRCReconfiguration, RRCResume and RRCReestablishment.

The end-to-end DRB(s) or end-to-end SRB(s) of L2 U2N Remote UE can be multiplexed to the PC5 Relay RLC channels and Uu Relay RLC channels in both PC5 hop and Uu hop, but any end-to-end DRB and any end-to-end SRB can neither be mapped into the same PC5 Relay RLC channel nor be mapped into the same Uu Relay RLC channel.

It is the gNB responsibility to avoid collision on the usage of local Remote UE ID. The gNB can update the local Remote UE ID by sending the updated local Remote UE ID via RRCReconfiguration message. The serving gNB can perform local Remote UE ID update independent of the PC5 unicast link L2 ID update procedure.

========================Next Change==== ========================

#### 16.12.5.1	RRC Connection Management

The L2 U2N Remote UE needs to establish its own PDU sessions/DRBs with the network before user plane data transmission.

The NR V2X PC5 unicast link establishment procedures can be used to setup a secure unicast link between L2 U2N Remote UE and L2 U2N Relay UE before L2 U2N Remote UE establishes a Uu RRC connection with the network via L2 U2N Relay UE.

The establishment of Uu SRB1/SRB2 and DRB of the L2 U2N Remote UE is subject to Uu configuration procedures for L2 UE-to-Network Relay.

The following high level connection establishment procedure in Figure 16.12.5.1-1 applies to L2 U2N Relay:

Figure 16.12.5.1-1: Procedure for L2 U2N Remote UE connection establishment

1.	The L2 U2N Remote and L2 U2N Relay UE perform discovery procedure, and establish PC5-RRC connection using NR sidelink PC5 unicast link establishment procedure.

2.	The L2 U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the L2 U2N Relay UE, using a specified PC5 Relay RLC channel configuration. If the L2 U2N Relay UE is not in RRC_CONNECTED, it needs to do its own connection establishment upon reception of a message on the specified PC5 Relay RLC channel. During L2 U2N Relay UE's RRC connection establishment procedure, gNB may configure SRB0 relaying Uu Relay RLC channel to the U2N Relay UE. The gNB responds with an RRCSetup message to L2 U2N Remote UE. The RRCSetup message is sent to the L2 U2N Remote UE using SRB0 relaying channel over Uu and a specified PC5 Relay RLC channel over PC5.

NOTE 1:	During the L2 U2N Relay UE’s RRC connection establishment procedure due to Remote UE's connection establishment, it is left to the L2 U2N Relay UE’s implementation on how to set the establishment cause value in the RRCSetupRequest message.

3.	The gNB and L2 U2N Relay UE perform relaying channel setup procedure over Uu. According to the configuration from gNB, the L2 U2N Relay/Remote UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the L2 U2N Remote/Relay UE over PC5.

4.	The RRCSetupComplete message is sent by the L2 U2N Remote UE to the gNB via the L2 U2N Relay UE using SRB1 relaying channel over PC5 and SRB1 relaying channel configured to the L2 U2N Relay UE over Uu. Then the L2 U2N Remote UE is RRC connected over Uu.

5.	The L2 U2N Remote UE and gNB establish security following Uu procedure and the security messages are forwarded through the L2 U2N Relay UE.

6.	The gNB sends an RRCReconfiguration message to the L2 U2N Remote UE via the L2 U2N Relay UE, to setup the SRB2/DRBs for relaying purpose. The L2 U2N Remote UE sends an RRCReconfigurationComplete message to the gNB via the L2 U2N Relay UE as a response. In addition, the gNB may configure additional Uu Relay RLC channels between the gNB and L2 U2N Relay UE, and PC5 Relay RLC channels between L2 U2N Relay UE and L2 U2N Remote UE for the relay traffic.

========================Next Change==== ========================

#### 16.12.5.2	Radio Link Failure

The L2 U2N Remote UE in RRC_CONNECTED suspends Uu RLM (as described in clause 9.2.7) when L2 U2N Remote UE is connected to gNB via L2 U2N Relay UE.

The L2 U2N Relay UE declares Uu Radio Link Failure (RLF) following the same criteria as described in clause 9.2.7.

After Uu RLF is declared, the L2 U2N Relay UE takes the following action on top of the actions described in clause 9.2.7:

-	PC5-RRC message(s) can be used for sending an indication to its connected L2 U2N Remote UE(s), which may trigger RRC connection re-establishment for L2 U2N Remote UE.

Upon detecting PC5 RLF, the L2 U2N Remote UE may trigger connection re-establishment.

========================End of Change==== ========================