3GPP TSG-RAN2 Meeting #119	R2-2207453

Electronic, 17-26 August, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0007 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SRAP header handling in L2 Relay UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For DL SRB0, the SRAP header is removed before transmitting in PC5 hop, so it is incorrect to use “BEARER ID = 0” as a condition to check this. Instead, this shall be simply checked by whether the SRAP header is present or not. Also, the SRB/DRB differentiation is only needed for remote UE receiving operation, but not needed for relay UE operation. For the mapping configured by gNB to relay UE in sl-SRAP-config, Uu RLC channels for SRB shall only be mapped to PC5 RLC channels for SRB. Uu RLC channels for DRB shall be only mapped to PC5 RLC channels for DRB. Thus, when the relay UE matches the local ID, BEARER ID and RLC channel index, there shall be only one unique satisfying mapping. The relay UE shall simply map ingress RLC channel to egress RLC channel based on this single valid mapping, w/o having an SRB/DRB differentiation step to examine whether the end-to-end RB is a SRB or DRB . Moreover, as the channel mapping configuration IEs always use the name “Egress”, but the assumption is that “egress” and “ingress” share the same LCID for the relay UE operation. When relay UE receiving SRAP data, there is no information or index in the headers to identify sl-EgressRLC-ChannelPC5 or sl-EgressRLC-ChannelUu , but only LCID in MAC subheader. So, it is important to clarify that the receiving relay UE need map LCID to the ingress Relay RLC channel ID with the configured mapping begins with the name “Egress”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Fixed the condition to check DL SRB0 by replacing “BEARER ID =0” with “SRAP header is absent”. - Removed the SRB/DRB differentiation part for relay UE operation in egress channel determination. - Added the clarification that relay UE need matching the LCID (where the SRAP PDU is received) to the “entry of "sl-EgressRLC-ChannelUu” or “sl-EgressRLC-ChannelPC5“ to determine the correct mapping. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How L2 Relay UE handle SRAP Data PDU will not be correctly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2, 5.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========================Begin of Change==== ========================

#### 5.2.2.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if the SRAP Data PDU is for SRB0 (i.e., the SRAP header is absent):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3];

-	else if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, and the SRAP Data PDU comes from the Uu Relay RLC Channel of the same LCID matching the sl-EgressRLC-ChannelUu associated with an sl-RemoteUE-RB-Identity that matches the BEARER ID field of the SRAP Data PDU,

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3];

========================Next Change============================

#### 5.3.3.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	if there is an entry in sl-SRAP-Config-Relay, whose sl-LocalIdentity matches the UE ID field in SRAP Data PDU, and and the SRAP Data PDU comes from the PC5 Relay RLC Channel of the same LCID matching the sl-EgressRLC-ChannelPC5 associated with an sl-RemoteUE-RB-Identity that matches the BEARER ID field of the SRAP Data PDU,

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3];

========================End of Changes============================