3GPP TSG-RAN WG2 Meeting #119 electronic	 R2-2207480

Online, August 17th- August 29th, 2022

Agenda Item:    8.13.4

Source:         Xiaomi

Title:         	Considerations on signaling based logged MDT override protection for E-UTRAN

Document for:	Discussion and Decision

Introduction

Currently, UE only maintains one RAT-specific logged measurement configuration for Logged MDT, when network provides a newly configuration, any previously configured logged measurement configuration will be entirely replaced by the new one and the logged measurements corresponding to the previous configuration will be cleared at the same time.

According to the WID[1] of the Rel-18 SON/MDT enhancement, one objective is to support the signaling based logged MDT override protection for E-UTRAN during the inter-RAT mobility from E-UTRAN to NR, as quoted as follows:

- Support of signalling based logged MDT override protection to address the scenario where the signalling based MDT is configured in E-UTRAN when [RAN2, RAN3]:

- UE reselects to NR while logged measurements are collected

- UE reselects to NR after logged measurements are collected and before uploading the logged MDT report.

In this contribution, we provide our considerations on the override protection mechanism for the signalling based logged MDT configured in E-UTRAN.

Discussion

In Rel-17, the override protection mechanism has been introduced to avoid overwriting of signaling-based logged MDT by the management-based logged MDT in NR. And there are two options proposed in Rel-17 which can also be considered for the signaling based logged MDT override protection for E-UTRAN.

Option 1: UE-based solution, which is UE rejects network configuration.

Option 2: UE-assisted and network-based solution, which relying on network implementation through UE providing assistance.

In the R17 override protection mechanism, the UE-assisted and network based solution is finally adopted assuming the network configuration is always proper, UE provides gNB with the assistance information every time when UE accesses to the network, including the signaling based logged MDT configuration availability for NR(whether is configured and whether the T330 is running) and logged MDT measurements availability for NR. Upon reception of the assistance information, NW shall be able to avoid the logged MDT being overwritten.

In R18, to support the signaling based logged MDT override protection for E-UTRAN when UE reselects to NR, similar to the R17 mechanism, the UE-assisted and network based override protection mechanism can be considered as preference, when UE reselects to the NR, UE provides gNB with the availability indication of the signaling based logged MDT configuration for E-UTRAN (whether is configured and whether the T330 is running) and the logged MDT measurements for E-UTRAN.

Proposal 1: The following options can be considered for the signaling based logged MDT override protection for E-UTRAN while the option2 is preference.

Option 1: UE-based solution, which is UE rejects network configuration.

Option 2: UE-assisted and network-based solution, which relying on network implementation through UE providing assistance.

Proposal 2: To support the signaling based logged MDT override protection for E-UTRAN when UE reselects to NR, UE can indicate assistance information to gNB as follows:

- Whether the signaling based logged MDT is configured in the E-UTRAN

- T330 timer status for the E-UTRAN

- Report availability for the E-UTRAN

Conclusions

During the discussion above, we have the following observations and proposals:

Proposal 1: The following options can be considered for the signaling based logged MDT override protection for E-UTRAN while the option2 is preference.

Option 1: UE-based solution, which is UE rejects network configuration.

Option 2: UE-assisted and network-based solution, which relying on network implementation through UE providing assistance.

Proposal 2: To support the signaling based logged MDT override protection for E-UTRAN when UE reselects to NR, UE can indicate assistance information to gNB as follows:

- Whether the signaling based logged MDT is configured in the E-UTRAN

- T330 timer status for the E-UTRAN

- Report availability for the E-UTRAN

References

RP-221825, CMCC, “New WID on further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)”.