3GPP TSG-RAN WG2 Meeting #119-e                                                   R2-2207508

Electronic, 17th – 26th August, 2022

Source:              CATT

Title:	             XR requirements and issues

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

A new study item was agreed in RAN#94-e addressing “XR Enhancements for NR” [1]. Both SA and RAN1 started working on this topic in earlier e-meetings and this contribution discusses the resulting requirements and issues to solve in RAN2.

Discussion

SA1 requirements

As usual, the key performance requirements are coming from SA1. TS 22.261 [2] provides the Service requirements for the 5G system where clause 7.6.1 specifically addresses the performance requirements for AR/VR, captured below in the KPI Table 7.6.1-1.

As a first observation, we can see from the below table that the rate of 120 frames per second results in a 8.333 ms traffic periodicity, which is currently not supported by CG, SPS, or DRX periods. And 5ms end-to-end latency and 10E-4 PER (= 1 – 99.99%) reflect the most stringent SA1’s XR requirements for the e2e latency and PER, respectively.

Table 7.6.1-1 KPI Table for additional high data rate and low latency service

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area (note 2) |
| Cloud/Edge/Split Rendering (note 1) | 5 ms (i.e. UL+DL between UE and the interface to data network) (note 4) | 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content. | 99,99 % in uplink and 99,9 % in downlink (note 4) | - | Stationary or Pedestrian | Countrywide |
| Gaming or Interactive Data Exchanging (note 3) | 10ms (note 4) | 0,1 to [1] Gbit/s supporting visual content (e.g. VR based or high definition video) with 4K, 8K resolution and up to120 frames per second content. | 99,99 % (note 4) | ≤ [10] | Stationary or Pedestrian | 20 m x 10 m; in one vehicle (up to 120 km/h) and in one train (up to 500 km/h) |
| Consumption of VR content via tethered VR headset (note 6) | [5 to 10] ms (note 5) | 0,1 to [10] Gbit/s (note 5) | [99,99 %] | - | Stationary or Pedestrian | - |
| NOTE 1: Unless otherwise specified, all communication via wireless link is between UEs and network node (UE to network node and/or network node to UE) rather than direct wireless links (UE to UE). NOTE 2: Length x width (x height). NOTE 3: Communication includes direct wireless links (UE to UE). NOTE 4: Latency and reliability KPIs can vary based on specific use case/architecture, e.g. for cloud/edge/split rendering, and may be represented by a range of values. NOTE 5: The decoding capability in the VR headset and the encoding/decoding complexity/time of the stream will set the required bit rate and latency over the direct wireless link between the tethered VR headset and its connected UE, bit rate from 100 Mbit/s to [10] Gbit/s and latency from 5 ms to 10 ms. NOTE 6: The performance requirement is valid for the direct wireless link between the tethered VR headset and its connected UE. |  |  |  |  |  |  |

SA2 requirements

SA2 complemented the SA1 requirements with further details on the specifics of the XR traffic and QoS in their LS [4] to SA4, cc RAN1/2/3, asking SA4 feedback on those. Specifically, the LS describes two aspects of the XR traffic, namely the Group Of Pictures (GOP) and Video Slice/PDU Set concepts referring to [6] and [5], respectively. We quote below extracts of [6] and [5], underlining the aspects that we believe have RAN2 impacts.

NOTE: In their reply LS [7], SA4 does not 100% back-up SA2 definitions, and essentially comments that there are cases where they apply and cases where they don’t. More work is needed from their side but it is not clear either when something more accurate will be provided. So our take is that it should not block starting the work in RAN2 based on SA2’s assumptions and definitions (e.g. PDU set concept and I/P/B frames). In any case, if we need to adjust later, based on further SA4 inputs, we can adjust.

Group of Pictures (GOP) [6]

A video consists of a sequence of consecutive video frames. Each video frame is a picture encoded/compressed using different codec mechanisms (e.g. H.264/H.265/H.266, AV1 or AVS) for efficient storage and transmission. During encoding, the video spatial and temporal redundancy can be removed using, e.g. inter-frame/intra-frame prediction, motion estimation/compensation. Typically, three major frame/picture types can be used during encoding, i.e. I-frame, P-frame and B-frame: - I-frame, as an intra-coded picture, is a complete picture and can be encoded and decoded independently, like a JPG image file. - P-frame, as a predicted picture, is not a complete frame and only contains the image changes compared to the previous frame. If the reference frame is lost, the P-frame cannot be decoded and displayed. - B-frame, as a bidirectional predicted picture, contains the changes between the previous and following reference frames. With more reference frames, the compression ratio can be higher. However, the B-frame can only be decoded when the previous and following reference frames are available. A Group of Pictures (GOP) includes a collection of successive video frames. The first frame in a GOP is an I-frame. And the following frames can be P-frames or B-frame. […] 2) Frame importance For XR and media service, different types of frame coexist and have different importance. For example, in Group of Picture (GOP), the first I-frame is the most important frame, other P-frames are encoded based on the I-frame. That is, the decoding of P-frames depends on the decoding of I-frame in the same GOP. Therefore, frame importance should be provided to enable 5GS to perform efficient packet dropping, e.g., the non-transmitted part with low importance of GOP will be discarded at the transmitter if the packets with high importance of the GOP have been lost or exceed the delay budget.

Video Slice / PDU set [5]

Video Slice: A spatially distinct region of a video frame that is encoded separately from other regions in the same frame. Editor's Note: The Video Slice definition above needs to be confirmed by SA4. PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g., a frame or video slice for XRM Services), which are of same importance requirement at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In some cases, the application layer can still recover parts of the information unit, when some PDUs are missing.

And, from [6]:

3) PDU set granularity XR applications impose requirements in terms of PDU sets(e.g., video/audio frame/tile, Application Data Units etc.), rather than in terms of single packets/PDUs. Packets of one PDU set need to be jointly processed for XR traffics. 5G system should be aware of the PDU set in order to improve transmission performance. Basically, the PDU set SN, the packet SN within PDU set, the total packet number of the PDU set or the last packet indication should be provided to 5G system for each packet.

We illustrate both above GOP and PDU set concepts in Figure 1 below.

Figure 1: Frame, slice, GOP and PDU set

Other traffic characteristics

[6] also describes other traffic characteristics in terms of periodicity, jitter and traffic correlation:

1) Periodic and jitter Data burst of video frame are always periodic with variable packet size. For example, audio frame ususally have a periodicity of 20ms, while video stream can have different frame rate, e.g., 60/90/120 fps. Besides, pose/control packets are always periodic (e.g., 4ms) with fixed packet size. Due to the unpredictable jitter, the packet arrival time might not be exacly periodic, which leads to transmission delay or packet loss. 5GS should take both periodicity and jitter into consideration for XR and media service. […] 4) Traffic correlation Some advanced XR or media services may include multiple traffic streams such as video, audio, haptic data or sensor data etc., which are called multi-modality communication. Different traffic streams with different QoS parameters are always mapped to different QoS flows. Packets within the same traffic stream with different packet charateristics (e.g., packet importance, frame type etc.) may also be classified and mapped into different QoS flows. These QoS flows belong to the same XR and media service have strong correlations, which should be coordinated for QoS requirement, packet processing and time synchronization etc.

RAN1 traffic models

RAN1 studied the XR traffic models and their conclusions so far are captured in [7] and can be summarized as follows:

The DL traffic for VR, CG and AR is modeled as:

- single stream (video) or

- multi-stream (I+P frames or video + audio/data  or FOV + omnidirectional stream)

where the statistical parameters for packet size, jitter, single and multi-stream are captured in Tables 5.1-1, 5.1-2, 5.3-1 and 5.3-3, respectively of [7].

In UL, VR and CG traffic is reduced to the generic pose/control traffic, characterized in Table 5.2-1 of [7], while AR supports a more symmetrical UL/DL traffic, where UL streams aggregate the generic pose/control stream, with video and audio streams with same characteristics as in DL.

We summarized the various requirements from the Tables of [7] in the below summary Table:

Table 1: XR traffic streams statistical parameters

| Traffic | Period (ms) |  |  | Jitter | Rate (Mb/s) | Packet size (kbytes) | Direction | PDB | Packet success rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (120fps) | (90fps) | (60fps - baseline) |  |  |  |  |  |  |
| Video | 8.33333 | 11.1111 | 16.66667 | +/- 4ms | 45 | 93.7 +/- 50% | DL & UL | 10ms (baseline DL), 30ms UL | 99% |
| Audio + data | 10 |  |  | 0 | 1.12 | 1.4 | DL & UL | 30ms | 99% |
| Pose/ control | 4 |  |  | 0 | 0.025 | 100 bytes | UL | 10ms | 99% |

Observations

Video stream: RAN1 uses more relaxed requirements compared with SA1:

- Bit rate: RAN1 =  45Mb/s and SA1 =  0.1-1Gb/s

- Packet success rate: RAN1 = 99% and SA1 = 99.99%

Explanation: during RAN1#104-e, SA4 provided RAN1 with some trace information of XR and CG for traffic model in XR evaluation. We suggest RAN2 also sticks to these values.

Proposal 1: RAN2 aligns with RAN1’s analysis and uses XR traffic streams statistical parameters from above Table 1 as reference requirements.

From Sections 2.1, 2.2, and 2.3, we also derive the following observations:

Observation 1: Video stream involves large jitter and large packet size variation.

Observation 2: Video stream has periodicities not supported by current SPS/ CG configurations and C-DRX.

Observation 3: Multi-flow handling results in semi-periodic behavior.

Issues from RAN2 perspective

From the requirements analysis and observations of Sections 2.1-2.4, we conclude on the following RAN2 impacts that require to be studied.

Configured Grants and SPS

Configured grants and SPS are currently not suited to address XR traffic periodicities, jitter and burst size variation, thus calling for CG/SPS enhancements. It should be noted that RAN1 already made some agreements along the same lines, for example [8]:

Agreement To study whether/how to support a candidate capacity enhancement technique for XR traffic based SPS/CG transmissions, companies are encouraged to consider the following studies: - Study enhancements related to multiple PDSCHs SPS transmission occasions in a period - Study enhancements related to multiple PUSCHs CG transmission occasions in a period - Study enhancements related to dynamic adaptation of SPS/CG parameters/configurations - Study enhancements related to non-integer periodicity for SPS/CG transmissions. - Note: Other studies are not precluded, as well as the combination of the above studies. Follow the common principle for assessment of the candidate capacity enhancement technique.

Proposal 2: RAN2 should study CG and SPS enhancements to cope with XR traffic periodicity, jitter and burst size variations, following RAN1’s recommended directions.

C-DRX

C-DRX enhancements are explicitly captured in the WID [1], and justified by, at least, the observations in Section 2.4 that XR periodicities are not currently supported. Moreover, RAN1 already made the following agreements [8]:

Agreement: For power saving study of Rel-18 XR SI, CDRX enhancements to evaluate in this study item are to be selected from the following: - High priority Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity for each flow - High priority Issue 1-2: C-DRX enhancements to handle jitter - Medium priority Issue 1-3: CDRX enhancements for multiple XR traffic flows [Note 2] - Low priority Issue 1-4: CDRX enhancements to adjust to variable burst sizes and frame rate - Note: Some companies think the adjustment for variable burst sizes can be realized by existing spec already - Low priority Issue 1-5: low latency handling - Low priority Issue 1-6: SFN wraparound mismatch (if handled in RAN1) FFS: how the solutions or the combination of the solutions can handle all the identified issues. Note 1: Other considerations are not precluded Note 2: It can also be adopted for addressing issue 1-1 Note 3: Companies are encouraged to clarify or provide more details of the proposed solutions, for addressing concerns from the group.

Therefore, considering C-DRX is a MAC function, RAN2 should study the C-DRX enhancements required in support of XR traffic specific requirements.

Proposal 3: RAN2 should study C-DRX enhancements to cope with XR traffic periodicity, jitter and burst size variations, following RAN1’s recommended directions.

PDU set level QoS

We think that guaranteeing the above described PDU set level QoS requirements will potentially impact:

- L2 stack U-plane (SDAP/PDCP/RLC/MAC), for PDU-set identification

- LCP, for differentiating PDU-sets within an LCH

- BSR, (e.g. to provide finer granularity)

- Duplication (e.g. autonomous trigger to avoid loss of a PDU set or an I-frame)

Proposal 4: RAN2 should study user plane enhancements (SDAP/PDCP/RLC/MAC), enabling PDU-set identification through the UP stack.

Proposal 5: RAN2 should study MAC enhancements in support of PDU-set QoS guarantee, potentially involving LCP, BSR and duplication trigger enhancements.

Power Saving

As discussed in Sections 2.1, 2.2 and 2.3, the XR QoS requirements can be viewed as R16/17 IIOT requirements (not as stringent though) but with an additional power saving constraint. To that end, it is expected that XR UEs leverage R16/17 IIOT features, but calling for a new reading of the R16/17 URLLC/IIOT enhancements, now considering the power saving aspects. In our view, one direction for enabling power saving on IIOT features is by allowing more autonomous UE behaviors (along the lines of Survival Time support with autonomous duplication activation in R17), thus preventing UE to receive and decode the associated legacy network commands.

Proposal 6: In view of addressing the power saving requirements on top of the stringent QoS requirements, RAN2 should study potential extensions of legacy IIOT/URLLC features, now involving more UE autonomous behaviors (e.g. autonomous activation/deactivation of some features) thus saving the associated network commands and related UE reception/decoding/feedback transmission.

UE assistance

UE assistance can facilitate network decisions for appropriately scheduling and configuring the related UEs. Specifically, [10] showed that UE reporting the level of its playout buffer can help the DL scheduler resulting in significant capacity improvements.

Proposal 7: RAN2 should study UE assistance information enhancements (e.g. on playout buffer) in support of XR capacity improvements.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: RAN2 aligns with RAN1’s analysis and uses XR traffic streams statistical parameters from above Table 1 as reference requirements.

From Sections 2.1, 2.2, and 2.3, we also derive the following observations:

Observation 1: Video stream involves large jitter and large packet size variation.

Observation 2: Video stream has periodicities not supported by current SPS/ CG configurations and C-DRX.

Observation 3: Multi-flow handling results in semi-periodic behavior.

Proposal 2: RAN2 should study CG and SPS enhancements to cope with XR traffic periodicity, jitter and burst size variations, following RAN1’s recommended directions.

Proposal 3: RAN2 should study C-DRX enhancements to cope with XR traffic periodicity, jitter and burst size variations, following RAN1’s recommended directions.

Proposal 4: RAN2 should study user plane enhancements (SDAP/PDCP/RLC/MAC), enabling PDU-set identification through the UP stack.

Proposal 5: RAN2 should study MAC enhancements in support of PDU-set QoS guarantee, potentially involving LCP, BSR and duplication trigger enhancements.

Proposal 6: In view of addressing the power saving requirements on top of the stringent QoS requirements, RAN2 should study potential extensions of legacy IIOT/URLLC features, now involving more UE autonomous behaviors (e.g. autonomous activation/deactivation of some features) thus saving the associated network commands and related UE reception/decoding/feedback transmission.

Proposal 7: RAN2 should study UE assistance information enhancements (e.g. on playout buffer) in support of XR capacity improvements.

References

- RP-213587, Study on XR Enhancements for NR, RAN#94-e, Nokia

- TS 22.261, Service requirements for the 5G system

- TR 22.842, Study on Network Controlled Interactive Service (NCIS) in the 5G System (5GS)

- S2-2201803/R2-2204439, LS on QoS support with PDU Set granularity, SA2 to SA4, cc RAN1/2/3.

- S2-2201851, FS_XRM: Media Unit definition, ZTE, Nokia, Nokia Shanghai Bell, Intel, Huawei, Hisilicon, OPPO, Qualcomm, VIVO, MediaTek Inc. Ericsson

- S2-2201087, Key issue on traffic characteristics of XR and media service, Lenovo, Motorola Mobility, Huawei, HiSilicon, Vivo, CATT

- R2-2206337, LS Reply on QoS support with PDU Set granularity, SA4

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR (Release 17)

- R1-220xxxx, Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0, RAN1 Chair

- R1-2203485, NR enhancement for XR capacity improvement, CATT