3GPP TSG-RAN WG2 Meeting #119-e                                                    R2-2207509

Electronic, 17th – 26th August, 2022

Source:              CATT

Title:	             Consideration on power saving for XR services

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

A new study item was agreed in RAN#94-e addressing “XR Enhancements for NR” [1]. The following are objectives on XR-specific Power saving.

Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement.

In RAN2#118e, RAN2 received the LS from SA2 [2]. In the LS:

2. Actions: To RAN WG1, RAN WG2: ACTION: SA2 kindly asks RAN1, RAN2 to take the above information into account and clarify which type of information should be provided to the RAN for power saving enhancements for XR applications.

RAN1 has already discussed and replied the LS [3]. In this contribution, we discuss the issue from RAN2 point of view.

Discussion

1) Periodicity and start time of XR/media traffic

C-DRX is an essential component of the power saving toolbox at the UE, but also relies on some knowledge on the expected traffic pattern in time. According to the traffic models in RAN1 R17 XR study item [4] and traffic characteristic in [5], the periodicity of video traffic model is non-integer, e.g. 60/90/120 fps. These are not supported by existing C-DRX cycles. If periodicity of XR/media traffic can be provided, RAN2 can extend the periodicity of C-DRX to match with XR/media traffic. In addition, the start time of XR/media traffic can be used to determine the start subframe and slot offset of C-DRX. Thus, periodicity and start time of XR/media traffic are expected to be provided to RAN to help configuring matching parameters of C-DRX.

Proposal 1: Periodicity and start time of XR/media traffic are expected to be provided to RAN to help configuring matching parameters of C-DRX.

2) Size of a XR/media traffic burst

The definition of PDU Set is introduced in SA2 [5].

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g., a frame or video slice for XRM Services), which are of same importance requirement at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In some cases, the application layer can still recover parts of the information unit, when some PDUs are missing.

XR applications impose requirements in terms of PDU sets (e.g., video/audio frame/tile, Application Data Units etc.), rather than in terms of single packets/PDUs. The size (e.g. in bits) of a PDU set can be used for effective transmission. If the size variation of one PDU set in a XR/media traffic is small, RAN can pre-configure suitable SPS/CG resource to reduce signaling overhead which can reduce power consumption, if the characteristic can be provided. On the other hand, if the size of one PDU set in a XR/media traffic varies with a wide range, RAN may need to consider some enhancements. For example: Additional quick dynamic grant coupled with pre-configured resources can be considered, which can reduce the wake up time of UE. In brief, the characteristic of the size of PDU set, e.g the mean size, and the  size range, are expected to be provided to RAN to assist the scheduling for power saving.

Proposal 2: The characteristic of the size of PDU set, e.g the mean size, and the size range, are expected to be provided to RAN to assist the scheduling for power saving.

3) Jitter information of XR/media traffic

Due to the unpredictable jitter, the packet arrival time might not be exactly periodic, which leads to transmission delay or packet loss. If statistical info of jitter of XR/media traffic can be provided to RAN, RAN can configure suitable parameters for power saving, e.g. suitable on-duration of C-DRX.

Proposal 3: Statistical info of jitter of XR/media traffic is expected to be provided to RAN for C-DRX configuration for power saving.

4) QoS parameters of XR/media traffic

With the introduction of PDU set, delay budget of PDU set can be used for C-DRX configuration, e.g. aligning DRX configuration with the most constrained PDU set.

In [5], SA2 asked SA4 on QoS support with PDU Set granularity. One question in the LS [5] is to clarify what, if any, dependency there is between the IP packets that make up PDU Sets, e.g. a frame/”slice”. And the following is the reply from SA4 [6].

3. Clarify what, if any, dependency there is between the IP packets that make up PDU Sets, e.g. a frame/”slice”. Again, based on the response to question 1, no single comprehensive answer can be provided. In some cases there are no dependencies, in other cases dependencies exist. The examples above provide some insight into possible dependencies, from none to a dependency that the information unit is prefix-dependent to the case that if any piece is lost of an information unit, the entire unit is useless. We believe that different application media layer mappings and receiver implementations can be addressed by the PDU Set concept and the media/application layer should be able to configure the appropriate handling. Further coordination between SA2 and SA4 is encouraged.

If there is dependency definition between the IP packets that make up PDU sets for XR/media traffic, it is expected to be provided to RAN. With this information, RAN can perform early discarding with power saving, e.g. if the entire unit is useless when any piece is lost of an information unit, early discarding can be performed when one piece is transmitted with failure.

In brief, delay budget of PDU set and dependency between the packets that make up PDU sets, if any, are expected to be provided to RAN. These can be used for C-DRX configuration, efficient scheduling or early discarding for power saving.

Proposal 4: QoS parameters of XR/media traffic, e.g. delay budget of PDU set and dependency between the packets that make up PDU sets, if any, are expected to be provided to RAN, for C-DRX configuration, efficient scheduling or early discarding for power saving.

Conclusion

In this contribution, which type of information should be provided to the RAN for power saving enhancements for XR applications is analyzed from RAN2 point of view. And it is proposed:

Proposal 1: Periodicity and start time of XR/media traffic are expected to be provided to RAN to help configuring matching parameters of C-DRX.

Proposal 2: The characteristic of the size of PDU set, e.g the mean size, and the size range, are expected to be provided to RAN to assist the scheduling for power saving.

Proposal 3: Statistical info of jitter of XR/media traffic is expected to be provided to RAN for C-DRX configuration for power saving.

Proposal 4: QoS parameters of XR/media traffic, e.g. delay budget of PDU set and dependency between the packets that make up PDU sets, if any, are expected to be provided to RAN, for C-DRX configuration, efficient scheduling or early discarding for power saving.

And a reply LS should be sent to SA2 based on conclusions in RAN2.

References

- RP-213587, Study on XR Enhancements for NR, RAN#94-e, Nokia

- R2-2204523, LS on UE Power Saving for XR and Media Services (S2-2203418; contact: Nokia), SA2, LS in, RAN2#118e

- R2-22xxxxx, Reply LS on UE Power Saving for XR and Media Services (R1-2205531; contact: Qualcomm)

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR (Release 17)

- R2-2204439, LS on QoS support with PDU Set granularity (S2-2201803; contact: Intel), SA2 	LS, RAN2#118e

- R2-2206337, LS Reply on QoS support with PDU Set granularity (S4-220505; contact: Qualcomm), SA4, RAN2#118e