3GPP TSG-RAN WG2 Meeting #119-e                                                    R2-2207511

Electronic, 17th – 26th August, 2022

Source:              CATT

Title:	             Network energy savings: issues for investigation in RAN2

Agenda Item:	8.3.2

Document for:	Discussion and Decision

# Introduction

A new study item was agreed in RAN#94-e addressing “Network energy savings” [1]. This contribution discusses the resulting requirements and issues to solve / impacts in RAN2.

# Discussion

## WID: RAN2/3 related objective

This SI is RAN1-led and the WID [1] includes two RAN1 objectives (ES model and evaluation methodology) and one RAN2/3 related objective as follows:

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded

The WID further provides example scenarios including single carrier, multi-carrier and DC configurations which we interpret as NES features should be applicable to both standalone cells (e.g. single-cell gNB), as well as non-standalone cells, e.g. a cell in a multi-cell gNB or in DC configuration.

Observation 1: NES features should be applicable to both standalone cells as well as non-standalone cells i.e. in CA or DC configuration.

The WID also expresses an important note:

Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments.

## NES cell states

So far, the NES cell state model assumed in RAN1 includes three states, with state transitions between each:

- On state (normal as in legacy)

- Sleep state

- Off state

Associated RAN1 agreements extracts are as follows [2]:

Agreement For evaluation purpose, the energy consumption modeling for a BS includes at least the following: - Reference configuration - FFS other details - Note FR1 and FR2 to be separately considered for detailed parameters - Multiple power state(s) including sleep/non-sleep mode(s) with relative power, and associated transition time/energy - Scaling method to be applied at least for non-sleep mode. - FFS other details including scaling for sleep mode […] Agreement For evaluation purpose, - Study how to define sleep modes and determine the characteristics for each mode from one or multiple of the below - Relative power - Transition time - Transition energy - Other approaches are not precluded - Note: BS components that can be turned off can be considered for discussion purpose when defining the specific values of the characteristics for sleep modes. - Study whether sleep mode is defined for DL(TX) and UL(RX) jointly or separately - Study the assumption of order for BS entering/resuming from a sleep mode to another mode (sleep or non-sleep) and the associated transition time and energy, i.e. state machine which may have impact on the transition energy. […] Agreement Further study techniques and enhancements for increasing time domain energy saving opportunities by the gNB, including (but not limited to) the following aspects: - […] - Mechanism to utilize potential energy saving states or sleep modes and the transition between states from leveraging cell on/off opportunities - including studies of waking up gNB due to user traffic, or user density, or gNB receiving wake up signal - including technique to allow discovery and measurement of cells in sleep or dormant states

It is interesting to further nail down the RAN2 aspects/impacts of these NES states.

### Off Cell

We distinguish the cases of a standalone cell and a cell in CA or DC configuration.

- Case 1: Cell on/off in standalone case

In such case, we understand the off state as the cell is totally off, meaning it is invisible to both R18 and legacy UEs, in any RRC state (Idle/Inactive/Connected).

In such case the decision to turn the cell on/off can be made by a coverage gNB, which is fully left to NW implementation with only RAN3 impacts which are already supported by the specifications. A more precise and flexible approach could also be to only turn on some beams of the cell covering certain area(s), and potentially for certain UEs only. Such approach would likely involve RAN impacts with e.g. UE feedback and/or new RS types. Similarly, autonomous on/off switching by the cell itself can be envisioned but with RAN impacts.

Figure 1: Legacy cell-level (left) vs more flexible beam/area/UE-specific (right) on/off control

Observation 2: Switching on and off a standalone cell (e.g. hot-spot gNB) can be done via a coverage gNB by NW implementation involving RAN3 impacts only, which are already supported in the specifications.

Proposal 1: RAN2 should study more flexible approach(es) with likely RAN impacts e.g. powering-on specific beams only, or autonomous on/off switching by the cell itself.

- Case 2: Cell(s) on/off  in CA or DC configuration

In such configuration, we understand the “off” state can simply be mapped to the “deactivated state” of the cell. Then, as in legacy, gNB can switch on/off its cell(s) based on UEs/load distribution, and as above, the baseline solution is to leave it to gNB implementation. However, legacy behavior requires explicit activation/deactivation commands which both consume power and involve some delay in the cell activation/deactivation. Therefore, a possible improvement that clearly falls in the RAN2 domain is to enable autonomous cell activation/deactivation triggering in the UE based on conditions unambiguously known at both the UE and gNB. Examples are the implicit secondary cell activation in a CA duplication configuration, when duplication is activated, or PUCCH SCell/sSCell autonomous activation/deactivation based on UCI mapping onto it.

It should be noted that this is already part of the FFS topics in RAN1 [2]:

Agreement Further study techniques and enhancements for frequency resource usage adaptation by the gNB, including (but not limited to) the following aspects: - For operations with single-carrier or within a single CC […] - For operation with multi-carrier - […] - enhancements on Scell activation and deactivation, enhancements on Scell dormancy and dynamic Pcell switching - including triggering conditions and methods for signaling activation/deactivation - including UE group common dynamic Pcell switching

Proposal 2: RAN2 should study autonomous/implicit SCell activation/deactivation mechanisms thus saving the associated activation/deactivation signaling from gNB which both consume power and involve delay in the cell activation/deactivation.

### Sleeping Cell

The concept of sleeping cell is yet to be finalized in RAN1 and can involve the following options:

- The cell works without any UL and reduced DL common signal/channel transmission

- SSB-less cell

- …

Although not 100% designed, it is interesting to already note the related questions and issues associated with the sleeping cell from RAN2 perspective.

One issue is whether idle/inactive UEs can camp on a sleeping cell. If sufficient DL support is provided, the answer is yes, but if SSB-less means no SSB at all or SSB period larger than 80ms, MIB transmission is also impacted. In such case, legacy UEs cannot camp on the sleeping cell. And even for R18 UEs, it may be possible to camp, with idle/inactive procedure impact, but the performance impact (e.g. larger access time) should be studied.

Idle/inactive UEs impact may also be different in different configurations:

- Non standalone cell (SCells in CA or DC configuration)

In such case, the sleeping cell typically is an SCell. Then, since SCell without SSB in intra-band CA has already been supported since R15, RAN1 could consider extending this to inter-band CA in R18. For intra-band, the current principle is that Connected UEs must rely on other RSs (e.g. CSI-RS) in the SSB-less cell, assumed QCL’ed with the SSB from another cell, to operate in that cell. Whether such principle can be extended to inter-band clearly is in RAN1 domain. And from network perspective, the power saving gain may be reduced if it still needs to transmit a CSI-RS in an SSB-less cell. Whether getting rid of the CSI-RS is possible also is in RAN1 domain. Anyways, the most ambitious NES enhancements might only be applicable to SCells in CA or DC configuration, when there is an anchor CC that implements no NES. For example, in such configurations, NES features could impact idle/inactive UEs which can camp on the anchor CC/CG as fallback. This has the advantage that the NW can decide to get a CC to sleep only based on the Connected UEs information (don’t need to care about idle/inactive UEs).

- Standalone cell (no CA or DC configuration)

Standalone cells should also be a target scenario for NES [1], with potentially reduced NES features/performance. In such case, given that NW gets no information from idle/inactive UEs in the cell (e.g. how many with potential future traffic, mobility status, etc) it would be safer that the sleeping cell has no impact on idle/inactive UEs (i.e. they can still camp on it). For example, if a sleeping cell operates SSB broadcast with a large period, then the legacy SMTC2/longer periodicity designed in R16 could be leveraged/extended for this purpose.

From the above discussion it appears that a sleeping cell could have different impacts on idle/inactive UEs as well as different NES benefits depending on its design and on the deployment configuration.

Proposal 3: RAN2 should study if/how legacy and R18 UEs can camp on a sleeping cell, and the related performance impact, taking into account different cell deployment scenarios.

## Other RAN2 enhancements for NES

### C-DRX enhancements

An approach for providing “sleeping” time intervals to the gNB is similar to C-DRX for UEs power saving, but where all on-duration intervals are grouped together so that the gNB can “wake-up” for serving all DRX UEs during about the same time, and then go back to sleep. This is illustrated in Figure 2.

Figure 2: Common C-DRX principle

Clearly, such configuration can be already provided by RRC to each involved UE by NW implementation. However, similar to the SCell activation/deactivation signalling discussed in Section 2.2.1, the point is to reduce as much as possible the amount of RRC signalling from the network, and the signalling enabling such above configuration can be optimized to be minimal (e.g. group-signalling).

Proposal 4: RAN2 should study C-DRX enhancements allowing supporting “sleep” periods at gNB side, with minimum signalling.

### Reduced SI transmission

NR already introduced significant NES since R15 with the design of the on-demand SI transmission, thus allowing the network to only broadcast permanently a reduced fraction of all SIBs, the other being requested by UEs on-demand. However, these on-demand requests can still be numerous considering high-density cells where the majority of UEs moving in and out are in idle/inactive. And in practice, a UE reselecting a cell will typically request the missing SIBs to that cell, although many other UEs in proximity likely already have acquired such SIBs. Thus enabling the UE to first check (e.g. via PC5-S connection, leveraging sidelink relay discovery) if other UEs have the needed SIBs, and acquire those from such UEs can save on-demand requests and so, further network SIB broadcast. Another option is to acquire those SIBs from an overlapped neighbour cell.

Proposal 5: RAN2 should study means to further reduce SI transmission, for example aiming at further reducing on-demand requests.

# Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Observation 1: NES features should be applicable to both standalone cells as well as non-standalone cells i.e. in CA or DC configuration.

Observation 2: Switching on and off a standalone cell (e.g. hot-spot gNB) can be done via a coverage gNB by NW implementation involving RAN3 impacts only, which are already supported in the specifications.

Proposal 1: RAN2 should study more flexible approach(es) with likely RAN impacts e.g. powering-on specific beams only, or autonomous on/off switching by the cell itself.

Proposal 2: RAN2 should study autonomous/implicit SCell activation/deactivation mechanisms thus saving the associated activation/deactivation signaling from gNB which both consume power and involve delay in the cell activation/deactivation.

Proposal 3: RAN2 should study if/how legacy and R18 UEs can camp on a sleeping cell, and the related performance impact, taking into account different cell deployment scenarios.

Proposal 4: RAN2 should study C-DRX enhancements allowing supporting “sleep” periods at gNB side, with minimum signalling.

Proposal 5: RAN2 should study means to further reduce SI transmission, for example aiming at further reducing on-demand requests.

# References

- RP-213554, New SI: Study on network energy savings for NR, RAN#94-e, Huawei

- RAN1 agreements