3GPP TSG-RAN WG2 Meeting #119-e                                                       R2-227514

Electronic, 17th - 26th August, 2022

Source:              CATT

Title:	             Discussion on SRAP Entity Release

Agenda Item:	6.7.2.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss whether additional triggers for SRAP entity release should be captured in RRC spec. Based on the analysis, the corresponding TP is attached in Annex.

Discussion

In TS 38.351, SRAP entity release is triggered by upper layer:

- 5.1.2 SRAP entity release When upper layers request release of an SRAP entity, UE shall: - release the SRAP entity and the related SRAP configurations.

In current RRC spec, there are two triggers for SRAP entity release listed below:

- When UE performs RRC connection re-establishment;

- When UE enters RRC_IDLE state.

- 5.3.7 RRC connection re-establishment - 5.3.7.2 Initiation … … Upon initiation of the procedure, the UE shall: … … 1> release sl-L2RelayUE-Config, if configured; 1> release sl-L2RemoteUE-Config, if configured; 1> release the SRAP entity, if configured; … …

- 5.3.11 UE actions upon going to RRC_IDLE The UE shall: … … 1> release all radio resources, including release of the RLC entity, the BAP entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs (except for broadcast MRBs), BH RLC channels, Uu Relay RLC channels, PC5 Relay RLC channels and SRAP entity; … …

But in our understanding, the above triggers for releasing SRAP entity are not enough. New SRAP entity release triggers should be introduced for relay UE and remote UE.

For L2 U2N remote UE:

In RAN2#117-e meeting, we have the following agreement:

[Easy] [22/23] Proposal 7: SRAP configuration is not stored in UE Inactive AS context when relay UE/remote UE enters RRC_INACTIVE state.

Based on the agreement above, the L2 U2N remote UE does not store the SRAP configuration when it enters RRC_INACTIVE state. Considering when the L2 U2N remote UE initiates the RRC connection resume, the remote UE may be already moved out of the coverage of relay UE, it is unnecessary to keep the SRAP entity once the L2 U2N remote UE enters RRC_INACTIVE.

Proposal 1: L2 U2N remote UE should release its SRAP entity when it enters RRC_INACTIVE.

For L2 U2N relay UE:

Based on the current spec, when the L2 U2N remote UE enters RRC_IDLE, it will release the SRAP entity, and based on Proposal 1, when the L2 U2N remote enters RRC_INACTIVE, it will also release the SRAP entity.

Consequently, for L2 U2N Relay UE, how to maintain the SRAP entity for the following two cases should also be considered:

- Case 1:  The last RRC_CONNECTED state remote UE is released.

  - Case 1.1:  For the relay UE, there is no PC5-RRC connected remote UE;

  - Case 1.2:  All the PC5-RRC connected remote UEs are in RRC_IDLE/INACTIVE.

- Case 2:  The last RRC_CONNECTED state remote UE enters RRC_IDLE/INACTIVE.

For Case 1.1, in case of there is no PC5-RRC connected remote UE, there is no need to maintain the SRAP entity of the relay UE. For Case 1.2, if all the PC5-RRC connected remote UE are in  RRC_IDLE/INACTIVE, since the SRAP entities of remote UEs are all released, the equivalent SRAP entity maintained in the relay UE should also be released.

For Case 2, similar as the analysis in Case 1.2, the equivalent SRAP entity maintained in the relay UE should also be released.

Proposal 2:  L2 U2N relay UE should release its SRAP entity when the last RRC_CONNECTED state remote UE is released or enters RRC_IDLE/RRC_INCATIVE.

Proposal 3: Agree the TP of TS38.331 in Annex.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: L2 U2N remote UE should release its SRAP entity when it enters RRC_INACTIVE.

Proposal 2:  L2 U2N relay UE should release its SRAP entity when the last RRC_CONNECTED state remote UE is released or enters RRC_IDLE/RRC_INCATIVE.

Proposal 3: Agree the TP of TS38.331 in Annex.

Reference

- TS 38.331 Radio Resource Control (RRC) protocol specification (Release 17)

Annex

---------------------------------< Text proposal of TS38.331 >-------------------------------------

5.3.5.15.2	L2 U2N Remote UE Release

The L2 U2N Relay UE shall:

1>	if the release is triggered by reception of the sl-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5 RRC connection to a L2 U2N Remote UE with SL-DestinationIdentity:

4>	perform the PC5-RRC connection release as specified in 5.8.9.5.

1>	if the L2 U2N Relay UE has no associated L2 U2N Remote UE in RRC_CONNECTED:

2> release the SRAP entity, if configured.

5.3.5.15.3	L2 U2N Remote UE Addition/Modification

The L2 U2N Relay UE shall:

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is not part of the current UE configuration (L2 U2N Remote UE Addition):

2>	if no SRAP entity has been established:

3>	establish a SRAP entity as specified in TS 38.351 [66];

2>	configure the parameters to SRAP entity in accordance with the sl-SRAP-Config-Relay;

2>	if SRB1 is included in sl-MappingToAddModList, and no dedicated PC5 Relay RLC channel configuration associated with SRB1 included in the same RRCReconfiguration message,

3>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 for the SRB1;

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is part of the current UE configuration (L2 U2N Remote UE modification):

2>	modify the configuration in accordance with the sl-SRAP-Config-Relay;

- if the SRAP entity associated with the L2 U2N Relay UE has no associated SRBs and/or associated DRBs:

2> release the SRAP entity, if configured.

- 5.3.8.3	Reception of the RRCRelease by the UE

The UE shall:

1>	delay the following actions defined in this clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	stop timer T380, if running;

1>	stop timer T320, if running;

1>	if timer T316 is running;

2>	stop timer T316;

2>	clear the information included in VarRLF-Report, if any;

1>	stop timer T350, if running;

1>	stop timer T346g, if running;

1>	if the AS security is not activated:

2>	ignore any field included in RRCRelease message except waitTime;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra:

2>	if cnType is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers;

NOTE 1:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation.

2>	if voiceFallbackIndication is included:

3>	consider the RRC connection release was for EPS fallback for IMS voice (see TS 23.502 [43]);

1>	if the RRCRelease message includes the cellReselectionPriorities:

2>	store the cell reselection priority information provided by the cellReselectionPriorities;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if deprioritisationReq is included and the UE supports RRC connection release with deprioritisation:

2>	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2>	store the deprioritisationReq until T325 expiry;

NOTE 1a:	The UE stores the deprioritisation request irrespective of any cell reselection absolute priority assignments (by dedicated or common signalling) and regardless of RRC connections in NR or other RATs unless specified otherwise.

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListNR:

4>	store the received measIdleCarrierListNR in VarMeasIdleConfig;

3>	if the measIdleConfig contains measIdleCarrierListEUTRA:

4>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

3>	if the measIdleConfig contains validityAreaList:

4>	store the received validityAreaList in VarMeasIdleConfig;

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

3>	for each RLC bearer that is not suspended:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for SRB2 (if it is resumed) and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

3>	if sdt-MAC-PHY-CG-Config is configured:

4>	configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

2>	if srs-PosRRC-Inactive is configured:

3>	apply the configuration and instruct MAC to start the inactivePosSRS-TimeAlignmentTimer;

2>	remove all the entries within VarConditionalReconfig, if any;

2>	for each measId of the MCG measConfig and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	re-establish RLC entities for SRB1;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	if timer T319a is running:

5>	replace the stored sdt-Config with the one received in the RRCRelease message;

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE):

5>	replace the C-RNTI with the value of the sl-UEIdentityRemote;

5>	replace the physical cell identity with the value of the sl-PhysCellId in sl-ServingCellInfo contained in the discovery message received from the connected L2 U2N Relay UE;

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

3>	stop the timer T319a if running;

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	sl-L2RelayUE-Config, if configured;

-	sl-L2RemoteUE-Config, if configured;

3>	store any previously or subsequently received application layer measurement reports for which no segment, or full message, has been submitted to lower layers for transmission;

NOTE 2:	NR sidelink communication related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE.

2>	suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs;

2> release the SRAP entity, if configured;

2>	if the t380 is included:

3>	start timer T380, with the timer value set to t380;

2>	if the RRCRelease message is including the waitTime:

3>	start timer T302 with the value set to the waitTime;

3>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2';

2>	if T390 is running:

3>	stop timer T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4;

2>	indicate the suspension of the RRC connection to upper layers;

2>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

1>	else

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.