3GPP TSG-RAN WG2 119-e	R2-2207534

17th August – 26th August 2022

GTW

Source:                    	NTT DOCOMO, INC.

Title:  			Discussion on selective activation

Document for:        	Discussion and decision

Agenda Item:    		8.4.3 NR-DC with selective activation cell of groups

- Introduction

In Rel18 WID of Further NR mobility Enhancements, the following objective was agreed [1]. In 119-e RAN2, An overview of the mechanism (in terms of target scenarios and triggers) is expected.

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements:

- To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

- Discussion

  - Trigger of selective activation

The following are possible use cases where selective activation, or continuous CPC/CPA without reconfiguration and re-initiation can be effective.

- The UE repeatedly performs HO/PSCell change between certain cells at an area covered by multiple cells. (e.g., event venues, large commercial facilities, etc.)

- The mobility of UE can be predicted, such as ring roads or certain routes (e.g., railway). And the UE performs HO/PSCell change in their predictable order.

In both cases, it is common to keep the pre-defined settings for multiple candidate cells for efficient mobility, but in case 2, it is not necessary to keep all of them (they can be discarded as needed) since the movement is in one direction.

While selective activation is expected to be effective in the above use cases, maintaining some configurations for selective activation by UE and network involves resource consumption. Therefore, it is better to have a mechanism to trigger selective activation when it is effective.

Fig1. Mobility pattern of the UE matches use case

For this purpose, it is necessary for the UE itself or the network to determine if the mobility pattern of the UE is such that the use cases described above is applicable like Fig1. Based on the mobility pattern of the UE, configuration of appropriate candidate cells to achieve efficient mobility. The mobility pattern of the UE can be categorized by referring to information such as UE history information which contains information about cells that a UE has been served and UE trajectory which contains the UE location and velocity, timestamp. Other information that can be used to identify the mobility pattern of the UE should be discussed

Observation1: selective activation is for use case such as event venues or railway

Proposal1: RAN2 study mechanism to trigger selective activation based on the UE mobility pattern.

Observation2: Mobility pattern can be derived from Information of UE mobility (e.g., UE history information. UE trajectory). And if other information for deriving mobility pattern can be provided by UE should be discussed.

  - The UE and the network maintain configurations for selective activation

Fig2. MN indicates UE and SN to maintain configurations

Next, we discuss the configurations that the UE and NW should maintain.

In the legacy CPC/CPA, if a conditional reconfiguration of suitable candidate cell is executed, the UE remove other candidate conditional reconfigurations. For selective activation, the UE should maintain the candidate conditional reconfigurations for the CPC/CPA by indication from the network to perform selective activation. In addition, from a resource management perspective, The UE should also be able to discard unnecessary configurations. e.g. the configuration of cells that UE is not going to access any more.

In S-SN and candidate T-SN, the UE configurations are conventionally released. (e.g., S-SN releases UE context according to SN RELEASE REQUEST).

For selective activation, S-SN should maintain UE context to re-connect even if the cell was once disconnected. And candidate T-SN needs to maintain dedicated RACH resource. It also needs to maintain the C-RNTI assigned to the UE to avoid collisions with other UEs. The maintain of these configurations should be indicated to the SN by the MN.

Observation3: In, S-SN and other candidate T-SN, UE configuration (UE context, C-RNTI, dedicated RACH config) needs to be reserved when UE access the target candidate cell, otherwise C-RNTI and RACH collision may happen with other UEs.

Proposal3: MN need to indicate candidate T-SN to maintain UE configuration (C-RNTI, dedicated RACH config) like Fig2

- Conclusion

Based on the above discussion, we have the following observations and proposals:

Observation1: selective activation is for use case such as event venues or railway

Proposal1: RAN2 study mechanism to trigger selective activation based on the UE mobility pattern.

Observation2: Mobility pattern can be derived from Information of UE mobility (e.g., UE history information. UE trajectory). And if other information for deriving mobility pattern can be provided by UE should be discussed.

Observation3: In, S-SN and other candidate T-SN, UE configuration (UE context, C-RNTI, dedicated RACH config) needs to be reserved when UE access the target candidate cell, otherwise C-RNTI and RACH collision may happen with other UEs.

Proposal3: MN need to indicate candidate T-SN to maintain UE configuration (C-RNTI, dedicated RACH config)

- References

- RP-213565