3GPP TSG-RAN2 Meeting # 119 Electronic	R2-2207559

# 17th August - 29th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0263 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE behavior when IMS emergency call is not supported in cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per 5.2.8 of present spec, if the UE supports voice services and the current cell does not support IMS emergency calls as indicated by the field ims-EmergencySupport in SIB1, UE shall perform cell selection/reselection to an acceptable cell which supports emergency call in any supported RAT. Further, while camped on Any Cell state, cell reselection procedure can happen to a different cell or RAT if the reselection criteria is satisfied. However, when the target cell does not support IMS emergency calls, then the procedure would fail and UE may camp back on the previous cell from which cell reselection was triggered. After reverting back to the previous cell, when the reselection criteria is evaluated, reselection may again be triggered to the target cell. This will result in an undesired pingpong for cell reselection, and should be prevented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified UE behavior in 5.2.8 that if the cell is identified to not support IMS emergency calls when camped on Any Cell state, the UE shall deprioritize the pertinent frequency to be the lowest priority frequency. Impacted functionality: Cell selection and reselection in Any Cell state Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no interoperability issues. - If the UE is implemented according to the CR and the network is not, there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may not be able to stay camped to a cell which supports IMS emergency calls due to unnecessary reselection to a cell where IMS emergency calls support is not available as it satisfies reselection criteria |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

#### <Start of Change>

### 5.2.8	Camped on Any Cell state

This state is only applicable for RRC_IDLE state. In this state, the UE shall perform the following tasks:

-	monitor Short Messages transmitted with P-RNTI over DCI as specified in clause 6.5 in TS 38.331 [3];

-	monitor relevant System Information as specified in TS 38.331 [3];

-	perform necessary measurements for the cell reselection evaluation procedure;

-	execute the cell reselection evaluation process on the following occasions/triggers:

1)	UE internal triggers, so as to meet performance as specified in TS 38.133 [8];

2)	When information on the BCCH used for the cell reselection evaluation procedure has been modified.

-	regularly attempt to find a suitable cell trying all frequencies of all RATs that are supported by the UE. If a suitable cell is found, UE shall move to camped normally state.

-	if the UE supports voice services, the UE is not in SNPN access mode, and the current cell does not support IMS emergency calls as indicated by the field ims-EmergencySupport in SIB1 as specified in TS 38.331 [3], the UE shall perform cell selection/reselection to an acceptable cell that supports emergency calls in any supported RAT regardless of priorities provided in system information from current cell, if no suitable cell is found. The UE shall deprioritize the frequency pertaining to the current cell as the lowest priority frequency.

-	if the UE supports voice services, the UE is in SNPN access mode, and the current cell does not support IMS emergency calls for any SNPN(s) as indicated by the field imsEmergencySupportForSNPN in SIB1 as specified in TS 38.331 [3], the UE shall perform cell selection/reselection to an acceptable cell of any available SNPN that supports emergency calls, if no suitable cell is found. The UE shall deprioritize the frequency pertaining to the current cell as the lowest priority frequency.

#### <End of Change>