3GPP TSG-RAN WG2 Meeting #119-e	R2-2207591

# E-meeting, 17 – 29 August 2022

Agenda item:	6.1.2

Source:	Huawei, HiSilicon

Title:	Clarification on early configuration of MBS broadcast search space

Document for:	Discussion

- Introduction

In this contribution, we will discuss the possibility of early configuration of MBS broadcast search pace to UE before MII.

- Discussion

As captured in the TS 38.331 [1], in case network allows the UE to receive MBS broadcast in a RRC configured BWP, the network shall configure the searchSpaceMCCH and searchSapceMTCH for the UE in the PDCCH-ConfigCommon of the BWP.

The UE applies the MCCH information acquisition procedure to acquire the MBS broadcast configuration information broadcasted by the network. The procedure applies to MBS capable UEs interested to receive or that are receiving MBS broadcast services that are in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED with an active BWP with common search space configured by searchSpaceMCCH.

The broadcast MRB configuration procedure is used by the UE to configure PDCP, RLC, MAC and the physical layer upon starting and/or stopping to receive a broadcast MRB transmitted on MTCH, or upon modification of a configuration of a broadcast MRB received by the UE. The procedure applies to MBS capable UEs interested to or receiving an MBS broadcast service that are in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED with an active BWP with common search space configured by searchSpaceMTCH.

The configuration of searchSpaceMCCH and searchSapceMTCH can be provided to the UE after MII is received from the UE. However, this would require one additional RRC Reconfiguration procedure after MII is reported. Another possibility is that the network configures searchSpaceMCCH and searchSapceMTCH for a RRC configured BWP to the UE even before MII report (e.g. first RRC Reconfiguration to the UE) given that this configuration has no impact to the UE if the UE is not interested in MBS broadcast reception. This early configuration of MBS broadcast search space could avoid potential RRC Reconfiguration procedures to configure/de-configure MBS broadcast search space as a response to MII report.

Observation 1: Early configuration of MBS broadcast search space before MII could avoid potential RRC Reconfiguration procedures to configure/de-configure MBS broadcast search space as a response to MII report.

However, since there is no MBS broadcast capability signaling, the network may configure MBS broadcast search space to the UE not supporting MBS broadcast if early configuration of MBS broadcast search space is used. Generally, the PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters as defined in 38.331 [1] as below:

–	PDCCH-ConfigCommon

The IE PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters provided in SIB as well as in dedicated signalling.

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=              SEQUENCE {

controlResourceSetZero              ControlResourceSetZero                                  OPTIONAL,   -- Cond InitialBWP-Only

commonControlResourceSet            ControlResourceSet                                      OPTIONAL,   -- Need R

searchSpaceZero                     SearchSpaceZero                                         OPTIONAL,   -- Cond InitialBWP-Only

commonSearchSpaceList               SEQUENCE (SIZE(1..4)) OF SearchSpace                    OPTIONAL,   -- Need R

searchSpaceSIB1                     SearchSpaceId                                           OPTIONAL,   -- Need S

searchSpaceOtherSystemInformation   SearchSpaceId                                           OPTIONAL,   -- Need S

pagingSearchSpace                   SearchSpaceId                                           OPTIONAL,   -- Need S

ra-SearchSpace                      SearchSpaceId                                           OPTIONAL,   -- Need S

...,

[[

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                              SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT          SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT               SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}                                                                                           OPTIONAL    -- Cond OtherBWP

]],

[[

commonSearchSpaceListExt-r16                                             SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-r16     OPTIONAL  -- Need R

]],

[[

sdt-SearchSpace-r17                 CHOICE {

newSearchSpace                      SearchSpace,

existingSearchSpace                 SearchSpaceId

}                                                                                           OPTIONAL,   -- Need R

searchSpaceMCCH-r17                 SearchSpaceId                                           OPTIONAL,   -- Need R

searchSpaceMTCH-r17                 SearchSpaceId                                           OPTIONAL,   -- Need S

commonSearchSpaceListExt2-r17       SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1700           OPTIONAL,   -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710 CHOICE {

sCS480KHZoneEighthT                      SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                                           OPTIONAL,   -- Need R

pei-ConfigBWP-r17		SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT-r17                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

}                                                                                           OPTIONAL     -- Cond InitialBWP-Paging

]]

}

The related issue was also discussed during RAN2#116 meeting resulting in the following note having being captured in TS 38.331:

#### 5.3.5.8 Reconfiguration failure ##### 5.3.5.8.1 Void ##### 5.3.5.8.2 Inability to comply with RRCReconfiguration NOTE 00: The UE behaviour specified in this clause does not apply to the fields in ServingCellConfigCommon that are defined in release-16 and later. The UE ignores, i.e. does not take an action on and does not store, the fields that it does not support or does not comprehend. The UE shall: (…)

However, in the discussed case, the configuration is provided via ServingCellConfig -> downlinkBWP-ToAddModList -> BWP-Downlink -> BWP-DownlinkCommon -> PDCCH-ConfigCommon, not via ServingCellConfigCommon as in the note. However, since in both cases the configuration is carried in PDCCH-ConfigCommon which contains only cell-specific configurations and which the UE may receive via SIB1 anyway, the configuration of MBS broadcast search space to a UE not supporting MBS broadcast in PDCCH-ConfigCommon for a RRC configured BWP shall not cause inter-operability issue in our understanding. However, we would like to confirm this understanding and we discuss this topic on a general level in [2], submitted to RRC General AI, and we can wait for the conclusion of this general discussion. However, if this cannot be confirmed as a general understanding, we think we should discuss solving this issue as part of MBS discussion and we see two potential options:

- Agree that configuration of MBS broadcast search space to a UE not supporting MBS broadcast in PDCCH-ConfigCommon will not cause inter-operability issue (that would be an exception from a general rule).

- . IntroduceUE broadcast capability signaling report, i.e change MBS broadcast capability from optional capability without signaling to optional capability with signaling to allow the network only configures MBS broadcast search space to the UEs supporting MBS before MII.

Proposal 1: Confirm the understanding that configuration of MBS broadcast search space to a UE not supporting MBS broadcast in PDCCH-ConfigCommon will not cause inter-operability issue.

Proposal 2: If Proposal 1 is not agreed, RAN2 is asked to change MBS broadcast capability from “optional capability without signaling” to “optional capability with signaling”.

- Conclusion

Based on the above discussion, we recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: Confirm the understanding that configuration of MBS broadcast search space to a UE not supporting MBS broadcast in PDCCH-ConfigCommon will not cause inter-operability issue.

Proposal 2: If Proposal 1 is not agreed, RAN2 is asked to change MBS broadcast capability from “optional capability without signaling” to “optional capability with signaling”.

Reference

- 3GPP TS 38.331 RRC Specification

- R2-2207776, UE handling of cell-specific parameters provided in dedicated signalling, Huawei, HiSilicon