3GPP TSG RAN WG2 #119-e		      R2-2207607

Electronic, 17th August – 29th August,2022

Source:	vivo

Title:	Remaining issue on early measurement for EPS Fallback

Agenda Item:	6.21.2

Document for:	Discussion and Decision

- Introduction

In last meeting, we made much progress on early measurement for EPS Fallback and agreed with the CR[1]. In which, we reached the following agreements:

- Capture the note “When idleModMeasVoiceFallback is included in SIB5, UE may decide to measure and report idle/inactive measurements for EUTRA carrier frequencies included in SIB5 even if it does not support NE-DC between the serving carrier and the EUTRA carrier frequencies.” in section 5.7.8.2a of TS 38.331. - It can be discussed whether frequencies in SIB11 and reference to RAN4 requirement shall be included in above note next meeting. - One bit “idleModeMeasVoiceFallback” is broadcasted in SIB5 and indicates whether E-UTRA idle/inactive measurements and reporting for EPS fallback can be used. FFS whether describe it from the network point of view or UE point of view next meeting.

In this paper, we will discuss the remaining issues on early measurement for EPS Fallback, including whether the frequencies in SIB11 could be used and what reference to RAN4 requirement for early measurement for EPS Fallback, besides, whether the “idleModeMeasVoiceFallback” should be described from the network point of view or UE point of view is also involved.

- Background

The following is the understanding of the early measurement for EPS Fallback:

- The NW broadcasts an indication in SIB5 to indicate whether the camped cell could support idle/inactive measurements and report for EPS Fallback. - The UE could measure the target frequencies by implementation. No procedure text for whether or when UE measure the candidate EPS fallback target frequencies. - After the UE goes to connect mode - If the network wants to have measurement report. It can configure normal connected mode measurement on the target frequency. - If the UE has performed measurement on the same target frequency based on SIB configuration. The required time for UE to send the report may be reduced. - However, there is no UE requirement to speed up the process. - If the network wants to use blind redirection. It can just redirect the UE to the target frequency. - If the UE has performed measurement on the same target frequency based on SIB configuration. The required time for UE to camp on a cell on that frequency could be reduced. - However, there is no UE requirement to speed up the process. - it is up to UE implementation whether reuse EMR reporting framework for early EPS fallback measurement, one note is added in TS 38.331.

It can be observed that the early measurement could reduce the latency of EPS Fallback and improve the user experience with two aspects. On one hand, the early measurement for EPS fallback will help to speed up the connected measurement on the same frequency to reduce the measurement latency for EPS fallback. On the other hand, if there is no connected measurement, the early measurement for EPS fallback will help UE to select a suitable E-UTRAN carrier to reduce the failure rate of blind redirection.

- Discussion

  - SIB11 issue

In RAN2#118-e meeting, we have the following remaining issues:

- It can be discussed whether frequencies in SIB11 and reference to RAN4 requirement shall be included in above note next meeting.

According to the RRC specification, whether a cell supports UE performing idle/inactive measurement and whether the cell supports early measurement report framework is determined by the parameter in SIB1, i.e. the idleModeMeasuremetsEUTRAN:

idleModeMeasurementsEUTRA This field indicates that a UE that is configured for EUTRA idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform EUTRA idle/inactive measurements.

In our view, the NE-DC related idle/inactive measurements and early measurement for EPS Fallback are possible to coexist in the same cell. However, we don’t intend to configure dedicated frequencies for early measurement for EPS Fallback, while re-using the frequency in SIB11 for idle/inactive measurement for EPS fallback may affect NE-DC related idle/inactive measurements considering how often the UE measures each frequency largely scales based on the number of frequencies the UE is configured to measure according to current RAN4 requirement.  Samely, it may also lead to further degraded measurement accuracy for idle/inactive measurement for EPS fallback. Hence, we prefer not to use the carrier frequencies in SIB11 for idle/inactive measurement for EPS fallback.

Observation 1: Re-using the frequency in SIB11 for idle/inactive measurement for EPS fallback may lead to further degraded measurement accuracy.

Proposal 1: EUTRA carrier frequencies included in SIB11 won’t be used for idle/inactive measurement for EPS fallback.

  - Reference to RAN4 requirement

Up to now, we have not determined the requirements for early measurement for EPS Fallback, then the following cases are possible:

- Case 1: The requirements of early measurement for EPS Fallback are based on the requirement for cell reselection in sections 4.2.2 and 5.1.2 of TS 38.133[2].

- Case 2: The requirements of early measurement for EPS Fallback are based on the idle Mode CA/DC measurements requirements in sections 4.4.2 and 5.4.2 of TS 38.133[2].

The requirements of the above cases are very similar, except that case1 will only measure higher priority inter-RAT E-UTRAN frequencies when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ while case 2 will measure all configured E-UTRAN frequencies when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ. In our understanding, the idle/inactive measurement for EPS Fallback will not distinguish whether the E-UTRAN frequency has higher priority since UE could fall back to all available E-UTRAN frequencies if needed, hence the idle Mode CA/DC measurements requirements are more suitable for early measurement for EPS Fallback.

However, the idle Mode CA/DC measurements requirements in [2] are only applied to the time within which T331 is running, which is different from early measurement for EPS Fallback, hence we think an LS may be needed to RAN4 to inform RAN4 of our progress on idle/inactive measurement in RAN2 and ask them to determine whether there is specification impact in RAN4.

Proposal 2: Reusing Idle Mode CA/DC Measurements requirements for early measurement for EPS Fallback.

Proposal 3: Send an LS to RAN4 to inform them of the progress on early measurement for EPS Fallback in RAN2 and ask them to determine whether there are specification impacts in RAN4.

  - Indication description of idleModMeasVoiceFallback

Another remaining issue for early measurement for EPS Fallback is as follows:

One bit “idleModeMeasVoiceFallback” is broadcasted in SIB5 and indicates whether E-UTRA idle/inactive measurements and reporting for EPS fallback can be used. FFS whether describe it from the network point of view or UE point of view next meeting.

For the bit in SIB5, some companies want to describe it from the network point of view while other companies want to describe it from UE point of view. In our understanding, the indication is used to indicate whether the cell has the capability of supporting early measurement for EPS Fallback, it is clearer to describe it from the network point. Besides, there are also a lot of parameters described from the network point of view in current specification as follows.

eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services as defined in TS 23.501 [32]. If absent, eCall over IMS is not supported by the network in the cell.

According to the above, we think the idleModMeasVoiceFallback in SIB5 should be described from the network point of view:

idleModeMeasVoiceFallback indicates whether the cell supports to use E-UTRA idle/inactive measurements and reporting for EPS fallback.

Proposal 4: Describe idleModeMeasVoiceFallback indication from the network point of view.

Proposal 5: Capture the idleModeMeasVoiceFallback description “indicates whether the cell supports E-UTRA idle/inactive measurements and reporting for EPS fallback.” in RRC specification.

  - UE capability on early measurement for EPS Fallback

We have not determined the issue on whether there is a need to introduce UE capability for early measurement for EPS fallback. In our view, it is not needed to force all UEs to support this capability, e.g. if a UE doesn’t have the voice capability, then there is no need to support early measurement for EPS Fallback. Hence, the UE capability for early measurement should be introduced in the chapter of Optional features without UE radio access capability parameters of TS 38.306[3].

Proposal 6: Introduce a UE capability for idle/inactive measurement for EPS Fallback in the chapter of Optional features without UE radio access capability parameters of TS 38306.

- Conclusion

In this paper, we discuss the reamining issue of early measurement for EPS/RAT Fallback. Based on the discussion, we have the following observations and proposals:

Observation 1: NE-DC related idle/inactive measurements and early measurement for EPS Fallback are possible to coexist in same cell.

Proposal 1: When idleModMeasVoiceFallback is included in SIB5, UE may decide to measure and report idle/inactive measurements for EUTRA carrier frequencies included in SIB5 and SIB11.

Proposal 2: Reusing Idle Mode CA/DC Measurements requirements for early measurement for EPS Fallback.

Proposal 3: Send an LS to RAN4 to inform them of the progress on early measurement for EPS Fallback in RAN2 and ask them to determine whether there are specification impacts in RAN4.

Proposal 4: Describe idleModeMeasVoiceFallback indication from the network point of view.

Proposal 5: Capture the idleModeMeasVoiceFallback description “indicates whether the cell supports E-UTRA idle/inactive measurements and reporting for EPS fallback.” in RRC specification.

Proposal 6: Introduce a UE capability for idle/inactive measurement for EPS Fallback in the chapter of Optional features without UE radio access capability parameters of TS 38306.

- Reference

- R2-2206825_CR_Early Measurements for EPS fallback

- 38133-h70, NR; Requirements for support of radio resource management

- 38306-h10, User Equipment (UE) radio access capabilities