3GPP TSG-RAN WG2 Meeting #119-e	R2-2207651

# Electronic Meeting, Aug, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3301 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for notification message with re-establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-8-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current specification (section 5.8.9.10.4), upon reception of NotificationMessageSidelink including indicationType by L2 U2N Remote UE in RRC_CONNECTED, the remote UE shall initiate re-establishment. There are four indication types including relayUE-Uu-RLF, relayUE-HO, relayUE-CellReselection, relayUE-Uu-RRC-Failure. Two indication types including relayUE-Uu-RLF and relayUE-HO are associated with RRC_CONNECTED. However, the remote UE shall perform the actions upon going to RRC_IDLE upon receiption of NotificationMessageSidelink indicating relayUE-HO or relayUE-CellReselection based on section 5.3.7.7 for the case that UE is performing re-establishment procedure. Therefore, there is collission between section 5.8.9.10.4 and section 5.3.7.7. Specifically, when UE receives NotificationMessageSidelink indicating relayUE-HO, UE initiates re-establishment based on 5.8.9.10.4 and enters RRC_IDLE based on 5.3.7.7. It is natural that the case that UE is performing re-establishment procedure should be excluded in 5.8.9.10.4. We proposale that upon reception of NotificationMessageSidelink including indicationType by L2 U2N Remote UE in RRC_CONNECTED and Timer T301 is not running, the remote UE shall initiate re-establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 5.8.9.10.4, upon reception of NotificationMessageSidelink including indicationType by L2 U2N Remote UE in RRC_CONNECTED and Timer T301 is not running, the remote UE shall initiate re-establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ‘Timer T301 is not running’ is added as a condition for RRC_CONNECTED UE to initiate re-establishment upon reception of NotificationMessageSidelink. Impact Analysis Impacted 5G architecture options: NG-RAN Architecture supporting the PC5 interface Impacted functionality: The reception of Notification message for remote UE Inter-operability: There is no inter-operatbility issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.8.9.10.4	Actions related to reception of NotificationMessageSidelink message

Upon receiving the NotificationMessageSidelink, the U2N Remote UE shall:

1>	if the indicationType is included:

2>	if the UE is L2 U2N Remote UE in RRC_CONNECTED while timer T301 is not running:

3>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

2>	else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE):

3>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

4>	perform the PC5-RRC connection release as specified in 5.8.9.5.

3>	else:

4>	maintain the PC5-RRC connection;

4>	if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection:

5>	consider cell re-selection occurs;

NOTE:	For L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, it is up to Remote UE implementation whether to release or keep the PC5 unicast link.

End of changes