3GPP TSG-RAN WG2 Meeting #119-e	R2-2207685

Online, 17th – 29th August, 2022

Agenda Item:	8.2.3

Source:	Spreadtrum Communications

Title:	Discussion on solutions for integrity of RAT-dependent positioning techniques

Document for:	Discussion and Decision

Introduction

According to the WID [1] of RAN #94, Rel-18 NR positioning will take RAT-dependent positioning integrity into account. And the study scope of integrity are given as follows.

  - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]:

    - Identify the error sources, [RAN1, RAN2].

    - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2]

    - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

In this contribution, we discuss the question of the methodologies and procedures of RAT-dependent positioning and express our opinions.

RAT-dependent positioning integrity

Methodologies of integrity

In Rel-17, RAN2 has discussed and developed the positioning integrity that allow UE to determine and report the integrity results of the calculated location for GNSS positioning methods. And in Rel-17, RAN2 has discussed the integrity concepts and positioning integrity methods.

Positioning integrity is defined as a measure of the trust in the accuracy of the position-related data and the ability to provide associated alerts.

Each time a location is provided, positioning integrity can be used to quantify the trust on the provided location.

To ensure the integrity of positioning, Rel-17 defines various concepts in section 8.1.1a in TS 38.305, e.g., Positioning integrity, error, Bound, TTA, DNU, Residual Risk, irMinimum, irMaximum, Correlation Times, Target Integrity Risk. It can be reused to the RAT-dependent integrity positioning.

And the principle of operation in TS 38.305 can also be applied to RAT dependent integrity.

And RAN1 #109 meeting has approved the following agreement. And for the purpose of discussion of error sources, we can reuse the definitions for RAT-dependent integrity and update the references to GNSS in Section 8.1.1a in TS38.305 to also include RAT-dependent methods.

Agreement For the purpose of discussion of error sources, reuse the definitions for RAT-dependent integrity and update the references to GNSS in Section 8.1.1a in TS38.305 to also include RAT-dependent methods. - Note: The intention of the proposal is not to make text proposals for TS 38.305 - FFS: whether to modify and/or how to modify, for the purpose of discussion in RAN1, terms in 8.1.1a in TS 38.305 (e.g., definitions for “Error”, “Bound”, “Time-to-Alert (TTA)”, “DNU”, “Residual Risk”, “irMinimum, irMaximum”) for RAT dependent positioning methods

Proposal 1: RAN2 should confirm that the definitions for GNSS integrity can be reused to RAT-dependent integrity.

And Rel-17 positioning integrity supports UE-based and UE-assisted integrity mode. For UE-based positioning, the UE determines and reports the integrity results of the calculated location based on the measurements of GNSS signals and the assistance data received from the network. For UE-assisted positioning, the UE performs measurements of the received GNSS signals and sends to LMF the measurement reports for calculating the UE location and integrity. For RAT-dependent positioning, UE-based and UE-assisted integrity can be considered.

Proposal 2: RAN2 should consider UE-based and UE-assisted positioning integrity methods.

And considering the signalling to deliver the KPIs and integrity results reporting, RAN2 agree that Rel-17 only supports mode1 reporting.

For mode 1, the integrity computing entity calculates the PL. Then, the calculated PL is directly reported to where the LCS client resides (Network or UE). For mode 2, the integrity computing entity calculates the PL. Then, the integrity computing entity further compares the calculated PL with the given AL to determine if the positioning system is still available to offer trustable position estimation. Thus, the integrity computing entity may only have to report a binary flag (0 and 1) to indicate whether the positioning system is available or not.

And for mode2, RAN2 decides that it will be revisited in future releases in rel-17. But mode1 and mode2 can be useful in different scenarios. Mode1 can be useful when the LCS client does not want expose the integrity KPIs to LMF/UE. And for mode2, it can reduce the complexity for LCS client. And RAN2 has discussed many issues of reporting mode2, such as whether should include an indicator to tell the UE either reporting mode1 or reporting mode2? Whether allows the UE to raise the flag indicating whether there is an integrity risk? And considering mode2 can be useful in simple APP implementation where the application requires only information on whether integrity performance is met with respect to the KPIs. Thus, in Rel-18, in order to support the positioning integrity in various scenarios, mode1 and mode2 integrity result reporting scan be considered for RAT-dependent positioning.

Proposal 3: Mode1 and Mode2 of integrity results reporting can be considered for RAT-dependent positioning integrity.

The table1 shows the specific spec impacts of positioning integrity for RAT-independent positioning technologies in Rel-17. And considering the specific signalling procedure, the signaling of the RAT-dependent positioning integrity should reuse the signalling of the GNSS positioning method. Specifically, in Rel-17, RAN2 confirms that LPP messages RequestLocationInformation and ProvideLocationInformation are used to transfer integrity KPIs/results. RAN2 also confirms that LPP messages RequestAssistanceData and ProvideAssistanceData are used to transfer integrity assistance data for GNSS positioning at least for UE-based mode.

Proposal 4: For RAT-dependent positioning integrity, LPP messages RequestLocationInformation and ProvideLocationInformation are reused to transfer integrity KPIs/results.

Proposal 5: For RAT-dependent positioning integrity, LPP messages RequestAssistanceData and ProvideAssistanceData are reused to transfer integrity assistance data.

Table 1: Summary of spec impacts of positioning integrity

| Signalling procedure | Specific IE | ASN.1 |
| --- | --- | --- |
| Provide GNSS capabilities | A-GNSS-ProvideCapabilities | GNSS-CommonAssistanceDataSupport ::= SEQUENCE { …… [[ gnss-Integrity-ServiceParametersSupport-r17 GNSS-Integrity-ServiceParametersSupport-r17 OPTIONAL, -- Cond IntServiceSup gnss-Integrity-ServiceAlertSupport-r17 GNSS-Integrity-ServiceAlertSupport-r17 OPTIONAL -- Cond IntAlertSup ]] } |
| Provide Assistance Data (LMF->UE) | A-GNSS-ProvideAssistanceData | GNSS-CommonAssistData ::= SEQUENCE { … gnss-Integrity-ServiceParameters-r17 GNSS-Integrity-ServiceParameters-r17 OPTIONAL, -- Need ON gnss-Integrity-ServiceAlert-r17 GNSS-Integrity-ServiceAlert-r17 OPTIONAL -- Need OR ]] } |
| Request Location Information (LMF->UE) | CommonIEsRequestLocationInformation | integrityInformationRequest-r17 TargetIntegrityRisk-r17 |
| Provide Location Information (UE->LMF) | CommonIEsProvideLocationInformation | integrityInfo-r17 |

Error sources analysis

In Rel-18 first RAN1 meeting, the following agreements has been approved. The research of error sources will be divided into timing-based positioning and angle-based positioning methods. And then, each error sources will be mapped to specific positioning method, e.g., DL, UL, DL&UL positioning method.

Agreement - Study sources of error for timing-based positioning and angle-based positioning methods, focusing on the following aspects - Origin of the error source - e.g., At UE and/or network side - e.g., From assistance information, and/or measurements - Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - Criteria to become an error source (e.g., whether it is quantifiable, how much influence an error source has on determination on integrity) Agreement - At least the following error sources for angle -based positioning methods are studied - TRP/UE measurements errors (e.g., AoA, RSRP, RSRPP) - FFS: Effect of multipath/NLoS channels on TRP/UE measurement errors - Error in assistance data (e.g TRP location, TRP beam antenna information) - FFS: Further study identification of error sources resulting from the multipath/NLoS channel/radio propagation environment, including multipath/NLoS channel itself as an error source - Other error sources are not precluded FFS: details of each error source, e.g., mean/standard deviation/range associated with each error Agreement In addition to the agreed aspects for the study, study the following aspects for error sources for timing/angle based positioning methods - Mapping between an error source and a positioning method (e.g., DL, UL, DL&UL positioning method) - e.g., error in TRP location can be an error source for UE-based DL-AoD - Other aspects are not precluded

For timing-based positioning methods, we give the error sources list which may be considered in integrity calculation:

- TRP/UE measurements error sources:

  - TRP measurement errors (RTOA/ UL-RSRP/UL-RSRPP/gNB Rx-Tx time difference measurements)

  - UE measurement errors (RSTD/ DL-RSRP/DL-RSRPP/UE Rx-Tx time measurements)

- Error in assistance data:

  - Synchronization errors (between TRPs)

  - TRP location

  - TRP geometry (GDOP) error

  - TRP antenna calibration error

- TRP/UE Timing error

  - Rx/Tx/RxTx TEG

  - Difference of Rx/Tx/RxTx TEG of the same UE

  - TRP Rx/Tx/RxTx TEG

  - Differences of Rx/Tx/RxTx TEG of the same TRP

- UE/TRP clock drifting

And for angle-based positioning methods, the following error sources may need to be considered in calculation integrity:

- TRP/UE measurements error sources:

  - TRP measurement errors (UL-AOA/UL-RSRP/UL-RSRPP)

  - UE measurement errors (DL-RSRP/DL-RSRPP)

- Error in assistance data:

  - TRP location

  - TRP geometry (GDOP) error

  - TRP antenna reference point (ARP) errors (or antenna phase center errors)

  - TRP beam antenna information

For RAT-dependent integrity, the error sources can be divided into normal error and abnormal error. The normal error may be able to model as Gaussian distribution with unknown mean and deviation. For example, measurement-related error may be able to model as Gaussian distribution. And the values of mean and deviation may be determined from assistance data. Specifically, for some error caused by TRP, the mean and deviation of error sources may be determined by gNB and be transmitted to LMF via NRPPa signalling. For UE-based integrity, the man and deviation of each error should be provided to UE by LMF via assistance data.

Proposal 6：For UE-based positioning integrity, the mean and deviation of each error should be provided to UE by LMF via assistance data.

And for UE-assisted positioning integrity, in order to calculate positioning integrity, the mean and deviation of the UE error sources, TRP error sources and measurement error sources need to be determined and transmitted to LMF.

Proposal 7: For UE-assisted positioning integrity, the mean and deviation of each error from UE/TRP error sources and measurements error sources need to be delivered to LMF.

Conclusions

In this contribution, we discuss the positioning enhancements of Rel-18 positioning integrity techniques. And the proposals are given as follows:

Proposal 1: RAN2 should confirm that the definitions for GNSS integrity can be reused to RAT-dependent integrity.

Proposal 2: RAN2 should consider UE-based and UE-assisted positioning integrity methods.

Proposal 3: Mode1 and Mode2 of integrity results reporting can be considered for RAT-dependent positioning integrity.

Proposal 4: For RAT-dependent positioning integrity, LPP messages RequestLocationInformation and ProvideLocationInformation are reused to transfer integrity KPIs/results.

Proposal 5: For RAT-dependent positioning integrity, LPP messages RequestAssistanceData and ProvideAssistanceData are reused to transfer integrity assistance data.

Proposal 6：For UE-based positioning integrity, the mean and deviation of each error should be provided to UE by LMF via assistance data.

Proposal 7: For UE-assisted positioning integrity, the mean and deviation of each error from UE/TRP error sources and measurements error sources need to be delivered to LMF.

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, 3GPP TSG-RAN Plenary Meeting #94e.