3GPP TSG-RAN#119e	R2-2207695

Online, August 2022

Agenda item:		8.4.4

Source:		Lenovo

Title:			Failure case for CHO with SCG

Document for:		Discussion and Decision

- Introduction

In RAN#96-e meeting, the WI of Further NR mobility enhancements was revised [1], with the following objective with regards to CHO with SCG:

To specify CHO including target MCG and target SCG [RAN3, RAN2]. Note 5: This is already being targeted for Rel-17, so this objective will be reviewed at RAN#97-e.

This document aims to discuss the potential failure case in CHO with SCG.

- Discussion

In Rel-17, we have agreed to support the case of CHO with SCG. In Rel-17, UE can receive CHO including target MCG and target SCG. Once CHO condition for PCell is met, UE performs CHO towards both MCG and SCG. Some CRs have been approved in RAN#96 meeting. However, the failure case in CHO with SCG is not discussed in Rel-17.

Case#1: CHO with target SCG which is same as source SCG is configured. CHO condition for PCell is met while UE is experiencing RLF on SCG. UE still performs random access to the failed SCG.

The case of CHO including same SCG as source SCG is supported based on the discussion and current specification. After UE receives CHO including target SCG which is same as source SCG, UE initiates SCG failure information and suspends SCG link e.g when detecting RLF on SCG. It is possible that CHO condition is met while UE still suspends SCG link. In legacy, UE performs random access towards SCG regardless of whether RLF on SCG happens or not. After UE moves to target MCG, UE declares RLF on SCG and transmits SCG failure information message to target MCG. It seems unnecessary to perform random access towards failed SCG.

Case#2: SN can perform an Intra-SN PSCell change or update source SCG configuration without MN involvement. The MN is not aware of PSCell change and the updated SCG configuration.

In legacy, SN can initiate an Intra-SN PSCell change or update source SCG configuration without MN involvement. That means SN will not inform MN with PSCell change and source SCG reconfiguration. the MN is not aware of PSCell change and the updated SCG configuration. If MN configures CHO with SCG to UE before PSCell change and source SCG reconfiguration, it may result in failure in SCG. The potential enhancement is that SN informs MN. Alternatively, UE does not apply the out-of-date SCG configuration since UE is aware that PSCell change/source SCG reconfiguration after CHO with SCG is configured.

Proposal: RAN2 is suggested to discuss the following failure cases:

- Case#1: CHO with target SCG which is same as source SCG is configured. CHO condition for PCell is met while UE is experiencing RLF on SCG. UE still performs random access to the failed SCG.

- Case#2: SN can perform an Intra-SN PSCell change or update source SCG configuration without MN involvement. The MN is not aware of PSCell change and the updated SCG configuration.

- Conclusion

In this contribution, the conclusion and proposal is made based on the discussion:

Proposal: RAN2 is suggested to discuss the following failure cases:

- Case#1: CHO with target SCG which is same as source SCG is configured. CHO condition for PCell is met while UE is experiencing RLF on SCG. UE still performs random access to the failed SCG.

- Case#2: SN can perform an Intra-SN PSCell change or update source SCG configuration without MN involvement. The MN is not aware of PSCell change and the updated SCG configuration.

References

RP-221799 was RP-221558 R18 MobEnh WID