3GPP TSG-RAN WG2 Meeting #119-e	R2-2207707

Online, 17 – 29 August 2022

Agenda item:	 8.13.3

Source:		Lenovo

Title:		MRO for handover failure or SCG failure in NR-U

Document for:		Discussion and Decision

1	Introduction

In R17, DL MLB for NR-U is specified, but MRO for NR-U is postponed to R18 due to time limitation. As agreed in [1], MRO for NR-U would be supported in R18:

- Support of SON/MDT enhancements for [RAN3, RAN2]:

•	MR-DC CPAC

•	Successful PScell change report

•	Successful Handover Report (e.g. inter-RAT)

•	NPN

•	RACH report

•	fast MCG recovery

•	NR-U (MRO and UL MLB)

In this paper, we would further discuss the details of MRO for handover failure or SCG failure in NR-U.

2	Discussion

NR-U is a new functionality introduced in R16 to enable the UE to operate in a unlicensed spectrum. In NR-U, not only user plane data but also L1/L2/RRC signalling can be transmitted in the unlicensed spectrum. On the other hand, both the SpCell and the SCells can communicate with the UE over the unlicensed spectrum.

In NR-U, before operating in the unlicensed spectrum, both the UE and the gNB would perform LBT to guarantee that the radio resource is not occupied by others e.g. Wifi, data/signalling transmission can be allowed if the wireless channnel is available. For UL, consistent LBT failure is detected per UL BWP by counting LBT failure indications from the lower layers to the MAC entity, if consistent LBT failure has been triggered in all UL BWPs configured with PRACH occasions on same carrier in the serving cell, the MAC entity indicates consistent LBT failure to upper layers and then RLF occurs.

In legacy, too late handover happens due to source RLF before HO command is received by the UE, which means network needs to trigger handover earlier. Also, if the mobility configuration is not proper, too early handover or handover to wrong cell would happen. To avoid too late or too early or handover to wrong cell, legacy MRO is used to optimize mobility configuration e.g. modify handover trigger threshold, or TTT for RRM measurement.

Similariy, in R17, too late PSCell change or too early PSCell change or triggering PSCell change to wrong PSCell is introduced, and MRO for PSCell change failure is applied to optimize PSCell change related parameters.

Considering mobility in NR-U, if channel is occupied in the nework, there is no shared spectrum to send SpCell change command to the UE, thus the UE may detect RLF in the source SpCell. On the other hand, after the UE receives the SpCell change command command, the UE would perform LBT, it is possible that the shared spectrum of the target SpCell is occupied, so consistent LBT failure may happen before or during RACH procedure with the target SpCell, in such case, SpCell change may fail when T304 expires.

Observation 1: During handover or PSCell change procedure in the unlicensed spectrum, RLF or HOF or PSCell change failure would happen due to LBT failure.

Currently in NR-U system, when RLF report is triggered due to consistent LBT failure in the unlicensed spectrum, the RLF cause can be set to “lbtFailure” while the connection failure type is set to “RLF”. When SCG Failure Information message is sent caused by consistent LBT failure, the failureType is set to “scg-lbtFailure”.

Observation 2: When failure is triggered due to consistent LBT failure in the unlicensed spectrum, the RLF cause can be set to “lbtFailure” while the connection failure type is “RLF”, or the failureType included in SCG Failure Information message is set to “scg-lbtFailure”.

When HOF happens, e.g. the UE fails to access to the target cell due to consistent LBT failure, it is beneficial for the network to know that handover fails due to consistent LBT failure in UE side, thus the network can modify LBT related configuration e.g. RSSI measurement configuration. The existing lbtFailure for rlf-cause IE in RLF Report is only used to indicate RLF case. Therefore, an indication is needed to indicate the cause for HOF e.g. consistent LBT failure in the RLF report, so that the source node can make failure cause analysis, and make corresponding optimization e.g. modify mobility configuration or LBT related configuration or both.

Simailarily, when PSCell change failure is caused due to consistent LBT failure, an indication is included in the SCG Failure Information message to indicate the cause for SCG failure e.g. consistent LBT failure.

Proposal 1: In NR-U, an indication to indicate HOF or SCG failure is due to consistent LBT failure can be included in the RLF report or SCG Failure Information message.

RSSI and channel occupancy measurements are introduced for NR-U to reflect the characteristics of the unlicensed spectrum, when failure occurred due to LBT failure, LBT related configuration, e.g. RSSI measurement configuration or configuration related with LBT failure detection and recovery (e.g. lbt-FailureRecoveryConfig) needs to be optimized. It is beneficial for the UE to include configuration related with LBT failure detection and recovery (e.g. lbt-FailureRecoveryConfig) in the RLF report or SCG Failure Information message.

Proposal 2: In NR-U, configuration related with LBT failure detection and recovery (e.g. lbt-FailureRecoveryConfig) can be included in the RLF report or SCG Failure Information message.

Since RSSI and channel occupancy measurements are used to indicate the channle characteristics, in order to enable the network know channel occupation and optimize unlicensed radio resources, the UE can store and report the measured RSSI and channel occupancy on the unlicensed spectrum of target SpCell in the RLF report or SCG Failure Information message.

Proposal 3: The measured RSSI and channel occupancy in the unlicensed spectrum of target SpCell can be included in the RLF report or SCG Failure Information message.

For mobility in NR-U, after the UE receives the SpCell change command, it is possible that LBT with target SpCell is successful before or during RACH procedure towards target SpCell but SpCell change fails when T304 expires. It is needed to distinguish between radio link quality issue and LBT issue for the purpose of MRO since LBT failure is different from coverage hole. Time duration for LBT during handover or PSCell change procedure is useful to decide how RLF report or SCG Failure Information message is used for MRO analysis, for example, if long time duration is spent for LBT, it may mean that the failure is mainly caused by channel occupancy rather than radio link quality, network may not perform a coverage optimization after receiving the failure information. Therefore, it is beneficial to include time duration for LBT in the RLF report or SCG Failure Information message.

Proposal 4: The time duration for LBT during SpCell change procedure can be included in the RLF report or SCG Failure Information message.

3	Conclusion

In this paper, MRO for handover failure or SCG failure in NR-U is discussed. We have the following observations and proposals:

Observation 1: During handover or PSCell change procedure in the unlicensed spectrum, RLF or HOF or PSCell change failure would happen due to LBT failure.

Observation 2: When failure is triggered due to consistent LBT failure in the unlicensed spectrum, the RLF cause can be set to “lbtFailure” while the connection failure type is “RLF”, or the failureType included in SCG Failure Information message is set to “scg-lbtFailure”.

Proposal 1: In NR-U, an indication to indicate HOF or SCG failure is due to consistent LBT failure can be included in the RLF report or SCG Failure Information message.

Proposal 2: In NR-U, configuration related with LBT failure detection and recovery (e.g. lbt-FailureRecoveryConfig) can be included in the RLF report or SCG Failure Information message.

Proposal 3: The measured RSSI and channel occupancy in the unlicensed spectrum of target SpCell can be included in the RLF report or SCG Failure Information message.

Proposal 4: The time duration for LBT during SpCell change procedure can be included in the RLF report or SCG Failure Information message.

Reference

[1] RP-221825, Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity), CMCC