3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2207713

Online, August 17-29, 2022

Agenda item:		8.7.2

Source:		Lenovo

Title:		Potential issues for Msg3 repetition in NTN

Document for:		Discussion

- Introduction

In RP-221819, the objective regarding coverage enhancement for Rel-18 NR NTN is approved as:

Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope): - Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4] - VoIP and low-data rate services for commercial handset terminals The following items are shown as examples of areas to consider in the next step of the study. The actual items for study will be based on the evaluation of coverage issues specific to NTN identified above. - NTN-specific repetitions enhancements beyond techniques covered in Rel-17 CovEnh WI for the relevant channels - NTN-specific techniques for improved diversity and/or reduced polarization loss - Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR - NOTE: Intent is not to introduce a new codec.

In this contribution we provide analysis for potential issues of applying Msg3 repetition in NR NTN.

- Discussion

Rel-17 enhancements to Msg3 repetition

Coverage is one of the key factors that network operators consider when commercializing cellular communication networks due to its direct impact on service quality and expenditure. 3GPP initiated “Study on NR coverage enhancements” in Rel-17 to identify issues and potential enhancements for radio channels and signals, and enhancements for Msg3 PUSCH were specified as a result.

Msg3 PUSCH has been identified as one of the bottleneck UL channels and Msg3 repetition is considered to solve the coverage problem. Msg3 repetition allows transmission of several successive Msg3 when UE is performing random access and thus the UL coverage can be extended. In Rel-17, RAN2 meetings also had discussion of Rel-17 NR coverage enhancement focusing on signaling and protocol support of Msg3 repetition. That is, when UE’s RSRP is below a configured threshold, it can transmit its Msg1 over a dedicated set of PRACH resource to indicate its request for Msg3 repetition.

Potential issues of Msg3 repetition in Rel-18 NR NTN

In TN with low altitude antennas, there is a clear RSRP difference between UEs at cell center and cell edge (i.e. cell edge effect). However in an NTN cell the antenna is on board satellites with much higher altitude, and different weather (e.g. rain, fog or cloud) may cause variable degrees of signal fading. As a result the RSRP difference between UEs at cell center and cell edge is typically insignificant. If RSRP threshold is used as the only criterion in determining to request for Msg3 repetition, it may be hard for the NTN to configure it appropriately. A smaller RSRP threshold may prevent most UEs from benefiting coverage enhancement while a larger RSRP threshold may allow excessive Msg3 repetition causing heavier contention on the dedicated PRACH resource for requesting Msg3 repetition.

Except for the smaller RSRP difference, another NTN characteristic of satellite movement needs to be considered as well. Msg3 repetition is requested by Msg1 transmission and is indicated by Msg2 reception from network, i.e. there is at least one RTT between requesting (or not requesting) and transmitting (or not transmitting) Msg3 repetition, and during that time period the necessity or availability of Msg3 repetition may vary due to satellite movement. Figure 1 shows an example wherein at T1 time UE1 and UE3 needs Msg3 repetition and requests with Msg1 using dedicated resource while UE2 does not, and the satellite is moving towards UE3 direction. At time T2 after T1 due to the satellite movement and consequent RSRP change, UE2 may fulfil RSRP threshold and need Msg3 repetition but cannot apply due to not requesting, while UE3 may not need Msg3 repetition but has to apply due to previous requesting. At T2 if UE1 is at edge of a moving cell wherein the cell moves with satellite movement, it may be totally out of the cell coverage. In this case UE1 is better to be transited to IDLE mode and try to camp on another cell, instead of trying Msg3 repetition which is possibly to fail.

Figure 1. Possible impact of satellite movement on Msg3 repetition

Observation 1: Due to the smaller RSRP difference in a cell and satellite movement in NTN, it may be difficult to configure an appropriate RSRP threshold for UE to request Msg3 repetition.

The ContentionResolutionTimer (CRT) is used to determine a time window for receiving contention resolution configuration from network. In legacy specifications for TN, ContentionResolutionTimer is started immediately upon first Msg3 transmission and restarted at HARQ retransmission, and it is definitely running when Msg3 repetition occurs (Figure 2-A). In NTN due to the large propagation delay, a time offset equalling to UE-gNB RTT (Figure 2-B) is applied to the start of ContentionResolutionTimer to avoid unnecessary monitoring (no contention resolution configuration will arrive within the RTT). As a result Msg3 repetition may occur when ContentionResolutionTimer is either running or not. If a Msg3 repetition occurs within the RTT offset (i.e. ContentionResolutionTimer is not running), the behavior of ContentionResolutionTimer needs to be specified.

Figure 2. ContentionResolutionTimer (CRT) behaviors for Msg3 repetition

Observation 2: When RTT offset is applied to the start of ContentionResolutionTimer, the behavior of ContentionResolutionTimer due to Msg3 repetition needs to be specified.

Therefore from RAN2 perspective some enhancements could be needed for applying Msg3 repetition in NTN.

Proposal 1: RAN2 to study enhancements to Msg3 repetition for Rel-18 NR NTN.

- Conclusion

In this contribution, we analyse potential issues of applying Msg3 repetition in NR NTN. The following observation are given:

Observation 1: Due to the smaller RSRP difference in a cell and satellite movement in NTN, it may be difficult to configure an appropriate RSRP threshold for UE to request Msg3 repetition.

Observation 2: When RTT offset is applied to the start of ContentionResolutionTimer, the behavior of ContentionResolutionTimer due to Msg3 repetition needs to be specified.

And we propose:

Proposal 1: RAN2 to study enhancements to Msg3 repetition for Rel-18 NR NTN.