3GPP TSG-RAN WG2#119-e	R2-2207719

Online, 17-29 August 2022

Agenda Item:	8.5.4

Source: 	MediaTek Inc.

Title:  	XR-specific capacity improvements

Document for:	Discussion, decision

- Introduction

In [1], the third objective is to study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…), which focus on the following mechanisms:

- SPS and CG enhancements;

- Dynamic scheduling/grant enhancements.

This paper introduces our views on XR-specific capacity improvements.

# 2	Discussion

2.1	Traffic profile

In Rel-17 Study Item “Study on XR Evaluations for NR”, many of the XR and CG use cases are studied and characterised by quasi-periodic traffic (with possible jitter) with high data rate in DL (i.e., video stream) combined with the frequent UL (i.e., pose/control update) and/or UL video stream. Both DL and UL traffic are also characterized by relatively strict packet delay budget (PDB), the results are captured in TR 38.838 [2] and summarized in Table 1.

Table 1: XR Traffic model [2]

| Traffic Direction | DL |  |  |  | UL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Application | Cloud Gaming |  | AR/VR |  | AR | Pose/Control |
| Data rate (R) (Mbps) | 8 | 30 | 30 | 45 | 10 | 0.2 |
| Packet size distribution | Truncated Gaussian |  |  |  |  | Fixed |  |
| Packet size (Min, Ave, Max) (Bytes) | (8334, 16667, 25001) | (31250, 62500, 93750) |  | (46875, 93750, 140625) | (10417, 20834, 31251) | 100 |
| FPS (1/Periodicty) | 60 |  |  |  |  | 250 |  |
| Jitter value distribution | Truncated Gaussian |  |  |  |  | Fixed |  |
| Jitter value (Min, Ave, Max) (ms) | (-4, 0, 4) |  |  |  |  | 0 |  |
| Packet delay budget (PDB) (ms) | 15 |  | 10 |  | 30 | 10 |
| Min. required data rate (Mbps) | 8.89 | 33.33 | 50 | 75 | 5.56 | 0.08 |
| Reliability requirement (%) | 99 |  |  |  |  |  |  |

Observation 1: Both XR DL and UL traffic have high data rate and strict latency requirement

2.2	Scheduling mechanisms

There are two types of scheduling for downlink. One is called 'Dynamic Scheduling' and the other one is SPS (Semi Persistent Scheduling). Dynamic scheduling is the mechanism in which each and every PDSCH is scheduled by DCI. SPS is the mechanism in which the PDSCH transmission is scheduled by RRC message, not always by PDCCH, which can save some control overhead and is more suitable for periodic traffic. However, compared to the XR packet size and jitter problem, the gain to save control overhead when using SPS is negligible.

For NR Dynamic DL Scheduling as shown in Figure 1. For example, PDCCH is transmittd on slot 1 and indicate that PDSCH will be transmitted on slot 3, and then the HARQ feedback is sent on slot 8. This mechanism provides gNB lots of flexibility (k0, k1) to schedule DL packet transmission and the feedback, gNB can adjust k0 and k1 based on PDB of XR packets, therefore, no further enhancement for Dynamic DL Scheduling is needed. Typically, PDCCH and PDSCH is in the same slot, that means k0 is 0, no additional delay in scheduling DL dynamically, SPS doesn't offer any latency benefit. K1 is the same for both SPS and dynamic scheduling, so there are no latency benefits offered by SPS as well.

Figure 1: Dynamic DL Scheduling

Observation 2: Existing NR DL scheduling mechanism are sufficient to support DL XR services.

There are also two types of scheduling for uplink. One is called 'Dynamic Scheduling' and the other one is CG (Configured Grant). Dynamic scheduling is the mechanism in which each and every PUSCH is scheduled by DCI. CG is the mechanism in which the PUSCH transmission is scheduled by RRC message.

For NR Dynamic UL Scheduling as shown in Figure 2. If there is no UL resource for data transmission, UE needs to send SR first to request for UL resource. Usually, the first UL grant after UE’s SR has a small PUSCH size as the gNB doesn’t know how much data is in the UE’s buffer. The gNB can only know the UE’s buffer status after the UE sends BSR to the gNB. In addition, there are additional scheduling delays (k2) between the UE receiving the UL grant from the gNB and the UL transmission. From above analysis, we see three points that can be further optimized for UL AR services:

- SR delay due to long SR periodicity

- Additional grant delay due to unknown UE’s buffer status from gNB side, gNB needs to first schedule a small PUSCH size to know how much buffer data UE has.

- Scheduling delay after UE receives UL grant from gNB

Observation 3: NR dynamic UL scheduling has room for latency improvement.

Proposal 1: RAN2 to study enhanced SR design for dynamic UL scheduling latency reduction.

Figure 2: Dynamic UL Scheduling

UL configured grant (CG) can partially solve some problems seen with dynamic UL scheduling mechanism, for example: long SR delay problem and scheduling delay. However, there is still some room for improvement to meet UL AR service requirements. For example: current CG period is not perfectly match with UL AR services. We use FPS = 60 (period = 16.67ms) under FDD and TDD (pattern: DDDSU) scenarios as example:

Table 2: Additional delay due to CG period mismatch with UL AR period

| CG period (ms) | xDD | SCS (kHz) | Additional average delay due to period mismatch (ms) |
| --- | --- | --- | --- |
| 16 | FDD | 30 | 7.9999 |
| 16 | TDD | 30 | 8.5522 |
| 16 | TDD | 120 | 8.2551 |
| 16.67 | FDD | 30 | 0 |
| 16.67 | TDD | 30 | 0 |
| 16.67 | TDD | 120 | 0 |

From Table 2 above, we can observe that periodicity mismatch problem leads to additional delay for UL data transmission.

Proposal 2: RAN2 to study the alignment of UL CG with typical XR frame rates

For fixed packet size and no jitter traffic such as pose/control traffic, CG can perfectly handle them. However, for UL AR traffic, packet size variations and jitter pose big challenges to the use of CG mechanism. We require enhancements to the CG mechanism to address these challenges.

For jitter, the solution can be the hybrid use of dynamic scheduling and CG mechanism. The solution is to use dynamic grant scheduling for early traffic arrival and configured grant scheduling for late traffic arrival. Dynamic grant scheduling suffers from extra latency due to the extra signaling overhead and the longer SR procedure. However, this can be tolerated by packets that arrive early. Configured grant resources could be allocated for packets that arrive late. When coupled together, the CG resources could be released if a DG is received, allowing these CG resources to be re-used by the gNB to schedule other UEs. An example of how this hybrid DG/CG mechanism would work is outlined below:

If jitter < α

Send SR

If DG received, then use it and release CG resources

Else

Use CG

End

Where α could be for example equal to zero. Hence, packets arriving with negative jitter will use DG scheduling and packets arriving with positive jitter will rely on CG scheduling.

Figure 3: Hybrid dynamic scheduling and CG mechanism for UL AR

Proposal 3: RAN2 to study the use of a hybrid DG/CG scheme to address UL AR traffic jitter.

For UL AR variable packet size problem, the solution can be that gNB first introduce a set of CG resources corresponding to traffic arrival. In the first CG resource, include an indication of number of CG resources that will be used. Then, gNB can re-schedule the remaining unused CG resources for other UEs.

Figure 4: Enhanced CG mechanism for UL AR variable packet size problem

Proposal 4: RAN2 to study ways to dynamically adapt the size of CG resources to address varying UL AR packet sizes.

2.3	Measurements Gaps (MG)

In measurement gap as shown in Figure 5, NW cannot schedule UE to transmit/receive data. Therefore, it leads to a significant loss of capacity. From [3], it can be observed that the XR DL Capacity falls from 10 (no MG) to less than 2 (Measurement Gap Repetition Period=80, Measurement gap length=6) and less than 1 (MGRP=40, MGL=6) as in Figure 6.

Figure 5: An example for measurement gap configuration

Figure 6: Capacity – max user number with (90% User @ “99% frame Tx done < 10ms”)

Proposal 5: RAN2 to study enhanced measurements gaps solutions for XR application capacity problem.

# 3	Conclusion

We have the following observations and proposals:

Observation 1: Both XR DL and UL traffic have high data rate and strict latency requirement

Observation 2: NR DL scheduling mechanism can support DL XR services.

Observation 3: NR dynamic UL scheduling has rooms for latency improvement.

Proposal 1: RAN2 to study enhanced SR design for dynamic UL scheduling latency reduction.

Proposal 2: RAN2 to study the alignment of UL CG with typical XR frame rates

Proposal 3: RAN2 to study the use of a hybrid DG/CG scheme to address UL AR traffic jitter.

Proposal 4: RAN2 to study ways to dynamically adapt the size of CG resources to address varying UL AR packet sizes.

Proposal 5: RAN2 to study enhanced measurements gaps solutions for XR application capacity problem.

# 4	References

[1]	RP-213587, New SID “Study on XR Enhancements for NR”, Nokia, RAN Meeting #94e

[2]	TR 38.838

[3]	R1-2204699, On XR specific capacity improvement enhancements, MediaTek Inc.