3GPP TSG-RAN WG2 Meeting #119 electronic	 R2-2207721

Online, August 17th- August 29th, 2022

Agenda Item:    8.13.3

Source:         Xiaomi

Title:         	Discussion on the SON/MDT enhancement for NPN and RACH report

Document for:	Discussion and Decision

Introduction

According to the WID[1] of the Rel-18 SON/MDT enhancement, The objective of this work item is to specify data collection enhancement in NR for SON/MDT purpose, and one of the specific objective is as follows:

- Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PScell change report

- Successful Handover Report (e.g. inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

In this contribution, we provide our considerations on the support of SON/MDT enhancement for NPN and RACH report.

Discussion

  - Support of SON/MDT enhancement for NPN

Currently, the data collection enhancement in NR for SON/MDT purpose is only supported in the PLMN scenario while that in the NPN scenario is not supported. In R18, the SON/MDT enhancement in NPN scenario is considered to be supported for the network optimization.

For the NPN, it can be deployed as a Stand-alone Non-Public Network (SNPN) which is operated by an NPN operator and not relying on the network functions provided by a PLMN, or a Public network integrated NPN (PNI-NPN) which is deployed with the support of a PLMN. To support the SON/MDT enhancement for NPN, both scenarios can be considered in this stage.

Proposal 1: The support of SON/MDT enhancement in both SNPN and PNI-NPN scenarios can be considered.

Currently, the NG-RAN node is not aware of the Non-Public Network (NPN) information of the cell during the accessing and mobility, which make the operator unable to get the user load of different NPN for the network capacity optimization. Therefore, to support the SON/MDT enhancement for NPN, it is benefit to record the NPN information of the cell (e.g. NPN ID) during the accessing and mobility, and including that into all existing reports can be considered as baseline.

Proposal 2: To support the SON/MDT enhancement for NPN, UE can record the NPN information of the cell (e.g. NPN ID) during to accessing and mobility.

Proposal 3: Including the NPN information (e.g. NPN ID) into all existing reports can be considered as baseline.

  - Support of SON/MDT enhancement for RACH report

SN RACH Report

In R17, RAN3 has completed the work on network interface signaling to support the RACH report for SN for both EN-DC cases and MR-DC cases connected with 5GC. However, from RAN2 perspective, the SN RACH report is only supported in NR-DC case while that in (NG) EN-DC and NE-DC cases are not supported. Thus in R18, the support of SN RACH report in the (NG) EN-DC and NE-DC scenarios can be considered.

According to the R17 SON/MDT discussion, RAN2 has the following agreements [2]:

- UE reports the SN RACH report to the MN, and then MN sends the SN RACH report to SN.

Based on this, for the support of the SN RACH report in the NE-DC scenarios, UE shall support to record the RACH information in the E-UTRAN cell and report to the NR MN, which requires for the TS 38.331 modification.

Proposal 4: To support the SN RACH report in the NE-DC scenario, UE shall support to log the E-UTRAN RACH information and report to the NR MN.

However, for the case of (NG) EN-DC, to support the MN fetching the SN RACH report, UE shall support to report the RACH information recorded in the NR cell to the E-UTRAN, which requires for TS 36.331 modification. To avoid the impacts on the LTE impacts, the SN RACH report in the (NG) EN-DC scenario can be deprioritized. Or as an alternative, the NR SN fetching the list of NR RA reports can be considered, i.e. UE could report the recorded SgNB related RACH information to the ‎current NR SN node via SRB3 which only requires for the TS 38.331 modification.

Proposal 5: To avoid the LTE impacts, the NR SN fetching the list of NR RA reports via SRB3 can be considered for the SN RACH report in the (NG) EN-DC scenario.

RACH report for RACH partitioning

In R17, the RACH partitioning for the feature combination is introduced to achieve the feature early indication or the RA isolation. To support the RACH optimization, the RACH information related to the RACH partitioning can be collected for SON/MDT enhancement.

Proposal 6: Extend current RA report to include the RACH partitioning related information.

The RACH partitioning is supported for both 2-step RACH and 4-step RACH. Considering that the 2-step RACH and 4-step RACH related information report have been supported for the RACH optimization, the straightforward way to support the RACH report for the RACH partitioning is to reuse the RACH related information included in R16 and R17 RA report, with the extension to include the feature combination associated with the selected RACH partition.

Proposal 7: The RACH related information included in R16 and R17 RA report can also be applied for RACH partitioning RA report, with the extension to include the feature combination associated with the selected RACH partition.

In additional, currently, as limited by the RACH resource and actual network deployment, the network may not provide specific RACH partitions for all feature combinations. In this case, UE may select RACH partitions based on the configured feature priority, and the selected RACH partition may not support all features applicable to the random access procedure, in other words, UE intended feature combination may not be supported by the selected RACH partition. Thus, to optimize RACH resource allocation, the UE intended feature combination can also be reported to the network as part of RACH partitioning RA report.

Proposal 8: To include the UE intended feature combination as part of RACH partitioning RA report.

Conclusions

During the discussion above, we have the following observations and proposals:

Proposal 1: The support of SON/MDT enhancement in both SNPN and PNI-NPN scenarios can be considered.

Proposal 2: To support the SON/MDT enhancement for NPN, UE can record the NPN information of the cell (e.g. NPN ID) during to accessing and mobility.

Proposal 3: Including the NPN information (e.g. NPN ID) into all existing reports can be considered as baseline.

Proposal 4: To support the SN RACH report in the NE-DC scenario, UE shall support to log the E-UTRAN RACH information and report to the NR MN.

Proposal 5: To avoid the LTE impacts, the NR SN fetching the list of NR RA reports via SRB3 can be considered for the SN RACH report in the (NG) EN-DC scenario.

Proposal 6: Extend current RA report to include the RACH partitioning related information.

Proposal 7: The RACH related information included in R16 and R17 RA report can also be applied for RACH partitioning RA report, with the extension to include the feature combination associated with the selected RACH partition.

Proposal 8: To include the UE intended feature combination as part of RACH partitioning RA report.

References

RP-221825, CMCC, “New WID on further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)”.

R2-2204401, Report of 3GPP TSG RAN WG2 meeting #117-e, Online.