3GPP TSG-RAN WG2 #119e	Tdoc R2-2207728

Electronic meeting, 2022-08-17 - 2022-08-29

Agenda Item:	6.2.3.1 Corrections to CPAC network aspects

Source:	Ericsson

Title:	Outstanding issue for CPC

Document for:	Discussion, Decision

# 1	Introduction

In rel-17 the work with standardizing inter-SN CPC was done. However, there is still an outstanding issue in the RRC specification related to CPC which needs to be resolved.

# 2	Discussion

## 2.1	Upper limit of number CPC candidates

In the latest version of 38.331 [1] there is still an FFS related to CPC. In CG-Config the source SN may include a list of CPC candidates which it proposes the target SN to consider configuring for CPC. However, the maximum number of candidates that the source SN may propose is still FFS:

CandidateCellInfoListCPC-r17 ::= SEQUENCE (SIZE (1..ffsUpperLimit)) OF CandidateCellInfo-r17   -- FFS

This FFS needs to be resolved. So far RAN2 didn’t agree on a suitable value for the maximum number of candidates. The UE may be configured with maximum eight conditional reconfigurations, so there does not seem to be a strong need to allow the source SN to propose more than eight candidates. On the other hand, there does not seem to be a reason to limit it to less than eight either. So RAN2 is proposed to consider the value eight for the maximum number of CPC candidates that the SN may propose.

The source SN may propose up to eight candidates for the target SN to consider for CPC.

This value should be added in a correction CR for CPAC.

Include the change of “ffsUpperLimit” to “8” in CandidateCellInfoListCPC in a correction CR for CPAC.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The source SN may propose up to eight candidates for the target SN to consider for CPC.

Proposal 2	Include the change of “ffsUpperLimit” to “8” in CandidateCellInfoListCPC in a correction CR for CPAC.

# References

TS 38.331, v 17.1.1, Radio Resource Control (RRC) protocol specification