3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2207730

Online, August 17 – 29, 2022

Agenda item:	8.11.2

Source:	Sharp

Title:	PTM Configuration for RRC_INACTIVE

WID/SID:	NR_MBS_enh-Core

Document for:	Discussion and Decision

1	Introduction

Rel-18 NR MBS aims to further enhance the NR multicast/broadcast functions based on Rel-17 NR MBS. One of the objectives is to support multicast reception by UEs in RRC_INACTIVE state [1]:

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3]

  - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2]

  - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE.  (Seamless/lossless mobility is not required) [RAN2, RAN3]

In this discussion paper, we discuss the issue related to PTM configuration for UEs receiving multicast in RRC_INACTIVE state..

2 Discussion

Two different delivery mode for multicast and broadcast are defined respectively in Rel-17 NR MBS.

For broadcast service, one copy of data are provided simultaneously to all UEs in a geographical area. UE is not required to interact with network when receive the data of a broadcast session and all UEs in the broadcast service area are authorized to receive the data from the broadcast session. A UE can receive a broadcast session in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. And broadcast MRB configuration is provided to UE by broadcast RRC signalling.

Observation 1: In Rel-17 NR MBS, all UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state in the broadcast service area are authorized to receive the data from a broadcast session.

For multicast service, one copy of data are provided simultaneously to a dedicated set of UEs. UE authorized to receive the data needs to join a MBS session before it can receive the data of the multicast session (i.e., not all UEs in the multicast service area are authorized to receive the data). A UE can receive a multicast service only in RRC_CONNECTED state with mechanisms such as PTP and/or PTM delivery. gNB sends RRCReconfiguration message to configure the MRB for the multicast session to the UE when the multicast session is activated.

Observation 2: In Rel-17 NR MBS, UE in RRC_CONNECTED state is authorized to receive a multicast session only when relevant MBS configuration is configured to UE by dedicated RRC signalling.

So, UE in RRC_INACTIVE state is only authorized to receive broadcast session and the multicast session can only be received by UEs in RRC_CONNECTED.

Observation 3: In Rel-17 NR MBS, UE in RRC_INACTIVE is only authorized to receive broadcast session and the multicast session can only be received by UEs in RRC_CONNECTED.

Table 1 shows the difference between Rel-17 NR broadcast and multicast. As showed in table 1, the mechanism defined for broadcast and multicast is totally different.

Table 1 Rel-17 NR Broadcast VS. Multicast

|  | Broadcast | Multicast |
| --- | --- | --- |
| Session join | Not required | required |
| Received by | All UEs | A group of UEs |
| Supported RRC state | RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED | RRC_CONNECTED |
| MRB configuration | Broadcast RRC signalling | Dedicate RRC signalling |
| HARQ ACK_NACK | Not support | support |
| Delivery Mode | PTM | PTM and PTP |

From the supported RRC state point of view, it seems the mechanism defined for broadcast can be reused for UE in RRC_INACTIVED to receive multicast service. But considering that multicast service aims to be received by a group of UEs but not all UE, broadcast-based mechanism is not suitable. So Rel-17 multicast mechanism is used as baseline for multicast reception for UE in RRC_INACTIVE state and the MRB used for multicast reception in RRC_INACTIVE state is configured by gNB in dedicated RRC signalling.

Proposal 1: The MRB used for multicast reception in RRC_INACTIVE state is configured by gNB in dedicated RRC signalling as in Rel-17 NR multicast.

In Rel-17 NR multicast, both PTM and PTP delivery mode are supported for UE in RRC_CONNECTED state. But for Rel-18 MBS, one of the purpose to support multicast received in RRC_INACTIVE is to reduce the power consumption for these UEs that has no other unicast transmission. So, PTP delivery for multicast in RRC_INACTIVE state should not be supported.

Proposal 2: PTP delivery for multicast in RRC_INACTIVE state is not be supported.

3 Summary

In this section, we summarized our proposals and observation as showed below:

Observation 1: In Rel-17 NR MBS, all UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state in the broadcast service area are authorized to receive the data from a broadcast session.

Observation 2: In Rel-17 NR MBS, UE in RRC_CONNECTED state is authorized to receive a multicast session only when relevant MBS configuration is configured to UE by dedicated RRC signalling.

Observation 3: In Rel-17 NR MBS, UE in RRC_INACTIVE is only authorized to receive broadcast session and the multicast session can only be received by UEs in RRC_CONNECTED.

Proposal 1: The MRB used for multicast reception in RRC_INACTIVE state is configured by gNB in dedicated RRC signalling as in Rel-17 NR multicast.

Proposal 2: PTP delivery for multicast in RRC_INACTIVE state is not be supported.

4 References

[1] RP-213568 New WID: Enhancements of NR Multicast and Broadcast Services.