3GPP TSG-RAN WG2 Meeting #119-e	R2-2207753

# E-meeting, 17th August – 29th August 2022

Source:	vivo

Title:	Discussion on candidate solutions for L1 L2 mobility

Agenda Item:	8.4.2.2

Document for:	Discussion and Decision

1. Introduction

According to the WID of Further NR mobility enhancements [1], one of the objectives is L1/L2 based inter-cell mobility to reduce the mobility latency.

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In this contribution, we will discuss some candidate solutions for L1/L2 based inter-cell mobility.

2. Discussion

2.1  L1/L2 based inter-cell mobility case

According to the description in WID below [1]:

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

It could be seen that the L1/L2 based inter-cell mobility could be applicable for cases of PCell change, PScell change for DC case, and SCell change for CA case. In our understanding, it is better to clarify that common procedure design, as shown in Figure 1 below, should be targeted for all use cases.

In all these use cases, “within one CG” should be restricted according to the WID. Thus, PCell change with PScell change should not be consided for L1/L2 based inter-cell mobility.

Proposal 1: The designed L1/L2 based inter-cell mobility is applicable to following use cases:

- Pcell change (with/without SCell change)

- PScell change (with/without SCell change)

- SCell change (without Spcell change)

2.2 Overall procedure for L1/L2 based inter-cell mobility

For simplicity, we discuss the overall procedure for L1/L2 based inter-cell mobility based on spcell change without SCell change firstly. For other use cases, similar procedure is expected.

In our understanding, the overall procedure of L1/L2 based inter-cell mobility is similar as L3 Spcell change, i.e. handover or PSCell change procedure, including measurement and report, network decision, transmission of RRC configuration of the target cell, HO execution, and HO completion.

According to our understanding, the basic procedure for L1/L2 based inter-cell mobility is illustrated in Figure 1 below.

Figure 1.	 basic procedure for L1/L2 based inter-cell mobility

The procedure in Figure1 is explained as below:

            - Network configuration: the source serving cell provides RRC reconfiguration message to UE, including the configuration of L1 measurement and corresponding reporting associated with candidate cells. Besides, the common and dedicated cell configurations of the candidate cells will also be pre-configured.

            - L1 measurement: UE performs the L1 measurement based on the DL RS or candidate gNBs perform L1 measurement based on the UL RS from UE, and the measurement result will be reported/forwarded to the source serving cell.

            - Network decision on L1/L2 HO: network decides to perform L1/L2 HO and indicates UE the TCI state associated with the target cell and the target cell ID via L1/L2 signalling.

            - L1/L2 HO execution: UE performs the serving cell change procedure, including applying the RRC configuration associated with the target cell.

Proposal 2: The basic procedure of L1/L2 based inter-cell mobility contains:

- Network configures UE with candidate cells, including L1 measurement and reporting configurations based on DL or UL signal, and the common and dedicated configurations for candidate cells are also pre-configured via RRC.

- L1 measurement performed at UE for DL or at network for UL, and report/forward the measurement results to the serving cell.

- Network decides to perform HO, and indicates UE the HO command via L1/L2 signaling, including TCI state of the target cell and the target cell ID.

- After receiving L1/L2 HO command, the UE applies the RRC configuration associated with the target cell and accesses to the target cell.

- After accessing the target cell, the L1/L2 HO is completed.

For SPcell change with SCell change, the RRC configuration of target cell group includes the SCell configuration. Upon the reception of L1/L2 HO command which indicates the cell ID and TCI state of the target SPcell cell, UE applies the RRC configuration associated with the SPcell cell and the configuration of SCell. In this way, the UE performs SPcell change with SCell in one procedure. But anyway, it is up to network whether to include SCell configurarion in case of SPcell change by L1/L2 HO.

Proposal 3: For SPcell change with SCell change, upon the reception of L1/L2 HO command which indicates the cell ID and TCI state of the target SPcell, UE applies the pre-configured RRC configuration of the related cell group, including both SPcell and SCell.

2.3 Solutions on L1/L2 based inter-cell mobility

2.3.1 Unified TCI states

In R17 inter-cell beam management, the unified TCI framework was introduced with the following characteristics:

- A pool of joint or separate DL/UL TCI states is introduced for joint/separate DL/UL TCI state update (beam indication)

  - Configuring a list of UL TCI state information in BWP-uplinkDedicated

  - Configuring a list of DL or joint TCI state information in PUSCH-Config

  - The additionalPCI in the TCI state indicates that this TCI state refers to a PCI different from the PCI of the serving cell, as configured in ServingCellConfig

- Support the following source RS type as a QCL reference for DL TCI or joint TCI, or an UL TX spatial reference for UL TCI:

  - SSB associated with a PCI same or different from the serving cell

- For inter-cell, DL UE-dedicated channels follow the indicated unified TCI state, and non-UE-dedicated channels and RSs don’t follow the indicated unified TCI state associated with PCI different from the serving cell.

Observation 1: Unified TCI state introduced in R17 ICBM contains at least some TCI states associated with PCI different from that of the serving cell.

The Beam indication introdcued in R17 ICBM was based on the unified TCI state and has the following characteristics:

- MAC-CE-based and DCI-based beam indication are used to indicate joint or separate DL/UL beam indication from the active TCI states, and UE acknowledges successful decoding of beam indication.

  - Both MAC-CE based and MAC-CE + DCI-based beam indication schemes are supported

Observation 2: Both MAC-CE based and MAC-CE + DCI-based beam indication in R17 ICBM based on the Unified TCI staten are supported.

For R18 L1/L2 based inter-cell mobility, we think it is more reasonable to reuse the unified TCI state framework for beam indication, and re-use the R17 ICBM beam indication as the baseline for R18 L1/L2 mobility indication.

2.2.2 RRC configuration of candidate cells

As shown in the basic procedure above, the RRC configuration of candidate cells includes the L1 measurement and report configuration, and it also contains the common and dedicated cell configuration of the candidate cell, e.g. the PDCCH/PUCCH/PDSCH/PUSCH configuration. In our view, the RRC configuration should be pre-configured before initiation of L1/L2 HO is performed for latency reduction.

For R17 ICBM, the configuration of candidate cells were included in the IE SSB-MTC-AdditionalPCI of servingcellConfig, which contains the PCI and SSB information. In R17 ICBM, due to no serving cell change, the candidate cells and serving cell share a set of common and dedicated cell configurations, that means there is no candidate cell related configuration information in R17 ICBM RRC configuration.

Observation 3: The RRC configuration of candidate cells is pre-configured in ServingCellConfig for R17 ICBM, which only includes the L1 measurement and report configuration.

According to the characteristics of R17 ICBM, it could be seen that the R17 ICBM provides a good framework for L1 measurement and reporting, RRC pre-configuration and beam indication, which can be the starting point for the design of L1/L2 based inter-cell mobility procedure.

However, R17 ICBM doesn’t change the serving cell while R18 L1/L2 based inter-cell mobility will change the serving cell. Besides, there are more applicable scenarios for R18 L1/L2 based inter-cell mobility, e.g. the inter-DU case. Hence some additional enhancements need to be introduced, e.g. the pre-configuration should include common and dedicated information of the candidate cells.

Proposal 4: The R17 ICBM framework should be used as the starting point for the design of R18 L1/L2 inter-cell mobility, including the following aspects:

- Reuse the R17 unified TCI framework.

- Reuse the R17 ICBM beam indication with TCI state ID as the starting point, and potentially adding cell ID.

- Reuse the R17 ICBM pre-configuration framework, and potentially introducing more RRC configurations associated with candidate cells besides SSB and PCI.

2.2.3 L1 measurement

Regarding the L1 measurement and reporting, it can be performed in only serving cell in R15/R16, while L1 measurement and reporting for neighbour cells was introduced in R17 ICBM.  However, the L1 measurement and reporting in R17 ICBM are only applied to intra-frequency and synchronized scenarios and only use the SSB for non-serving cell measurement and reporting. In our view, the  L1 measurement based on DL RS for R18 inter-cell mobility could use the R17 ICBM mechanism as baseline, while some enhancements need to be studied for inter-frequency and asynchronized scenarios.

Besides, gNB performs L1 measurement based on the UL reference signal from the UE can also be considered. Compared with L1 measurement based on DL RS, L1 measurement based on UL RS can reduce the latency and signaling overhead of L1 measurement reporting.

In both cases, the source gNB could acquire the L1 measurement results and make the decision on L1/L2 inter-cell mobility.

Proposal 5: L1 Measurement based on both DL and UL RS should be considered in L1/L2 inter-cell mobility.

3. Conclusion

In this contribution, we discuss candidate solutions for L1/L2 inter-cell mobility. Based on the discussion, we have the following observation and proposals:

Observation 1: Unified TCI state introduced in R17 ICBM contains at least some TCI states associated with PCI different from that of the serving cell.

Observation 2: Both MAC-CE based and MAC-CE + DCI-based beam indication in R17 ICBM based on the Unified TCI staten are supported.

Observation 3: The RRC configuration of candidate cells is pre-configured in ServingCellConfig for R17 ICBM, which only includes the L1 measurement and report configuration.

Proposal 1: The designed L1/L2 based inter-cell mobility is applicable to following use cases:

- Pcell change (with/without SCell change)

- PScell change (with/without SCell change)

- SCell change (without Spcell change)

Proposal 2: The basic procedure of L1/L2 based inter-cell mobility contains:

- Network configures UE with candidate cells, including L1 measurement and reporting configurations based on DL or UL signal, and the common and dedicated configurations for candidate cells are also pre-configured via RRC.

- L1 measurement performed at UE for DL or at network for UL, and report/forward the measurement results to the serving cell.

- Network decides to perform HO, and indicates UE the HO command via L1/L2 signaling, including TCI state of the target cell and the target cell ID.

- After receiving L1/L2 HO command, the UE applies the RRC configuration associated with the target cell and accesses to the target cell.

- After accessing the target cell, the L1/L2 HO is completed.

Proposal 3: For SPcell change with SCell change, upon the reception of L1/L2 HO command which indicates the cell ID and TCI state of the target SPcell, UE applies the pre-configured RRC configuration of the related cell group, including both SPcell and SCell.

Proposal 4: The R17 ICBM framework should be used as the starting point for the design of R18 L1/L2 inter-cell mobility, including the following aspects:

- Reuse the R17 unified TCI framework.

- Reuse the R17 ICBM beam indication with TCI state ID as the starting point, and potentially adding cell ID.

- Reuse the R17 ICBM pre-configuration framework, and potentially introducing more RRC configurations associated with candidate cells besides SSB and PCI.

Proposal 5: L1 Measurement based on both DL and UL RS should be considered in L1/L2 inter-cell mobility.

4. References

- RP-221799, Revised WID: Further NR mobility enhancements, MediaTek.