3GPP TSG-RAN WG2 Meeting #119-e                                                                 R2-2207771

Online, Aug 17 –Aug 29, 2022

Source:	CATT, CBN

Title:	Discussion on Multicast Reception in RRC_INACTIVE

Agenda Item:	8.11.2

Document for:	Discussion and Decision

Introduction

This contribution discuss the following objective in the WID [1]

Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3] - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3]

The contribution is organized as the following. Section 2 discusses from high level the overall procedures, requirements and scenarios to support multicast reception by UEs in RRC_INACTIVE state. Section 3, 4 and Section 5 provide some initial discussions on the technical issues regarding PTM configuration, mobility and state transition, and multicast session state change. The proposals are summarized in Section 6.

Overall procedures, requirements and Scenarios

Before going to the detailed discussions for the sub-bullets in objectives as shown in Section 1, we may need to firstly discuss from high level the overall procedures, requirements and scenarios that impact these sub-bullets.

Main scenarios for multicast reception in INACTIVE

Based on [1], motivations of multicast reception for UEs in INACTIVE include a) in some cases there are large number of UEs in a cell which means the Rel-17 mechanism which requires all UEs in CONNECTED may not fulfill the requirements for this kind of cases, and b) for the sake of UE power efficiency. With such understanding, we believe the main scenarios for multicast reception in INACTIVE should cover both of the following scenarios, i.e.,

- Scenario 1: UEs are receiving a multicast session in CONNECTED, but due to e.g., increased number of UEs or amount of traffics in the cell, there is a need to move some of the UEs to INACTIVE but still continue with the multicast session reception.

- Scenario 2: UEs has joined a multicast session and are in INACTIVE, but due to activation of the multicast session, some or all of UEs can stay in INACTIVE (i.e., no need to go to CONNECTED) and are able to receive the multicast session.

The detailed mechanism can be further discussed for these scenarios.

Proposal 1	In Rel-18, multicast reception for UEs in INACTIVE supports the following scenarios:

- Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues the multicast reception.

- Scenario 2: a UE has joined a multicast session and has been in INACTIVE, the UE starts to receive the multicast session upon activation of this session without going back to CONNECTED.

Who decides whether for a multicast session ‘reception in INACTIVE’ is enabled, and how

For the previously mentioned scenarios, there are questions that who is to decide whether a multicast session may be received in INACTIVE (or a UE should receive this session in INACTIVE), and how it is decided.

Options are RAN node (gNB), UE, or even 5GC to do such decisions. In our understanding it should be gNB, given that the gNB knows the situations including traffic, number of UEs, and other possible information. Of course, it can be further discussed what information gNB may need to form such decisions. For example, SA2 is currently discussing possible assistance information to RAN.

Proposal 2	It is up to gNB to decide whether a multicast session may be received by UE(s) in INACTIVE. FFS what information gNB may be provided to form such decision.

Co-existence with Rel-17 UEs or UEs receiving multicast in CONNECTED

It is clear the Rel-18 mechanism should introduce no impact to Rel-17 UEs. Furthermore, in our understanding the scenario where both UEs in CONNECTED (Rel-17 or Rel-18 UEs) and INACTIVE (Rel-18 UEs only) are receiving the same multicast session should not be ruled out.

Proposal 3	It is supported that gNB transmit the same multicast session to both UEs in CONNECTED (Rel-17 or Rel-18 UEs) and INACTIVE (Rel-18 UEs only) in the same cell.

For a given multicast session, from resource efficiency point of view gNB should typically use the same resources for PTM transmissions to all the interested UEs. This has been the assumption in Rel-17. In our view, this should be the baseline no matter which RRC state UEs are in.

Proposal 4	The following is taken as baseline: the same user plane resources (i.e. resources used for MTCH) are assumed for all UEs (including UEs in CONNECTED and/or INACTIVE states) for receiving the same multicast session.

PTM configuration

In order for UEs to receive a multicast session in INACTIVE, the corresponding configurations (i.e., PTM configurations in the context of this WID objective) need to be provided to the UEs by the network. Basically, these are what have been defined in Rel-17 and may include configurations for L2 configurations (MRB configuration, PDCP, RLC and MAC parameters), as well as L1 configurations (CFR, etc.)

Some further discussions are provided in the remainder of this section, regarding how to deliver the configuration and what are the contents of the configuration.

How to deliver multicast PTM configuration to INACTIVE UEs

In Rel-17, the PTM configurations are provided to UEs via dedicated RRC signaling. Based on Proposal 1, we can further discuss possible options for UEs to obtain the PTM configurations for use in INACTIVE.

| Scenarios (see Proposal 1) | How to deliver to a UE the PTM configurations for use in INACTIVE |
| --- | --- |
| Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues to receive the multicast reception. | Option a) PTM configurations are delivered to a UE only via dedicated RRC signaling (supported in Rel-17) when UE is in CONNECTED Option b) PTM configurations may be delivered in RRCRelease message when UE is moved from CONNECTED to INACTIVE Note: Option a) and b) assume UE store the configuration that it received in CONNECTED. It can be further discussed under which conditions the configurations remain valid. These options do not work for scenario 2 as it requires UEs to go back to CONNECTED. |
| Scenario 2: a UE has joined a multicast session and has been in INACTIVE, the UE starts to receive the multicast session upon activation of this session without going back to CONNECTED. | Option c) PTM configurations may be delivered by common control channel (e.g., MCCH) so that a UE may obtain the configurations when the UE is in INACTIVE Option b) PTM configurations may be delivered in RRCRelease message as a response to UE’s RRCResumeRequest, so that UE may obtain the configurations without going back to CONNECTED |

While the details of these options can be further discussed, our understanding for their pros and cons are mainly that Option a) uses existing mechanism but may not fully solve the issue as discussed already in section 2, Option b) requires some standardization effort and similar as for Option a) it might not fully solve the issue, and Option c) seems meaningful to handle the case with higher number of UEs or amount of traffics in the cell, but it may require more standardization effort.

Proposal 5	The following potential options are further discussed for PTM configurations for multicast reception in INACTIVE

- Option a) PTM configurations are delivered to a UE only via dedicated RRC signaling (supported in Rel-17) when UE is in CONNECTED

- Option b) PTM configurations may be delivered in RRCRelease message (e.g. when UE is moved from CONNECTED to INACTIVE, or as a response to RRCResumeRequest from UE)

- Option c) PTM configurations may be delivered by common control channel (e.g., MCCH) so that a UE may obtain the configurations when the UE is in INACTIVE

On the content of PTM configuration

From high level, the objective does not intend to change how multicast reception works as has been specified in Rel-17. So the baseline for configuration content is to reuse the Rel-17 PTM configuration whenever possible.

According to our initial analysis, most of the PTM configurations defined in Rel-17 should be still applicable, but there may be some issues to discuss.

For example, in Rel-17 CFR-ConfigMulticast is used to indicate UE specific common frequency resource configuration for multicast, and it is configured as part of dedicated BWP configuration. When a UE is in INACTIVE, it needs further discussion on how to determine the CFR for multicast reception.

Another potential issue is that the physical layer configurations for the HARQ feedback may not be needed if UE does not send HARQ feedback when it is in INACTIVE.

Proposal 6	The following is taken as baseline for PTM configuration for multicast reception in INACTIVE: parameters in the Rel-17 PTM configurations for multicast are reused as much as possible. FFS on potential issues if any, e.g., on handling CFR configuration,  HARQ feedback configuration, etc.

The impact of mobility and state transition

In this section, we provide initial discussions on the main scenarios and potential impacts for mobility and state transition.

Mobility for multicast reception in INACTIVE

One important issue to discuss is the potential impact of cell reselection for a UE in INACTIVE when it is receiving multicast. In our view, if there are neighboring cells on which the same multicast session can be received by UE in INACTIVE, it is reasonable for the UE to prioritize those cells so that multicast reception in INACTIVE can continue after cell reselection. This seems to give more resource efficiency and UE power saving benefits. On the other hand, there may be other options with different level of impacts to the cell reselection procedure, which can be further discussed.

Proposal 7	The following options can be further discussed for cell reselection when UE is receiving multicast in INACTIVE

- Option 1: neighboring cell(s) on which the same multicast session can be received in INACTIVE may be prioritized for cell reselection

- Option 2: a frequency on which multicast reception in INACTIVE is supported may  be prioritized for cell reselection

- Option 3: no enhancement to the cell reselection.

In the following, we discuss the main scenarios for mobility for multicast reception in INACTIVE.

Basically there may be two types of cells from a UE’s standpoint, i.e.,

- Type 1: a cell in which UE can receive the interested multicast session(s) when the UE is in INACTIVE

- Type 2: a cell in which UE cannot receive the interested multicast session(s) when the UE is in INACTIVE

The reason why a cell belongs to cell type 2 may include: a) the gNB does not support multicast at all, b) gNB supports multicast but does not support multicast delivery to UEs in INACTIVE, or c) gNB supports so but the related multicast session(s) are not being transmitted to UEs in INACTIVE (e.g., only to CONNECTED UEs).

In our view, Mobility for multicast reception in INACTIVE should consider all the cases when UE is from or to a cell in which UE can or cannot receive the interested multicast session(s) when the UE is in INACTIVE.

Some initial considerations are provided in the table below regarding the main impacts

| mobility cases (in terms of which cell types to/from) | Potential issues/impact to discuss |
| --- | --- |
| type 1 to type 1 | It is assumed that UE stays in INACTIVE before or after mobility, as it can receive the interested multicast session(s) when the UE is in INACTIVE in both cells. Main issues to further discuss for this case may include: - Issue 1: Before cell reselection, how does UE determine whether the interested MBS session is being transmitted in INACTIVE on a specific neighboring cell - Issue 2：How does UE acquire the PTM configuration for the target cell |
| type 1 to type 2 | It is assumed that UE needs to move to CONNECTED after the mobility procedure. How UE receives the interested multicast session afterwards depends on gNB capability and decision. Main issues to further discuss for this case may include: - Issue 1: Trigger of RRC resume procedure when UE after the mobility procedure - Issue 2：Whether/what information the gNB may need to configure the UE’s multicast reception afterwards. |
| type 2 to type 1 | UE can only receive multicast in CONNECTED before mobility procedure takes place. Therefore it is assumed R17 multicast HO procedure applies. It is up to the network how to configure multicast reception for the UE afterwards (i.e., remains in CONNECTED or moves to INACTIVE, etc.). Seems no issues to discuss further. |
| type 2 to type 2 | UE can only receive multicast in CONNECTED in both cells so Rel-17 mechanism applies. Seems no issues to discuss further. |

Proposal 8	Mobility for multicast reception in INACTIVE should consider the cases when UE is from or to a cell in which UE can or cannot receive the interested multicast session(s) when the UE is in INACTIVE.

State transition for multicast reception in INACTIVE

Based on Proposal 1 in Section 2, we mainly discuss two types of state transition,

- from CONNECTED state to INACTIVE state.

- from INACTIVE state to CONNECTED state.

State transition from CONNECTED state to INACTIVE state

For a UE receiving multicast in CONNECTED, it may be released to INACTIVE due to different reasons (e.g. session deactivation, gNB wanting UE to receive multicast in INACTIVE, etc). Therefore, there is a need for gNB to inform the case to the UE. The straightforward way seems to indicate via RRCRelease. The detailed solution can be discussed further.

Proposal 9 	For state transition from CONNECTED to INACTIVE, it is further discussed how gNB informs the UE whether multicast reception is continued in INACTIVE.

State transition from INACTIVE state to CONNECTED state

For a UE which is receiving multicast in INACTIVE, it is possible to move to CONNECTED and continues with the multicast reception. In our view the procedure is under network’s control, e.g., via network control or some pre-configurations. The detailed solution can be discussed further.

Proposal 10	It is further discussed how state transition from INACTIVE to CONNECTED is triggered for a UE that is receiving multicast session when it is in INACTIVE.

Multicast session state change

In Rel-17, UE in CONNECTED may be released to IDLE/INACTIVE when the session is deactivated. If so, these UEs interested in the multicast session monitor the group paging for notification of session activation. For session release, NAS is expected to inform UE about multicast session release (e.g., to stop monitoring for multicast session activation).

For multicast reception in INACTIVE, session state transition may be further discussed. In the table below, some initial considerations are provided based on comparisons between the cases of Rel-17 and Rel-18.

| Session state change | R17 MBS UE (inactive state) behavior | R18 MBS UE (in inactive state) behavior |
| --- | --- | --- |
| Session deactivation/release | NA | Stop multicast reception |
| Session activation | Enter connected state when group paging is received | - Start multicast reception in inactive state without RRC state change if it is allowed - Enter connected state if inactive multicast reception is not allowed |

In Rel-18, it should be possible that UEs in INACTIVE are informed on the session deactivation, session release. Also, the UEs in INACTIVE should be indicated whether a multicast session can be received in INACTIVE upon session activation. The detailed solutions can be further discussed.

Proposal 11 It is further discussed whether/how to inform UE the session activation, deactivation, release when UE is in INACTIVE.

Conclusion

This contribution initially discusses multicast reception in RRC_INACTIVE and proposes the following:

Overall procedures, requirements and Scenarios

Proposal 1	In Rel-18, multicast reception for UEs in INACTIVE supports the following scenarios:

- Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues the multicast reception.

- Scenario 2: a UE has joined a multicast session and has been in INACTIVE, the UE starts to receive the multicast session upon activation of this session without going back to CONNECTED.

Proposal 2	It is up to gNB to decide whether a multicast session may be received by UE(s) in INACTIVE. FFS what information gNB may be provided to form such decision.

Proposal 3	It is supported that gNB transmit the same multicast session to both UEs in CONNECTED (Rel-17 or Rel-18 UEs) and INACTIVE (Rel-18 UEs only) in the same cell.

Proposal 4	The following is taken as baseline: the same user plane resources (i.e. resources used for MTCH) are assumed for all UEs (including UEs in CONNECTED and/or INACTIVE states) for receiving the same multicast session.

PTM configuration

Proposal 5	The following potential options are further discussed for PTM configurations for multicast reception in INACTIVE

- Option a) PTM configurations are delivered to a UE only via dedicated RRC signaling (supported in Rel-17) when UE is in CONNECTED

- Option b) PTM configurations may be delivered in RRCRelease message (e.g. when UE is moved from CONNECTED to INACTIVE, or as a response to RRCResumeRequest from UE)

- Option c) PTM configurations may be delivered by common control channel (e.g., MCCH) so that a UE may obtain the configurations when the UE is in INACTIVE

Proposal 6	The following is taken as baseline for PTM configuration for multicast reception in INACTIVE: parameters in the Rel-17 PTM configurations for multicast are reused as much as possible. FFS on potential issues if any, e.g., on handling CFR configuration,  HARQ feedback configuration, etc.

The impact of mobility and state transition

Proposal 7	The following options can be further discussed for cell reselection when UE is receiving multicast in INACTIVE

- Option 1: neighboring cell(s) on which the same multicast session can be received in INACTIVE may be prioritized for cell reselection

- Option 2: a frequency on which multicast reception in INACTIVE is supported may be prioritized for cell reselection

- Option 3: no enhancement to the cell reselection.

Proposal 8	Mobility for multicast reception in INACTIVE should consider the cases when UE is from or to a cell in which UE can or cannot receive the interested multicast session(s) when the UE is in INACTIVE.

Proposal 9 	For state transition from CONNECTED to INACTIVE, it is further discussed how gNB informs the UE whether multicast reception is continued in INACTIVE.

Proposal 10	It is further discussed how state transition from INACTIVE to CONNECTED is triggered for a UE that is receiving multicast session when it is in INACTIVE.

Multicast session state change

Proposal 11	It is further discussed whether/how to inform UE the session activation, deactivation, release when UE is in INACTIVE.

Reference

[1] RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services