3GPP TSG-RAN WG2 #119-e	R2-2207796

E-meeting, August 2022

Agenda Item:	6.5.3

Source:	OPPO

Title:	Issue on a CG transmission cancelled by a DG without UL-SCH

Document for:	Discussion, Decision

Introduction

In the RAN2#118-e meeting, there is one issue left.

5	FFS When a CG-PUSCH transmission is cancelled by a DG-PUSCH without UL-SCH (i.e. MAC PDU is not delivered to PHY) in Rel-17, de-prioritization relies on existing Rel-16 LCH-based Prioritization. The CG is not considered as a de-prioritized uplink grant. (no specification change).  Check issue described for UCI only case

This paper focuses on the above issue.

Discussion

In the latest RAN2 meeting, RAN2 has concluded no spec change for the case of a CG-PUSCH transmission cancelled by a DG-PUSCH with UL-SCH. However, there is no consensus on the case of a CG-PUSCH transmission cancelled by a DG-PUSCH without UL-SCH (i.e. UCI only case).

We understand the following is a potential case, i.e., the gNB requests an A-CSI reporting transmitted on an HP DG PUSCH and no data is available for the DG when a MAC PDU associated with an overlapped LP CG has already been delivered to the physical layer. For such DG, the MAC layer cannot deliver the associated MAC PDU to the physical layer, because the MAC layer understands that such DG is a deprioritized grant (based on the LCH-based prioritization rule and also considering that the UCI multiplexing is not a factor of grant prioritization decision [1]). Additionally, this HP A-CSI without UL-SCH can also overlap with other PUCCH whose UCI can be further multiplexed on this HP A-CSI without UL-SCH.

Figure 1

According to RAN1 specs, this A-CSI request is indicated in a DCI and the MAC layer is not aware of the CSI reporting, as a result, it could happen that the MAC layer does not know that the PHY decides to transmit the HP A-CSI without UL-SCH and has dropped the LP CG.

In this case, if the MAC layer does not know the LP CG is eventually deprioritized, the MAC layer will not consider the CG as deprioritized and stop the corresponding timers. As a result, the transmission latency is enlarged or the MAC PDU associated with this LP CG may be lost.

However, in Rel-17, RAN1 focuses on the cases where two MAC PDUs are delivered to PHY for the overlapping between CG and DG. (See RAN1 agreements as below)

- For collision between HP CG PUSCH and LP DG PUSCH, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH.

- For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest.

Thus, we understand this case is similar to the one that already exists in Rel-16(i.e. an LP CG with UL-SCH overlaps an HP A-CSI without UL-SCH), and thus it is more like a legacy issue. With this in mind, we do not see a strong intention to change the MAC spec for this case.

For the case where a CG-PUSCH transmission is cancelled by a DG-PUSCH without UL-SCH (i.e. MAC PDU is not delivered to PHY) in Rel-17, de-prioritization relies on existing Rel-16 LCH-based Prioritization. The CG is not considered as a de-prioritized uplink grant. (no specification change)

Conclusion

We have the following proposals:

Proposal 1	For the case where a CG-PUSCH transmission is cancelled by a DG-PUSCH without UL-SCH (i.e. MAC PDU is not delivered to PHY) in Rel-17, de-prioritization relies on existing Rel-16 LCH-based Prioritization. The CG is not considered as a de-prioritized uplink grant. (no specification change)

Reference

[1] R1-2112862 Reply LS on UL skipping with LCH prioritization