3GPP TSG-RAN WG2 #119-e	R2-2207800

E-meeting, August 2022

Agenda Item:	8.3.2

Source:	OPPO

Title:	Discussion on the UE assistance information

Document for:	Discussion, Decision

Introduction

This paper will discuss the following bullet in SID, i.e., the techniques to improve network energy savings.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded

Discussion

It is approved in the SID to study potential network energy savings techniques to reduce network energy consumption. In the meanwhile, the impact on network and user performance should also be considered. Typically, the more the network power is saved, the more the transmission occasion is reduced and the UE performance is decreased. To avoid the impact on the UE performance, the UE-specific information is necessary and would be reported to the network for a good network energy savings decision.

- The UE assistance information reporting can be used to balance the network energy saving and the data transmission performance.

In the RAN1#109-e meeting, RAN1 has achieved the following directions for further study.

Agreement

Further study techniques and enhancements on assistance information from the UE to aid the gNB to perform energy saving techniques

- Some examples of assistance information are, but not limited to:

  - preferred SSB configurations,

  - indication of semi-static UL channel transmissions,

  - indication of UE’s buffer status for UL channel transmissions,

  - UE traffic information such as service priority, delay tolerance, data rate, data volume, traffic type, time criticality, and packet size(s),

  - coverage, mobility status, location.

  - conditions for triggering the assistance information from the UE

As the above directions are still under evaluation and other information is not excluded, we understand RAN2 can do our analysis and consider which UE assistance information is useful to balance the network energy saving and the UE performance.

With this in mind, we understand that the UE can report the UE preference or the recommended configuration to the network. The intention is to let the network be aware of what the UE wants and adjust the configuration accordingly. We understand the following information can be considered.

- Traffic characteristics

Currently, the gNB cannot be aware of the UE traffic characteristics (e.g. period, arrival time, data size, etc.) by itself or via the core network. If such information can be reported to the gNB, the gNB can do a trade-off and configure a suitable transmission interval based on the traffic characteristics associated with one QoS flow and the QoS parameters of the QoS flow.

The UE traffic characteristics or its variation can be the recommended traffic pattern/resource configuration, DTX preference (the DRX preference is already supported from Rel-16), or cell on/off situation suggestion.

- Request on cell off or cell DTX/DRX

Once there is no transmission requirement for a long time, the connected UE can request its serving cell to turn to the cell off situation or use cell DTX/DRX configuration. The serving gNB can take the request into account when doing a network energy savings decision.

RAN2 studies which UE assistance information to be used for network energy savings decisions.

Conclusion

We have the following observations:

Observation 1	The UE assistance information reporting can be used to balance the network energy saving and the data transmission performance.

We have the following proposals:

Proposal 1	RAN2 studies which UE assistance information to be used for network energy savings decisions.

Reference

[1] RP-213554  New SI: Study on network energy savings for NR