3GPP TSG-RAN WG2 #119-e	R2-2207801

E-meeting, August 2022

Agenda Item:	8.5.2

Source:	OPPO

Title:	Discussion on XR-awareness in RAN

Document for:	Discussion, Decision

Introduction

The following objectives are approved for XR enhancement SID [1].

Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

This paper will discuss the first bullet in SID, i.e., XR-awareness in RAN.

Discussion

SA2 progress on XR

In the current 5G system, the QoS Flow is the finest granularity of QoS differentiation for the PDU Session. It means that each packet in a QoS flow is treated with the same QoS requirements. For XR/media services, the PDU set is additionally introduced to enhance system efficiency and improve the UE experience. In a PDU set, one or more PDUs can be included [2].

Aim to the PDU set, SA2 proposes the 5G system to be aware of the PDU set, including e.g. how to identify the PDUs of one PDU set, what information should be provided to the 5GS for PDU set integrated packet handling, how to provide information to the UE for PDU set integrated packet handling, what is the definition of the importance/dependency information enabling differentiated PDU set handling, etc. Accordingly, SA2 has gathered such PDU-set-related issues in KI#4&5 (PDU Set based handling) and includes the following directions in [2],

- KI#4: The key issue proposes to study PDU set integrated packet handling in 5G network, in which the group of packets belongs to a same PDU Set will be handled in an integrated manner.

- It means that packets in such a PDU set are decoded/handled as a whole.

- KI#5: The key issue proposes to support differentiated QoS handling considering different importance of PDU Sets e.g. by treating packets (i.e. PDUs) belonging to less important PDU Set(s) differently to reduce the resource wasting.

- It means that different QoS handlings may exist between different PDU sets.

In [2], SA2 also indicates the need for coordination with RAN WGs.

Based on the above, a basic question in RAN2 is whether the AS layer needs to be aware of the PDU set and then whether some enhancement is needed to support the PDU set. Considering the PDU set is an XR-specific requirement/characteristic and SA2 already mentions the target e.g., the UE awareness of PDU set integrated packet handling, the UE AS should be aware of the PDU set and consider if any enhancement is needed for the PDU set requirements.

- SA2 introduces a concept of the PDU set for XR service.

- According to TR 23. 700-60, SA2 agrees to support integrated and differentiated PDU set handling, which may need coordination with RAN.

For XR service, RAN2 studies if any AS layer enhancement is needed to support the PDU set.

What to do in RAN2

As mentioned above, we understand some RAN2 impact is anticipated for PDU set handling. Respond to what SA2 proposed in [2], the following aspects can be considered.

- PDU set identification

- Integrated PDU set handling

- Differentiated QoS handling for different PDU sets

PDU set identification

According to the latest SA2 progress, SA2 would like to focus on DL PDU set handling.

Obviously, for DL, RAN2 can follow SA2 preference and wait for the SA2’s further progress.

For DL, RAN2 waits for SA2 progress on how to identify the PDU set.

In [2], several solutions for DL PDU set identification are raised in SA2, which are aimed at the awareness of which PDUs belong to which PDU set and/or what type the PDU set is. Take solution#52 as an example, the following information is proposed to be known.

If RAN2 agrees that PDU set-level operation is also needed in UL, we think the UE AS layer should know some information for the PDU set identification, and the PDU set information proposed for DL can be as candidate information for UL. We think that RAN2 has the responsibility to determine what exactly the PDU set information is and may need to check with SA2 on the feasibility of the determined PDU set information.

For UL, RAN2 has the responsibility to identify the PDU set.

Integrated PDU set handling

In legacy, a UE can discard a packet if the corresponding discard timer expiry. The discard timer is configured by the network taking PDB as one of the configuration factors. For XR/media service, a similar definition, i.e. the PSDB is proposed by one solution for the PDU set [2].

As described in [2], for one PDU set, all or a certain number of PDUs are necessary for PDU set decoding. For GBR QoS Flows using the delay-critical resource type, a PDU set is considered lost if the delay of this PDU set is more than the PSDB.

In reality, one PDU from the upper layer is usually carried by an SDAP PDU and multiple PDUs of one PDU set are hard to be transmitted at the same time. As a result, some PDU(s) of a PDU set may lose before the remaining PDU(s) transmission, or, some PDU(s) of a PDU set have not been transmitted when the PSDB approaches. In some cases, discarding the remaining PDU(s) may benefit resource efficiency, while in other cases, transmitting such PDU(s) is still valuable if the codec can succeed in decoding by using partial information. With this in mind, we think the AS layer can rely on the network configuration and/or decode preference on whether to discard or deliver the remaining PDUs. This mechanism can be performed at least by the TX side, e.g., the transmitting PDCP entity.

Besides, other potential directions for the integrated PDU set handling are the LCP and/or prioritization procedure enhancement if the MAC layer can know the PDU set information.

RAN2 studies whether to deliver or discard the PDU(s) which has not been transmitted for the integrated PDU set requirement.

Differentiated PDU set handling

Although XR has its specific requirement, we think the current principle of the QoS framework should keep as it is and the existing QoS framework should be used as a baseline for XR service.

The current QoS framework can be used as a baseline for XR service.

According to the existing QoS flow framework, the data flows of the same QoS requirement are mapped into one QoS flow and the data flows of different QoS requirements are mapped into different QoS flows. Accordingly, different PDU sets can map with the corresponding QoS flow(s) based on their QoS requirements. As discussed in SA2, different PDU sets can have the same or different QoS requirements. Thus, the different PDU sets can be mapped into one or different QoS flows

In solution#52 in [2], three mapping options are proposed and we think the latter two can be classified as associated with one QoS flow.

Based on the associations between QoS flow and PDU set, we have the following consideration.

- Case1: Assume different PDU sets map into different QoS flows.

  - In this case, we can re-use the current mechanism for different PDU sets handling. How to configure the radio bearer to handle different PDU sets relies on the QoS requirements. For example, the QoS flow/DRB /PDCP entity 1 is associated with one PDU set type and the QoS flow/DRB/PDCP entity 2 is associated with another PDU set type.

Figure 2 Example of different PDU sets mapping into different QoS flows

- Case2: Assume different PDU sets map into a QoS flow.

  - In this case, we understand either of the following two solutions can be considered.

    - Solution1(Figure 3-Left): Map one QoS flow with only one PDCP entity. The PDCP entity links with more than one RLC entity. Different RLC entities are used for different PDU set types. (i.e. Reuse split architecture)

    - Solution2(Figure 3-Right): Map one QoS flow with more than one PDCP entity. Different PDCP entities are used for different PDU set types. Each PDCP entity links with only one RLC entity. (This solution implies the allowance of one to more mapping between one QoS flow and DRBs).

Figure 3 Example of different PDU sets mapping into one QoS flow

Considering SA2 is still in the TR discussion phase and different associations between QoS flow and PDU set may need different RAN2 solutions, we prefer RAN2 continues the study on the differentiated PDU set handling based on the further conclusion in SA2 on the association between QoS flow and PDU set.

- According to the SA2 progress, different PDU sets can map into one or different QoS flows according to their QoS requirements.

RAN2 waits for the SA2 progress on how to map different PDU sets with QoS flow(s) and then continues RAN2 study of the differentiated PDU set handling accordingly.

Conclusion

We have the following observations:

Observation 1	SA2 introduces a concept of the PDU set for XR service.

Observation 2	According to TR 23. 700-60, SA2 agrees to support integrated and differentiated PDU set handling, which may need coordination with RAN.

Observation 3	According to the SA2 progress, different PDU sets can map into one or different QoS flows according to their QoS requirements.

We have the following proposals:

Proposal 1	For XR service, RAN2 studies if any AS layer enhancement is needed to support the PDU set.

Proposal 2	For DL, RAN2 waits for SA2 progress on how to identify the PDU set.

Proposal 3	For UL, RAN2 has the responsibility to identify the PDU set.

Proposal 4	RAN2 studies whether to deliver or discard the PDU(s) which has not been transmitted for the integrated PDU set requirement.

Proposal 5	The current QoS framework can be used as a baseline for XR service.

Proposal 6	RAN2 waits for the SA2 progress on how to map different PDU sets with QoS flow(s) and then continues RAN2 study of the differentiated PDU set handling accordingly.

Reference

[1] RP-220285  New SID “Study on XR Enhancements for NR”

[2] 3GPP TR 23.700-60 Study on XR (Extended Reality) and media services