3GPP TSG-RAN WG2 Meeting #119 electronic   						    	R2-2207805

E-Meeting, 17 – 29 August, 2022

Source: 	Xiaomi

Title:  	Candidate TDM solutions for IDC

Agenda Item:	8.10.3

Document for:	Discussion and Decision

# Introduction

According to the WID [1] of the Rel-18 IDC, the TDM solution is considered complementary to the FDM solution. The objective related to the IDC TDM solution is quoted as follows:

This WI expects to address interference between 3GPP (including various MR-DC architectures, i.e. NR-DC and EN-DC) and non-3GPP RAT (e.g. WiFi). - Introduction of TDM solution (e.g. indication of UE preferred TDM pattern for UL/DL). (RAN2, RAN4). Note: The TDM solution is considered complementary to the FDM solution. Note: LTE IDC solution should be considered as the baseline for the solutions developed in this WI.

In this contribution, we provide our analysis on the LTE IDC TDM solutions and some new solutions for NR. Since the LTE TDM solutions does not consider the impact of DC, some extra standard impacts on EN-DC and NR-DC are also provided.

# Discussion

## LTE TDM baseline solution

According to the 3GPP TS 36.331 and the Section 5.2.1.2 and 5.2.2.1 of the 3GPP TR 36.816, the TDM solutions specified for LTE IDC include the followings:

- Solution 1: DRX based solution (i.e. UE reporting of its preferred DRX configuration drx-AssistanceInfo-r11)

- Solution 2: HARQ process reservation based solution (i.e. UE reporting of its preferred UL/DL subframe pattern idc-SubframePatternList-r11)

- Solution 3: Network controlled UE autonomous denial (i.e. UE autonomous denial based on the configuration autonomousDenialParameters-r11)

The reason to specify three different TDM solutions for LTE is because each TDM solution would cover different use cases. Solution 1 can be used to deal with the periodic interference from/to 3GPP due to periodic traffic data transmission via 3GPP or non-3GPP RAT (e.g. BT voice). Solution 2 can have more accurate interference mitigation for each UL/DL subframe, and “allow the network to ensure at least a pair of clean BT Tx/Rx instances in each BT interval”. Solution 3 is used for “some critical short-term events of ISM side, e.g. some events during BT/WiFi connection-setup or other important signalling”. Based on the 3GPP TR 36.816 [3], the applicability of LTE TDM solutions is summarized in the following table.

Table 1: Applicability of LTE IDC TDM solutions

| TDM solution | LTE+BT earphone (VoIP service) | LTE+BT earphone (Multimedia service) | LTE+GNSS Receiver |
| --- | --- | --- | --- |
| HARQ process reservation based solution | Applicable | Applicable for BT Master, but not applicable for BT Slave | Applicable |
| DRX based solution | Applicable | Applicable | Applicable |
| Autonomous denial solution | Complementary solution for receiving important signalling |  |  |

From our understanding, all of the above three LTE TDM solutions for IDC can be reused for mitigating the IDC interference in the NR frequency, since the use cases for the interference between 3GPP and non-3GPP RAT(s) are the same for both LTE and NR.

Proposal 1: The following LTE TDM solutions for IDC are included in Rel-18 IDC.

- Solution 1: DRX based solution (i.e. UE reporting of its preferred DRX configuration)

- Solution 2: HARQ process reservation based solution (i.e. UE reporting of its preferred UL/DL transmission pattern)

- Solution 3: UE autonomous denial (i.e. UE autonomous denial based on the configuration provided by the gNB)

## MR-DC impacts for TDM solutions

For TDM based solution, since the TDM configuration is per CG (cell group) and the traffic transmission pattern via MCG bearer and SCG bearer would be different, the UE preferred TDM configuration or the autonomous denial configuration for resolving the IDC issue should be per CG. For EN-DC, the LTE TDM solution is not applicable to NR SCG (or SN), since the NR SN is not able to understand the LTE ASN.1 of the UE reporting of its preferred DRX configuration drx-AssistanceInfo-r11 and its preferred UL/DL subframe pattern idc-SubframePatternList-r11, and it is also not applicable for the UE to use the LTE autonomousDenialParameters-r11 to deny the NR SCG transmission.

Proposal 2: The DRX configuration preference or the UL/DL transmission pattern preference reported by the UE is per cell group.

Proposal 3: The autonomous denial configuration is per cell group.

# Conclusions

According to the analysis given above, we have the following Proposals:

Proposal 1: The following LTE TDM solutions for IDC are included in Rel-18 IDC.

- Solution 1: DRX based solution (i.e. UE reporting of its preferred DRX configuration)

- Solution 2: HARQ process reservation based solution (i.e. UE reporting of its preferred UL/DL transmission pattern)

- Solution 3: UE autonomous denial (i.e. UE autonomous denial based on the configuration provided by the gNB)

Proposal 2: The DRX configuration preference or the UL/DL transmission pattern preference reported by the UE is per cell group.

Proposal 3: The autonomous denial configuration is per cell group.

# Reference

[1] RP-221281, Xiaomi, “Revised WID: In-Device Co-existence (IDC) enhancements for NR and MR-DC”.

[2] 3GPP TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification”

[3] 3GPP TS 36.816, “Evolved Universal Terrestrial Radio Access (E-UTRA); Study on signalling and procedure for interference avoidance for in-device coexistence”