3GPP TSG-RAN WG2 Meeting #119 electronic   						    	R2-2207808

E-Meeting, 17 – 29 August, 2022

Source: 	Xiaomi

Title:  	Discussion on shared processing for MBS broadcast and unicast reception

Agenda Item:	8.11.3

Document for:	Discussion and Decision

# Introduction

According to the WID [1] of the Rel-18 MBS enhancements, one objective is to specify “UE assistance information reporting” for simultaneous unicast reception and MBS broadcast reception, which is quoted as follows:

- Specify Uu signalling enhancements to allow a UE to use shared processing for MBS broadcast and unicast reception, i.e., ‎including UE capability and related assistance information reporting regarding simultaneous unicast reception in RRC_CONNECTED and MBS broadcast reception from the same or different operators [RAN2]

In this contribution, we provide our understandings on how the UE provides the assistance information for the network for simultaneous unicast reception and MBS broadcast reception, by using the MBS interest indication specified for LTE and NR.

# Discussion

## Shared processing for MBS broadcast and unicast reception

According to the Rel-17 MBS discussion, the NR MII message includes the following information, by comparing the MBS information (i.e. MBS frequencies and MBS services) provided by SIB21 and USD.

38.331:

According to the LTE ROM (Read-Only-Mode) MBMS discussion, the UE can indicate the ROM MBMS frequency information as quoted below, when SIB2 indicates “mbms-ROM-ServiceIndication”. Since the ROM MBMS frequency is used for ROM MBMS reception, the eNB not supporting MBMS should ensure that the serving frequency configured for the unicast reception of the CONNECTED UE and the ROM MBMS frequency belong to the same supportedBandCombination, so that the serving on unicast serving frequency and MBMS neighbour frequency can be kept.

36.331:

In NR, we think that the MII framework for LTE ROM MBMS can be reused as the baseline. Namely, the gNB receiving the UE assistance information for simultaneous unicast reception in RRC_CONNECTED and MBS broadcast reception does not have to support the Rel-17 MII (i.e. without SIB21), as the Rel-18 function for the UE assistance information for simultaneous unicast reception and MBS broadcast is to avoid the packet loss via the unicast. The gNB can indicate via SIB1 whether the UE assistance information for simultaneous unicast reception and MBS broadcast is allowed. If the UE assistance information for simultaneous unicast reception and MBS broadcast is allowed, the UE can indicate the NR MBS frequency of interest, SCS and bandwidth of the NR MBS frequency.

Observation: the MII framework for LTE ROM MBMS can be reused as the baseline.

Proposal 1: The gNB supporting the UE assistance information for simultaneous unicast reception and MBS broadcast does not have to support the Rel-17 MII, i.e. does not providing SIB21.

Proposal 2: The UE assistance information for simultaneous unicast reception and MBS broadcast includes the NR MBS frequency of interest and the SCS and bandwidth of the NR MBS frequency of interest.

According to 38.331, the NR MBS broadcast is received via the TDM mode, which is provided as follows. For the unicast reception which overlaps with the MBS reception in time, the gNB can use less UE capability for unicast reception (e.g. less L2 buffer and less MIMO layer). For the unicast reception which does not overlap with the MBS reception in time, the gNB can use more UE capability for unicast reception.

38.331:

Proposal 3: The UE can indicate the DRX reception configuration of the MBS frequency of interest.

In the Rel-17 MBS broadcast configuration as quoted below, the PDSCH configuration (e.g. mcs-Table) included will also impact the UE reception capability (e.g. L2 buffer) used for MBS. Then we think that indicating the PDSCH configuration for the MBS frequency can also help the gNB to provide more accurate unicast configuration, so that the packet loss due to exceeding the UE processing capability can be avoided.

38.331:

Proposal 4: The UE can indicate the PDSCH configuration (e.g. mcs-Table) of the MBS frequency of interest.

# Conclusions

According to the analysis given above, we have the following Observation and Proposals:

Observation: the MII framework for LTE ROM MBMS can be reused as the baseline.

Proposal 1: The gNB supporting the UE assistance information for simultaneous unicast reception and MBS broadcast does not have to support the Rel-17 MII, i.e. does not providing SIB21.

Proposal 2: The UE assistance information for simultaneous unicast reception and MBS broadcast includes the NR MBS frequency of interest and the SCS and bandwidth of the NR MBS frequency of interest.

Proposal 3: The UE can indicate the DRX reception configuration of the MBS frequency of interest.

Proposal 4: The UE can indicate the PDSCH configuration (e.g. mcs-Table) of the MBS frequency of interest.

# Reference

[1] RP-221458, CATT, CBN, “Revised WID: Enhancements of NR Multicast and Broadcast Services”.

[2] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”.

[3] 3GPP TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); protocol specification”.