3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2207819

Online, August 17-26, 2022

Agenda Item:	6.8.2

Source:	CATT

Title:	Discussion paper on the mapping between slices and NSAG

Document for:	Discussion and Decision

# 1	Introduction

In Rel-17, the definition of NSAG (Network Slice AS Group) is introduced for slice based cell reselection and slice specific RACH configuration. The mapping relationship between slices and slice groups (NSAG) is specified in the specification of SA2 in detail. But this is not reflected in RAN2’s specification. This discussion paper is trying to provide some proposals on how to reflect the mapping relationship between slices and NSAG in RAN2’s specification.

# 2	Discussion

In Rel-17, in order to avoid exposing the S-NSSAIs in system information and reduce the signalling size, the definition of NSAG is used for slice based cell reselection and slice specific RACH configuration. In RAN2#116e meeting [1], the following agreement is achieved.

- 2 The solution for how the nw operator configures the following (CN and/or RAN OAM): - mapping of slices to slice groups, sent from CN to UE in NAS signalling - broadcast of slice group and its slice specific RACH configuration in SIB.

In RAN2#117e meeting [2], the following agreement is achieved.

- 13. A slice is not associated with multiple slice groups for the same purpose. A slice can be associated with one slice group for RACH and one slice group for reselection. - RAN2 assumes (based on majority views in RAN2) that the mapping of slice to the slice groups for cell reselection are per TA.

According to above agreements, the mapping of slices to slice groups (NSAGs) is used for RACH or cell reselection and the mapping of slice to the slice groups for cell reselection is per TA. The mapping relationship is provided to UE via NAS signalling. It is specified in detail in SA2’s specification TS23.501 [3] as follow. The RAN provides the AMF with the values of the NSAG(s) an S-NSSAI is associated with in a TA using the NG Set Up and RAN Configuration Update procedures. And the AMF configures the UE with the NSAG Information by including this NSAG Information in the Registration Accept message or the UE Configuration Command message.

### 5.15.14 Network Slice AS Groups support The RAN may support Network Slice AS Groups (NSAGs) which are used as specified in TS 38.300 [27] and TS 38.304 [50]. A Network Slice AS Group may be valid in one or more Tracking Areas. The RAN provides (and updates) the AMF with the values of the NSAG(s) an S-NSSAI is associated with in a TA using the NG Set Up and RAN Configuration Update procedures (see TS 38.413 [34]). The AMF in turn provides this information to the NSSF. In deployments where the total number of groups does not exceed the number of groups associated with the NSAG size limit defined in TS 38.331 [28]), all the NSAGs configured in the RAN may be unique per PLMN. If the UE has indicated that the UE supports NSAG in the 5GMM Core Network Capability (see clause 5.4.4a), the AMF may, with or without NSSF assistance, configure the UE with NSAG Information e for one or more S-NSSAIs in the Configured NSSAI, by including this NSAG Information in the Registration Accept message or the UE Configuration Command message. The UE uses the NSAG Information as defined in clause 5.3.4.3.1. The AMF shall indicate in the NSAG Information in which TA a specific NSAG association to S-NSSAI(s) is valid if the AMF provides in the UE configuration a NSAG value which is used in different TAs with a different association with NSSAIs. The configuration the AMF provides includes at least the NSAGs for the UE for the TAs of the Registration Area. ……

However, in current RAN2’s specification [4], it only indicates that “In the UE, NAS provides the NSAG to be considered during RA to AS” and “In the UE, NAS provides the NSAG(s) and their priorities to be considered during cell reselection”. The mapping relationship between slices and NSAG(s) provided by NAS signalling is not specified.  In our understanding, the NSAG is defined in SA’s specification and UE need to know how to map the NSAG to slices. So it is beneficial to specify that the mapping relationship between slices and NSAGs is provided to UE via NAS signalling in RAN2’s specification. The corresponding TPs are provided in Annex A and Annex B.

Proposal: It should be reflected that the mapping relationship between slices and NSAGs is provided to UE via NAS signalling in TS38.300 and adopt the TPs in Annex A.

# 3 Conclusions

According to the above discussion, the following proposal is made:

Proposal: It should be specified that the mapping relationship between slices and NSAGs is provided to UE via NAS signalling in TS38.300 and adopt the TPs in Annex A.

# 4 References

- R2-2201970 Report of 3GPP TSG RAN WG2 meeting #116-e, Online

- R2-2204401 Report of 3GPP TSG RAN WG2 meeting #117-e, Online

- 3GPP TS 23.501v17.5.0: "System Architecture for the 5G System; Stage 2".

- 3GPP TS 38.300 v17.1.0: "NR Overall Description; Stage 2".

# Annex A 	Text Proposal for TS38.300 V17.1.0

Start of change

### 16.3.3	Resource Isolation and Management

#### 16.3.3.1	General

{…Omitted…}

Slice specific RACH configuration for RA isolation and prioritization can be included in SIB1 messages. The slice specific RACH configurations are associated to specific NSAG(s), and if not provided for a NSAG that UE considers for selecting the RACH configuration, then the UE does not consider the NSAG for selecting the slice specific RACH configuration, i.e., the UE uses the common RACH configuration. In the UE, NAS provides the NSAG to be considered during RA to AS. The mapping relationship between slices and NSAG(s) are provided to UE via NAS signalling as specified in TS 23.501[5].

Next change

### 16.3.3a	Slice aware cell reselection

Slice specific cell reselection information can be included in SIB16 and in RRCRelease messages. The slice specific cell reselection information may include reselection priorities per NSAG per frequency and corresponding list(s) of cells where the slices of the NSAG are supported or not supported. In the UE, NAS provides the NSAG(s) and their priorities to be considered during cell reselection. The mapping relationship between slices and NSAG(s) are provided to UE via NAS signalling as specified in TS 23.501[5].

{…Omitted…}

End of change