3GPP TSG RAN WG2 Meeting #119-electronic               	R2-2207832

# Online meeting, 17th - 26th August 2022

Agenda Item: 	8.5.3

Source:		Sony

Title:			Considerations on XR specific C-DRX power saving enhancements

Document for:	Discussion & Decision

# Introduction

A new Rel-18 study item was approved at RANP#94 in December 2021, including the following objectives. . The study is to be based on RAN1, Rel-17 TR 38.838 [2], SA4 work in SP-210043 [3], and SA2 work as per SP-220705 [4]..

The study is to be based on Rel-17 TR 38.838, on corresponding Rel-17 work from SA4 (as per SP-210043) and on Rel-18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; Dynamic scheduling/grant enhancements

At RAN1#109e meeting, the following agreements related to XR power saving were made:

For power saving study of Rel-18 XR SI, CDRX enhancements to evaluate in this study item are to be selected from the following: - High priority Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity for each flow - High priority Issue 1-2: C-DRX enhancements to handle jitter - Medium priority Issue 1-3: CDRX enhancements for multiple XR traffic flows [Note 2] - Low priority Issue 1-4: CDRX enhancements to adjust to variable burst sizes and frame rate Note: Some companies think the adjustment for variable burst sizes can be realized by existing spec already - Low priority Issue 1-5: low latency handling - Low priority Issue 1-6: SFN wraparound mismatch (if handled in RAN1) For power saving study of Rel-18 XR SI, PDCCH monitoring enhancements to evaluate in this study item are to be selected from the following - Low priority Issue 2-1: Alignment between PDCCH monitoring and XR traffic to resolve the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. - Low priority Issue 2-2: XR-dedicated PDCCH monitoring window to supplement CDRX for multi-flow traffic. - High priority Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation.

RAN2 has not discussed any Rel-18 topics yet, and the upcoming RAN2#119e meeting is the first one to discuss amongst other this topic.

In this contribution, we discuss details of our proposed power saving techniques to enhance the C-DRX for XR traffic, following RAN1 prioritization order in the list above.

# Discussions on C-DRX enhancements for XR

The existing C-DRX configuration including enhanced C-DRX (eC-DRX) of Rel-17 are not optimized for XR type traffics. This is due to the following characteristics of XR type traffics

- variable XR data rate,

- non-integer XR traffic periodicity and

- quasi-periodic XR periodicity (with possible jitter).

The proposed C-DRX enhancement below include adaptation of C-DRX operation based on XR traffic characteristics, described earlier, to reduce the total on duration of the device.

## Adaptation of C-DRX configuration to handle non-integer XR traffic

As listed above, one of the main characteristics of XR traffic is its periodic but non-integer arrival time. In order to save UE power consumption, ideally, the UE should be configured with the DRX configuration in which the DRX ON duration is aligned with XR packet arrival timing. With the existing CDRX configuration, the DRX configuration may not be aligned with the packet arrival time, due to the non-integer arrival rate of XR applications. This causes misalignment as the downlink packet may arrive prior or after the scheduled DRX ON duration. This misalignment increases the packet transfer delay and has negative impact on delivering packets within a given delay bound. An example of this is shown in Figure 1. In this example, XR traffic has packet frame arrival of 60 fps which gives a non-integer arrival rate of 16.67 msec. CDRX cycle values support only integer multiples of 1 msec and here it is configured to 16 msec.

Figure 1 - Illustration of misalignment between XR traffic arrival time and existing DRX configuration

As we can see, DRX ON duration cannot be exactly aligned with DL frame arrival timing. This would mean that the packets would arrive further and further outside the DRX active window or ON duration after the first two transmission, assuming we have a 1 msec DRX ON duration window. Extending the DRX ON duration may ensure the reception of the packets at the UE. However, it will increase the power consumption. All above means when deciding on enhanced C-DRX configuration we need to also take the XR non-integer periodicity into account such that the starting of the ON duration is aligned with periodicity of XR traffic and its duration is matched with the packet arrival time.

Observation 1: UE power saving can be optimally obtained by arranging the DRX configuration where the starting of the ON duration is aligned with periodicity of XR traffic and its duration is matched with the packet arrival time.

To address this, we propose a solution where the DRX cycles are arranged so that the UE power consumption is reduced while ensuring the reception of the packet arrival at the UE side. In our proposed scheme, whenever the packet arrival is occurred outside the ON duration of a DRX, depending on whether the length of DRX cycle is longer or shorter than the packet arrival time, the start of ON duration is delayed or advanced, respectively by a certain period, Δ msec.  As shown in Figure 2, we decide whether it is needed to delay or advance the start of the ON duration based on the value of Toffset,i+1. At each upcoming packet arrival, the expected packet arrival time are compared to the expected/configured ON duration, by assessing the value of Toffset,i+1. Whenever the expected arrival time is earlier than the start of the next ON duration, i.e., Toffset,i+1<0, the start of the subsequent ON duration needs to be advanced by Δ msec and if the expected arrival time is after the next ON duration, i.e., Toffset,i+1>Ton, the start of subsequent ON duration needs to be delayed by Δ msec.

Figure 2 - Timing diagram for calculation and evaluation of the start of the DRX ON duration

Proposal 1: Consider enabling dynamic DRX parameters of the pre-assigned DRX configuration, such as advancing/delaying the start of DRX ON duration.

The details on signaling for configuration and enabling of the dynamic usage of the DRX parameters should be further discussed.

Proposal 2: Study the details on signaling for configuration and enabling of the dynamic usage of the DRX parameters.

## C-DRX configuration adaptation to handle jitter

Another characteristic of XR traffic is that the packet arrival rate is impacted by potential jitter that comes from generating and transmitting the video frames. These types of traffic might not be perfectly periodic which can be characterized as jitter. Jitter basically means the arrival of video frames becomes less predictable. This makes the arrival rate not fully deterministic, and some flexibility would be needed to monitor DL channel. Therefore, when enhancing the C-DRX configuration for XR type traffic, quasi-periodic XR characteristics needs to also be taken into account. By configuring the DRX to a short DRX ON period, the packet may arrive outside the ON-duration and the packet cannot be received until the subsequent ON duration. This increases the delay. A long DRX ON period can avoid delay, but the drawback is increased UE power consumption. It also does not solve the issue if the packet arrived earlier than the ON duration.

Observation 2: Adjusting only the length of the ON duration can result either in a long delay or unnecessary increased power consumption.

We address jitter problem by further improving the dynamic DRX parameters of the previous section. In our proposed scheme, the gNB performs measurement on the jitter itself, i.e., jitter deviation, and do an estimation on, e.g., a max jitter and then signals that to the UE. The UE then adjusts the ON duration accordingly, i.e., if the packet due to jitter arrives earlier than the ON duration, calculated based on above techniques, we need to advance the DRX ON duration, and if the packet arrives after the ON duration, then the DRX ON duration needs to be delayed, similar to what is described in the approach above.

Proposal 3 – Consider dynamic DRX configuration to delay or advance DRX ON duration based on the experienced jitter.

## Two-level C-DRX configuration

As stated earlier, the eC-DRX mechanisms mentioned in Rel-17 are not optimized for XR traffics since adaptation of DRX ON duration and multi-C-DRX cycle values configuration are almost based on single C-DRX structure. Another approach to improve energy savings is to reduce UE power consumption during DRX ON. Aiming at this direction, a two-level C-DRX configuration scheme, shown in  Figure 3, with extra finer granularity of inactive duration indication is proposed.

Figure 3 – Two-level C-DRX configuration

In this scheme, the 1st level DRX can be configured/adjusted with existing DRX schemes, such as legacy in Rel-15/16 and eC-DRX mechanisms in Rel-17. For the 2nd level DRX configuration, since scheduling DCI for dynamic grant will be sent frequently in the 1st level DRX ON, which inspired us to indicate minor/finer inactive duration by DCI in 1st level DRX ON for extra sleep, and additional/reserved bits or existing field can be used/repurposed for this related indication with either absolute value or index. Through two-level DRX configuration, extra inactive duration in 1st level DRX ON is introduced for further energy saving with less specification impact.

Proposal 4 – Two-level DRX configuration with finer granularity inactive indication during DRX ON can be considered for further power saving.

## Application aware multi-C-DRX configuration

Another important characteristic of the XR Traffic is it may have one or more data streams in which each stream may have its own periodicity. Additionally, each stream may have a fix or a varying data-rate. The current C-DRX parameters and configuration are for regular data traffics and therefore not optimized for XR applications with multi-stream and quasi-periodic traffic periodicity. For instance, if there is no transmission after an activity timer then the OFF duration starts. However, if the UE continues receiving data the ON duration gets continuously extended by restarting the inactivity timer, which is normal and expected legacy behavior, but if the data arrival is known, the On-duration period could be reduced, and unnecessarily activating of the inactivity timer could be minimized or prevented, and hence avoiding unnecessary extra power consumption at the UE.

Observation 2 -The existing C-DRX configuration is not optimized for XR traffic characteristics, resulting in unnecessary activation of inactivity timer and consequently leading to increased power consumption at the UEs.

We can use the unique “deterministic” characteristic of XR traffic and introduce an application aware multi-C-DRX configuration to reduce the unnecessary on duration.  In this approach, as illustrated in  Figure 4, each stream is associated with a certain DRX configuration(s) and the UE can adaptively switch-off the receiver for a certain period when the UE does not expect to receive the downlink data.

Figure 4 - Illustration of multi DRXs operation

Proposal 5 - Application aware multi-C-DRX configuration can be considered as an enhanced C-DRX mechanism to further reduce power consumption in UEs with XR traffic.

# Summary

In this contribution, we have discussed our view on potential XR power saving techniques  in NR. Our observation and proposals are listed below:

Observation 1: UE power saving can be optimally obtained by arranging the DRX configuration where the starting of the ON duration is aligned with periodicity of XR traffic and its duration is matched with the packet arrival time.

Proposal 1: Consider enabling dynamic DRX parameters of the pre-assigned DRX configuration, such as the start of DRX ON duration can be advancing / delaying.

Observation 2: Adjusting only the length of the ON duration can result either in a long delay or unnecessary increased power consumption.

Proposal 3 – Consider dynamic DRX configuration to delay or advance DRX ON duration based on the experienced jitter.

Proposal 4 – Two-level DRX configuration with finer granularity inactive indication during DRX ON can be considered for further power saving.

Observation 2 -The existing C-DRX configuration is not optimized for XR traffic characteristics, resulting in unnecessary activation of inactivity timer and consequently leading to increased power consumption at the UEs.

Proposal 5 - Application aware multi-C-DRX configuration can be considered as an enhanced C-DRX mechanism to further reduce power consumption in UEs with XR traffic.

# References

- RP-213587, “Study on XR Enhancements for NR””, RANP #94e, December 2021

- TR38.838,

- SP-210043, SA4

- SP-220705, SA2