3GPP TSG RAN WG2 #119-e	R2-2207841

e-Meeting, August 17th – 29th, 2022

Agenda Item:	8.6.2

Source:	ZTE Corporation, Sanechips

Title:	Consideration on HARQ and GNSS enhancements

Document for:	Discussion and Decision

Introduction

As mentioned in WID RP-221806 [1], for IoT-NTN Performance Enhancements in Rel-18, some remaining issues from Rel-17 will continue to be discussed in Rel-18. Rel-18 IoT NTN considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome. The related objectives are listed below:

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2]

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1]

- NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

In this paper, we will have some initial discussion on the disabling of HARQ feedback and improved GNSS operations.

Discussion

Disabling of HARQ feedback

For R17 NR NTN, disabling on HARQ feedback per HARQ process has been supported. But for R17 IoT NTN, due to the limited time, disabling on HARQ feedback is not supported. For R18 IoT NTN, RAN1 has already had some discussion in RAN1#109[2] and confirmed the necessity of disabling on HARQ feedback in IoT NTN to mitigate the throughput loss caused by HARQ stalling. They also have some discussion on how to configure/indicate the enabling/disabling of HARQ feedback. The related agreements are as below:

Agreement For IoT NTN, to configure/indicate enabling/disabling on HARQ feedback for downlink transmission, one or more of the following options can be considered: - Option 1: per HARQ process via UE specific RRC signaling - Option 2: per HARQ process via SIB signaling - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field) - Option 4: implicitly determined by existing configured/indicated parameter(s) (e.g., repetition number, TBS) - Option 5: per HARQ process via MAC CE - Other options or combinations are not excluded Note: Option(s) for eMTC and NBIoT can be separately discussed.

For IoT NTN, on the one hand, similarly as NR NTN, the disablement of HARQ feedback is mainly beneficial for GEO scenario which has large RTT or for the service that data arrives frequently. So, it should be configured in some cases.

On the other hand, different from NR NTN, due to the possible long duration of PDCCH and PDSCH transmission caused by DL/UL repetitions, especially large repetitions in the bad coverage cases, the performance improvement by disabling HARQ feedback would not be as obvious as that in NR NTN. Considering in IoT the maximum repetition numbers is configured by RRC message, the semi-static configuration for disabling on HARQ feedback is suitable.

In final, we prefer Option1, because it can facilitate all the possible configuration in different scenarios.

For R18 IoT NTN, disabling on HARQ feedback is configured per HARQ process via UE specific RRC signaling.

As mentioned above, for R17 IoT NTN, no HARQ feedback disabling is supported, only UE-eNB RTT (RTToffset) has been added to all the HARQ RTT timers and UL HARQ RTT timers.

In R17 NR NTN, three cases are supported for HARQ-RTT-TimerDL with combined considerations on HARQ feedback disabling and UE-eNB RTT:

- DL_Case 1: if downlinkHARQ-FeedbackDisabled is configured and DL HARQ feedback of a HARQ process is enabled, for example, eNB could schedule retransmission based on HARQ feedback, UE-eNB RTT is added to HARQ-RTT-TimerDL-NTN;

- DL_Case 2: if downlinkHARQ-FeedbackDisabled is configured and DL HARQ feedback of a HARQ process is disabled, for example, eNB could determine no retransmission or blind retransmission without HARQ feedback, HARQ-RTT-TimerDL-NTN is not started.

- DL_Case 3: if downlinkHARQ-FeedbackDisabled isn’t configured, it is legacy as TN, for example, eNB could schedule retransmission based on HARQ feedback in LEO, drx-HARQ-RTT-TimerDL is started.

It can be considered that the above DL_Case 1 has already been supported in R17 IoT NTN as it is same as the existing process, e.g., HARQ feedback is enabled and UE-eNB RTT is added in HARQ RTT timer. For DL_case 2, it can save UE power consumption if HARQ RTT timer is not started. For DL_case 3, it takes legacy process as TN under LEO scenario. Since RAN1 has determined the necessity of supporting HARQ feedback disabling, we see the straightforward way is to adapt the process of, HARQ-RTT-TimerDL in R17 NR NTN, for the HARQ RTT timers in IoT NTN, especially the process in the DL_Case 2 and DL_Case 3.

For R18 IoT NTN, the process of HARQ-RTT-TimerDL based on whether downlinkHARQ-FeedbackDisabled is configured and whether DL HARQ feedback is disabled in R17 NR NTN could be adapted for HARQ RTT timer in IoT NTN.

In R17 NR NTN, three cases are also supported for HARQ-RTT-TimerUL with combined considerations on HARQ feedback disabling and UE-eNB RTT:

- UL_Case 1: if uplinkHARQ-Mode is configured and HARQModeA is configured, for example, eNB could schedule retransmission based on UL decoded result, UE-eNB RTT is added to HARQ-RTT-TimerUL-NTN;

- UL_Case 2: if uplinkHARQ-Mode is configured and HARQModeB is configured, for example, eNB could determine no retransmission or blind retransmission without UL decoded result, HARQ-RTT-TimerUL-NTN is not started.

- UL_Case 3: if uplinkHARQ-Mode isn’t configured, it is as legacy TN, for example, eNB could schedule retransmission based on HARQ feedback in LEO, drx-HARQ-RTT-TimerUL is started.

For 18 IoT NTN, similarly as DL, it’s also suggested to adapt the process of HARQ-RTT-TimerUL in R17 NR NTN for the UL HARQ RTT timers in IoT NTN, especially the process in the UL_Case 2 and UL_Case 3.

For R18 IoT NTN, the process of HARQ-RTT-TimerUL in R17 NR NTN could be adapted for UL HARQ RTT timer in IoT NTN.

Improved GNSS operations

For R17 IoT NTN, acquisition of the GNSS position during the short transmission has been specified, that is, UE enters into idle mode when GNSS becomes invalid. For R18 IoT NTN, acquisition of the GNSS position during the long transmission needs to be studied. It’s expected that, in connected mode, when GNSS is invalid, UE could require GNSS and then continue the service transmission. In RAN1#109[2], this issue has been discussed and the related agreements are as below:

Agreement At least the following options can be considered on GNSS measurement in connected for potential enhancements for improved GNSS operations: - Option 1: UE re-acquires GNSS position fix during RLF procedure - Option 2: UE re-acquires GNSS position fix with a new gap Note: this does not imply that a Rel-18 IoT NTN UE is mandated to support one or both of the options. Agreement UE reports additional GNSS assistance information and further study the detailed GNSS assistance information, including e.g. GNSS position fix measurement time - Note: Since RAN1 agreed that GNSS validity duration is reported by UE in Rel-17, it is already included in GNSS assistance information. Agreement Further study on whether there is a need for potential enhancements on the following for long connection time - UE triggered GNSS measurement. - Network triggered GNSS measurement.

From RAN2 perspective, for Option 1, when GNSS is invalid, UE triggers a RRC reestablishment and re-acquires GNSS position fix during the RRC reestablishment. For Option 2, when GNSS is invalid, UE re-acquires GNSS position fix during a gap that is not scheduled for data transmission. We understand that Option 2 needs less time to recover link and continue the transmission than that in Option 1. So, we prefer Option2.

For Option 2, UE re-acquires GNSS during a GNSS position fix measurement time. And the length of GNSS position fix measurement time is determined by the conditions for GNSS position fix. For example, for hot start, the time is 1s or 2s, for warm start, the time is several seconds, and for cold start the time is 30s [3].

Considering simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed, UE couldn’t monitor the downlink radio link quality during GNSS position fix measurement time. The missing samples in RLM during GNSS position fix measurement time may trigger out-of-sync indication from PHY to RRC, and even finally trigger a RLF, especially for cell edge UE. To be specific, if the evaluation period for out-of-sync indication is less than the GNSS position fix measurement time, when UE re-acquires GNSS, at least one out-of-sync indication would be initiated. For example, for eMTC UE with DRX cycle 0.04s, if it needs 2s of GNSS position fix measurement time, the evaluation period (e.g. 0.8s) is less than GNSS position fix measurement time. When UE re-acquires GNSS, many out-of-sync indications would be initiated, and it’s highly possible that RLF would be triggered when UE re-acquires GNSS.

In conclusion, in order to avoid the RLF after UE re-acquires GNSS, if the evaluation period is more than the GNSS position fix measurement time, UE could re-acquire GNSS position fix with GNSS position fix measurement time.

For R18 IoT NTN, we suggest that if the evaluation period is more than the GNSS position fix measurement time, UE could re-acquire GNSS position fix with GNSS position fix measurement time.

Conclusion

In previous sections, the following observations and proposals were made:

Proposal 1 For R18 IoT NTN, disabling on HARQ feedback is configured per HARQ process via UE specific RRC signaling.

Proposal 2 For R18 IoT NTN, the process of HARQ-RTT-TimerDL based on whether downlinkHARQ-FeedbackDisabled is configured and whether DL HARQ feedback is disabled in R17 NR NTN could be adapted for HARQ RTT timer in IoT NTN.

Proposal 3 For R18 IoT NTN, the process of HARQ-RTT-TimerUL in R17 NR NTN could be adapted for UL HARQ RTT timer in IoT NTN.

Proposal 4 For R18 IoT NTN, we suggest that if the evaluation period is more than the GNSS position fix measurement time, UE could re-acquire GNSS position fix with GNSS position fix measurement time.

References

- RP-221806 Revised WID on IoT NTN enhancements MediaTek Inc.

- RAN1#109 chairman notes

- 38.821 Solutions for NR to support non-terrestrial networks (NTN) (Release 16)