3GPP TSG-RAN2 Meeting #119e	R2-2207873

# Electronic, 17th – 29th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3320 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for SRS-PeriodicityAndOffset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2022-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN1#99 has agreed that the periodicities of the SRS for positioning support the union of periodicities (and associated slot offsets) supported for NR SRS Rel-15 and the NR DL-PRS Rel-16: Agreement: The union of periodicities (and associated slot offsets) supported for NR SRS Rel-15 and the NR DL-PRS Rel-16 is supported for SRS for positioning in Rel-16. And RAN1#99 has agreed the following periodicity values are supported depending on SCS for NR DL-PRS Rel-16: Agreement: The following periodicity values of DL PRS resource allocation are supported depending on SCS - 2μ{4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots, µ = 0, 1, 2, 3 for SCS 15, 30, 60 and 120kHz respectively However, the relationship between SCS and the periodicity is not mentioned in the current description in 38.331. And the periodicity for pos-SRS with 128, 256, 512, and 20480 slots is missing in the current RRC spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the periodicity values of SRS for positioning are supported depending on SCS, and add the value sl128, sl256, sl512, and sl20480 in the SRS-PeriodicityAndOffset-r16, and add the capability srs-ExtendedPeriodictyAndOffset-v16xy in the UE capability information elements to ensure backward compatibility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The periodicity of SRS for positioning is not aligned with the RAN1 agreement. Impact analysis Impacted functionality: UL-TDOA positioning, and UL-AOA positioning Inter-operability: If the UE is implemented according to the CR while the network is not, there is no inter-operability issue. If the network is implemented according to the CR while the UE is not, the UE may not be able to correctly transmit SRS resource for positioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.306 CR R2-2207875 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Ver0 in RAN2#119e: R2-2207873 |  |  |  |  |  |  |  |  |

=========================================================CHANGE BEGINS=======================================================

6.3.2	Radio resource control information elements

–	SRS-Config

The IE SRS-Config is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources, a list of SRS-PosResources, a list of SRS-PosResourceSets and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources or SRS-PosResources. The network triggers the transmission of the set of SRS-Resources or SRS-PosResources using a configured aperiodicSRS-ResourceTrigger (L1 DCI).

SRS-Config information element

-- ASN1START

-- TAG-SRS-CONFIG-START

SRS-Config ::=                          SEQUENCE {

srs-ResourceSetToReleaseList            SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId                OPTIONAL,   -- Need N

srs-ResourceSetToAddModList             SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet                  OPTIONAL,   -- Need N

srs-ResourceToReleaseList               SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId                      OPTIONAL,   -- Need N

srs-ResourceToAddModList                SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource                        OPTIONAL,   -- Need N

tpc-Accumulation                        ENUMERATED {disabled}                                                           OPTIONAL,   -- Need S

...,

[[

srs-RequestDCI-1-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-RequestDCI-0-2-r16                  INTEGER (1..2)                                                          OPTIONAL, -- Need S

srs-ResourceSetToAddModListDCI-0-2-r16  SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet          OPTIONAL, -- Need N

srs-ResourceSetToReleaseListDCI-0-2-r16 SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId        OPTIONAL, -- Need N

srs-PosResourceSetToReleaseList-r16     SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16

OPTIONAL, -- Need N

srs-PosResourceSetToAddModList-r16      SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16        OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r16        SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16            OPTIONAL,-- Need N

srs-PosResourceToAddModList-r16         SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16              OPTIONAL -- Need N

]]

}

SRS-ResourceSet ::=                     SEQUENCE {

srs-ResourceSetId                       SRS-ResourceSetId,

srs-ResourceIdList                      SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId    OPTIONAL, -- Cond Setup

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

aperiodicSRS-ResourceTrigger            INTEGER (1..maxNrofSRS-TriggerStates-1),

csi-RS                                  NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

slotOffset                              INTEGER (1..32)                                        OPTIONAL, -- Need S

...,

[[

aperiodicSRS-ResourceTriggerList            SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL  -- Need M

]]

},

semi-persistent                         SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

},

periodic                                SEQUENCE {

associatedCSI-RS                        NZP-CSI-RS-ResourceId                                  OPTIONAL, -- Cond NonCodebook

...

}

},

usage                                   ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching},

alpha                                   Alpha                                                          OPTIONAL, -- Need S

p0                                      INTEGER (-202..24)                                             OPTIONAL, -- Cond Setup

pathlossReferenceRS                     PathlossReferenceRS-Config                                     OPTIONAL, -- Need M

srs-PowerControlAdjustmentStates        ENUMERATED { sameAsFci2, separateClosedLoop}                   OPTIONAL, -- Need S

...,

[[

pathlossReferenceRSList-r16             SetupRelease { PathlossReferenceRSList-r16}                    OPTIONAL  -- Need M

]]

}

PathlossReferenceRS-Config ::=              CHOICE {

ssb-Index                                   SSB-Index,

csi-RS-Index                                NZP-CSI-RS-ResourceId

}

PathlossReferenceRSList-r16 ::=             SEQUENCE (SIZE (1..maxNrofSRS-PathlossReferenceRS-r16)) OF PathlossReferenceRS-r16

PathlossReferenceRS-r16 ::=                 SEQUENCE {

srs-PathlossReferenceRS-Id-r16              SRS-PathlossReferenceRS-Id-r16,

pathlossReferenceRS-r16                     PathlossReferenceRS-Config

}

SRS-PathlossReferenceRS-Id-r16 ::=          INTEGER (0..maxNrofSRS-PathlossReferenceRS-1-r16)

SRS-PosResourceSet-r16 ::=                  SEQUENCE {

srs-PosResourceSetId-r16                    SRS-PosResourceSetId-r16,

srs-PosResourceIdList-r16                   SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-PosResourceId-r16

OPTIONAL, -- Cond Setup

resourceType-r16                            CHOICE {

aperiodic-r16                               SEQUENCE {

aperiodicSRS-ResourceTriggerList-r16        SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-1))

OF INTEGER (1..maxNrofSRS-TriggerStates-1)     OPTIONAL, -- Need M

...

},

semi-persistent-r16                         SEQUENCE {

...

},

periodic-r16                                SEQUENCE {

...

}

},

alpha-r16                                   Alpha                                                      OPTIONAL, -- Need S

p0-r16                                      INTEGER (-202..24)                                         OPTIONAL, -- Cond Setup

pathlossReferenceRS-Pos-r16                 CHOICE {

ssb-IndexServing-r16                        SSB-Index,

ssb-Ncell-r16                               SSB-InfoNcell-r16,

dl-PRS-r16                                  DL-PRS-Info-r16

}                                                                                                      OPTIONAL, -- Need M

...

}

SRS-ResourceSetId ::=                   INTEGER (0..maxNrofSRS-ResourceSets-1)

SRS-PosResourceSetId-r16 ::=            INTEGER (0..maxNrofSRS-PosResourceSets-1-r16)

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }                                           OPTIONAL,   -- Need R

transmissionComb                        CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

}

},

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                        OPTIONAL,   -- Need R

...,

[[

resourceMapping-r16                     SEQUENCE {

startPosition-r16                       INTEGER (0..13),

nrofSymbols-r16                         ENUMERATED {n1, n2, n4},

repetitionFactor-r16                    ENUMERATED {n1, n2, n4}

}                                                                                                      OPTIONAL    -- Need R

]]

}

SRS-PosResource-r16::=                  SEQUENCE {

srs-PosResourceId-r16                   SRS-PosResourceId-r16,

transmissionComb-r16                    CHOICE {

n2-r16                                  SEQUENCE {

combOffset-n2-r16                       INTEGER (0..1),

cyclicShift-n2-r16                      INTEGER (0..7)

},

n4-r16                                  SEQUENCE {

combOffset-n4-r16                        INTEGER (0..3),

cyclicShift-n4-r16                      INTEGER (0..11)

},

n8-r16                                  SEQUENCE {

combOffset-n8-r16                       INTEGER (0..7),

cyclicShift-n8-r16                      INTEGER (0..5)

},

...

},

resourceMapping-r16                       SEQUENCE {

startPosition-r16                           INTEGER (0..13),

nrofSymbols-r16                             ENUMERATED {n1, n2, n4, n8, n12}

},

freqDomainShift-r16                       INTEGER (0..268),

freqHopping-r16                           SEQUENCE {

c-SRS-r16                                 INTEGER (0..63),

...

},

groupOrSequenceHopping-r16                ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType-r16                          CHOICE {

aperiodic-r16                             SEQUENCE {

slotOffset-r16                            INTEGER (1..32)                                      OPTIONAL,   -- Need S

...

},

semi-persistent-r16                       SEQUENCE {

periodicityAndOffset-sp-r16               SRS-PeriodicityAndOffset-r16,

... ,

[[

periodicityAndOffset-sp-Ext-r16             SRS-PeriodicityAndOffsetExt-r16                     OPTIONAL   -- Need R

]]

},

periodic-r16                              SEQUENCE {

periodicityAndOffset-p-r16                SRS-PeriodicityAndOffset-r16,

... ,

[[

periodicityAndOffset-p-Ext-r16             SRS-PeriodicityAndOffsetExt-r16                     OPTIONAL   -- Need R

]]

}

},

sequenceId-r16                            INTEGER (0..65535),

spatialRelationInfoPos-r16                SRS-SpatialRelationInfoPos-r16                               OPTIONAL,   -- Need R

...

}

SRS-SpatialRelationInfo ::=     SEQUENCE {

servingCellId                       ServCellIndex                                                      OPTIONAL,   -- Need S

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId,

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

SRS-SpatialRelationInfoPos-r16 ::=      CHOICE {

servingRS-r16                           SEQUENCE {

servingCellId                           ServCellIndex                                              OPTIONAL,   -- Need S

referenceSignal-r16                     CHOICE {

ssb-IndexServing-r16                    SSB-Index,

csi-RS-IndexServing-r16                 NZP-CSI-RS-ResourceId,

srs-SpatialRelation-r16                 SEQUENCE {

resourceSelection-r16                   CHOICE {

srs-ResourceId-r16                      SRS-ResourceId,

srs-PosResourceId-r16                   SRS-PosResourceId-r16

},

uplinkBWP-r16                           BWP-Id

}

}

},

ssb-Ncell-r16                           SSB-InfoNcell-r16,

dl-PRS-r16                              DL-PRS-Info-r16

}

SSB-Configuration-r16  ::=          SEQUENCE {

ssb-Freq-r16                     ARFCN-ValueNR,

halfFrameIndex-r16                  ENUMERATED {zero, one},

ssbSubcarrierSpacing-r16            SubcarrierSpacing,

ssb-Periodicity-r16                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2,spare1 }   OPTIONAL, -- Need S

sfn0-Offset-r16                     SEQUENCE {

sfn-Offset-r16                      INTEGER (0..1023),

integerSubframeOffset-r16           INTEGER (0..9)                                                 OPTIONAL  -- Need R

}                                                                                                      OPTIONAL, -- Need R

sfn-SSB-Offset-r16                  INTEGER (0..15),

ss-PBCH-BlockPower-r16              INTEGER (-60..50)                                                  OPTIONAL  -- Cond Pathloss

}

SSB-InfoNcell-r16  ::=              SEQUENCE {

physicalCellId-r16                  PhysCellId,

ssb-IndexNcell-r16                  SSB-Index                                                          OPTIONAL, -- Need S

ssb-Configuration-r16               SSB-Configuration-r16                                              OPTIONAL  -- Need S

}

DL-PRS-Info-r16  ::=                SEQUENCE {

dl-PRS-ID-r16                      INTEGER (0..255),

dl-PRS-ResourceSetId-r16           INTEGER (0..7),

dl-PRS-ResourceId-r16              INTEGER (0..63)                                                     OPTIONAL  -- Need S

}

SRS-ResourceId ::=                      INTEGER (0..maxNrofSRS-Resources-1)

SRS-PosResourceId-r16 ::=               INTEGER (0..maxNrofSRS-PosResources-1-r16)

SRS-PeriodicityAndOffset ::=            CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559)

}

SRS-PeriodicityAndOffset-r16 ::=        CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559),

sl5120                                  INTEGER(0..5119),

sl10240                                 INTEGER(0..10239),

sl40960                                 INTEGER(0..40959),

sl81920                                 INTEGER(0..81919),

...

}

SRS-PeriodicityAndOffsetExt-r16 ::=        CHOICE {

sl128									INTEGER(0..127),

sl256									INTEGER(0..255),

sl512									INTEGER(0..511),

sl20480									INTEGER(0..20479)

}

-- TAG-SRS-CONFIG-STOP

-- ASN1STOP

SRS-Config field descriptions

tpc-Accumulation If the field is absent, UE applies TPC commands via accumulation. If disabled, UE applies the TPC command without accumulation (this applies to SRS when a separate closed loop is configured for SRS) (see TS 38.213 [13], clause 7.3).

SRS-Resource, SRS-PosResource field descriptions

cyclicShift-n2 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

cyclicShift-n4 Cyclic shift configuration (see TS 38.214 [19], clause 6.2.1).

freqHopping Includes parameters capturing SRS frequency hopping (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, the network always configures this field such that b-hop > b-SRS.

groupOrSequenceHopping Parameter(s) for configuring group or sequence hopping (see TS 38.211 [16], clause 6.4.1.4.2). For CLI SRS-RSRP measurement, the network always configures this parameter to 'neither'.

nrofSRS-Ports Number of ports. For CLI SRS-RSRP measurement, the network always configures this parameter to 'port1'.

periodicityAndOffset-p, periodicityAndOffset-p-Ext Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, sl1280 and sl2560 cannot be configured. For SRS-PosResource, sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, sl40960 and sl81920 cannot be configured for SCS=30kHz, and sl81920 cannot be configured for SCS=60kHz. When the field periodicityAndOffset-p-Ext is present, the field periodicityAndOffset-p shall be ignored by the UE. When the field periodicityAndOffset-p is present, the field periodicityAndOffset-p-Ext shall be absent.

periodicityAndOffset-sp, periodicityAndOffset-sp-Ext Periodicity and slot offset for this SRS resource. All values are in "number of slots". Value sl1 corresponds to a periodicity of 1 slot, value sl2 corresponds to a periodicity of 2 slots, and so on. For each periodicity the corresponding offset is given in number of slots. For periodicity sl1 the offset is 0 slots (see TS 38.214 [19], clause 6.2.1). For SRS-PosResource, sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, sl40960 and sl81920 cannot be configured for SCS=30kHz, and sl81920 cannot be configured for SCS=60kHz. When the field periodicityAndOffset-sp-Ext is present, the field periodicityAndOffset-sp shall be ignored by the UE. When the field periodicityAndOffset-sp is present, the field periodicityAndOffset-sp-Ext shall be absent.

ptrs-PortIndex The PTRS port index for this SRS resource for non-codebook based UL MIMO. This is only applicable when the corresponding PTRS-UplinkConfig is set to CP-OFDM. The ptrs-PortIndex configured here must be smaller than the maxNrofPorts configured in the PTRS-UplinkConfig (see TS 38.214 [19], clause 6.2.3.1). This parameter is not applicable to CLI SRS-RSRP measurement.

resourceMapping OFDM symbol location of the SRS resource within a slot including nrofSymbols (number of OFDM symbols), startPosition (value 0 refers to the last symbol, value 1 refers to the second last symbol, and so on) and repetitionFactor (see TS 38.214 [19], clause 6.2.1 and TS 38.211 [16], clause 6.4.1.4). The configured SRS resource does not exceed the slot boundary. If resourceMapping-r16 is signalled, UE shall ignore the resourceMapping (without suffix). For CLI SRS-RSRP measurement, the network always configures nrofSymbols and repetitionFactor to 'n1'.

resourceType Periodicity and offset for semi-persistent and periodic SRS resource (see TS 38.214 [19], clause 6.2.1). For CLI SRS-RSRP measurement, only 'periodic' is applicable for resourceType.

sequenceId Sequence ID used to initialize pseudo random group and sequence hopping (see TS 38.214 [19], clause 6.2.1).

servingCellId The serving Cell ID of the source SSB, CSI-RS, or SRS for the spatial relation of the target SRS resource. If this field is absent the SSB, the CSI-RS, or the SRS is from the same serving cell where the SRS is configured.

spatialRelationInfo Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS (see TS 38.214 [19], clause 6.2.1). This parameter is not applicable to CLI SRS-RSRP measurement.

spatialRelationInfoPos Configuration of the spatial relation between a reference RS and the target SRS. Reference RS can be SSB/CSI-RS/SRS/DL-PRS (see TS 38.214 [19], clause 6.2.1). If the IE srs-ResourceId-Ext is present, the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 63. Otherwise the IE srs-ResourceId in spatialRelationInfoPos represents the index from 0 to 31.

srs-RequestDCI-0-2 Indicate the number of bits for "SRS request"in DCI format 0_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 0_2 is applied. If the parameter srs-RequestDCI-0-2 is configured to value 1, 1 bit is used to indicate one of the first two rows of Table 7.3.1.1.2-24 in TS 38.212 [17] for triggered aperiodic SRS resource set. If the value 2 is configured, 2 bits are used to indicate one of the rows of Table 7.3.1.1.2-24 in TS 38.212 [17]. When UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication.

srs-RequestDCI-1-2 Indicate the number of bits for "SRS request" in DCI format 1_2. When the field is absent, then the value of 0 bit for "SRS request" in DCI format 1_2 is applied. When the UE is configured with supplementaryUplink, an extra bit (the first bit of the SRS request field) is used for the non-SUL/SUL indication (see TS 38.214 [19], clause 6.1.1.2).

srs-ResourceSetToAddModListDCI-0-2 List of SRS resource set to be added or modified for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

srs-ResourceSetToReleaseListDCI-0-2 List of SRS resource set to be released for DCI format 0_2 (see TS 38.212 [17], clause 7.3.1).

transmissionComb Comb value (2 or 4 or 8) and comb offset (0..combValue-1) (see TS 38.214 [19], clause 6.2.1).

============================================================NEXT CHANGE=====================================================

6.3.3	UE capability information elements

#### –	Phy-Parameters

The IE Phy-Parameters is used to convey the physical layer capabilities.

Phy-Parameters information element

-- ASN1START

-- TAG-PHY-PARAMETERS-START

Phy-Parameters ::=                  SEQUENCE {

phy-ParametersCommon                Phy-ParametersCommon                        OPTIONAL,

phy-ParametersXDD-Diff              Phy-ParametersXDD-Diff                      OPTIONAL,

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                      OPTIONAL,

phy-ParametersFR1                   Phy-ParametersFR1                           OPTIONAL,

phy-ParametersFR2                   Phy-ParametersFR2                           OPTIONAL

}

Phy-ParametersCommon ::=            SEQUENCE {

csi-RS-CFRA-ForHO                   ENUMERATED {supported}                      OPTIONAL,

dynamicPRB-BundlingDL               ENUMERATED {supported}                      OPTIONAL,

sp-CSI-ReportPUCCH                  ENUMERATED {supported}                      OPTIONAL,

sp-CSI-ReportPUSCH                  ENUMERATED {supported}                      OPTIONAL,

nzp-CSI-RS-IntefMgmt                ENUMERATED {supported}                      OPTIONAL,

type2-SP-CSI-Feedback-LongPUCCH     ENUMERATED {supported}                      OPTIONAL,

precoderGranularityCORESET          ENUMERATED {supported}                      OPTIONAL,

dynamicHARQ-ACK-Codebook            ENUMERATED {supported}                      OPTIONAL,

semiStaticHARQ-ACK-Codebook         ENUMERATED {supported}                      OPTIONAL,

spatialBundlingHARQ-ACK             ENUMERATED {supported}                      OPTIONAL,

dynamicBetaOffsetInd-HARQ-ACK-CSI   ENUMERATED {supported}                      OPTIONAL,

pucch-Repetition-F1-3-4             ENUMERATED {supported}                      OPTIONAL,

ra-Type0-PUSCH                      ENUMERATED {supported}                      OPTIONAL,

dynamicSwitchRA-Type0-1-PDSCH       ENUMERATED {supported}                      OPTIONAL,

dynamicSwitchRA-Type0-1-PUSCH       ENUMERATED {supported}                      OPTIONAL,

pdsch-MappingTypeA                  ENUMERATED {supported}                      OPTIONAL,

pdsch-MappingTypeB                  ENUMERATED {supported}                      OPTIONAL,

interleavingVRB-ToPRB-PDSCH         ENUMERATED {supported}                      OPTIONAL,

interSlotFreqHopping-PUSCH          ENUMERATED {supported}                      OPTIONAL,

type1-PUSCH-RepetitionMultiSlots    ENUMERATED {supported}                      OPTIONAL,

type2-PUSCH-RepetitionMultiSlots    ENUMERATED {supported}                      OPTIONAL,

pusch-RepetitionMultiSlots          ENUMERATED {supported}                      OPTIONAL,

pdsch-RepetitionMultiSlots          ENUMERATED {supported}                      OPTIONAL,

downlinkSPS                         ENUMERATED {supported}                      OPTIONAL,

configuredUL-GrantType1             ENUMERATED {supported}                      OPTIONAL,

configuredUL-GrantType2             ENUMERATED {supported}                      OPTIONAL,

pre-EmptIndication-DL               ENUMERATED {supported}                      OPTIONAL,

cbg-TransIndication-DL              ENUMERATED {supported}                      OPTIONAL,

cbg-TransIndication-UL              ENUMERATED {supported}                      OPTIONAL,

cbg-FlushIndication-DL              ENUMERATED {supported}                      OPTIONAL,

dynamicHARQ-ACK-CodeB-CBG-Retx-DL   ENUMERATED {supported}                      OPTIONAL,

rateMatchingResrcSetSemi-Static     ENUMERATED {supported}                      OPTIONAL,

rateMatchingResrcSetDynamic         ENUMERATED {supported}                      OPTIONAL,

bwp-SwitchingDelay                  ENUMERATED {type1, type2}                   OPTIONAL,

...,

[[

dummy                               ENUMERATED {supported}                      OPTIONAL

]],

[[

maxNumberSearchSpaces               ENUMERATED {n10}                            OPTIONAL,

rateMatchingCtrlResrcSetDynamic     ENUMERATED {supported}                      OPTIONAL,

maxLayersMIMO-Indication            ENUMERATED {supported}                      OPTIONAL

]],

[[

spCellPlacement                             CarrierAggregationVariant           OPTIONAL

]],

[[

-- R1 9-1: Basic channel structure and procedure of 2-step RACH

twoStepRACH-r16                             ENUMERATED {supported}              OPTIONAL,

-- R1 11-1: Monitoring DCI format 1_2 and DCI format 0_2

dci-Format1-2And0-2-r16                     ENUMERATED {supported}              OPTIONAL,

-- R1 11-1a: Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space

monitoringDCI-SameSearchSpace-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 11-10: Type 2 configured grant release by DCI format 0_1

type2-CG-ReleaseDCI-0-1-r16                 ENUMERATED {supported}              OPTIONAL,

-- R1 11-11: Type 2 configured grant release by DCI format 0_2

type2-CG-ReleaseDCI-0-2-r16                 ENUMERATED {supported}              OPTIONAL,

-- R1 12-3: SPS release by DCI format 1_1

sps-ReleaseDCI-1-1-r16                      ENUMERATED {supported}              OPTIONAL,

-- R1 12-3a: SPS release by DCI format 1_2

sps-ReleaseDCI-1-2-r16                      ENUMERATED {supported}              OPTIONAL,

-- R1 14-8: CSI trigger states containing non-active BWP

csi-TriggerStateNon-ActiveBWP-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 20-2: Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities

separateSMTC-InterIAB-Support-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 20-3: Support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors

separateRACH-IAB-Support-r16                ENUMERATED {supported}              OPTIONAL,

-- R1 20-5a: Support semi-static configuration/indication of UL-Flexible-DL slot formats for IAB-MT resources

ul-flexibleDL-SlotFormatSemiStatic-IAB-r16  ENUMERATED {supported}              OPTIONAL,

-- R1 20-5b: Support dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources

ul-flexibleDL-SlotFormatDynamics-IAB-r16    ENUMERATED {supported}              OPTIONAL,

dft-S-OFDM-WaveformUL-IAB-r16               ENUMERATED {supported}              OPTIONAL,

-- R1 20-6: Support DCI Format 2_5 based indication of soft resource availability to an IAB node

dci-25-AI-RNTI-Support-IAB-r16              ENUMERATED {supported}              OPTIONAL,

-- R1 20-7: Support T_delta reception.

t-DeltaReceptionSupport-IAB-r16             ENUMERATED {supported}              OPTIONAL,

-- R1 20-8: Support of Desired guard symbol reporting and provided guard symbok reception.

guardSymbolReportReception-IAB-r16          ENUMERATED {supported}              OPTIONAL,

-- R1 18-8 HARQ-ACK codebook type and spatial bundling per PUCCH group

harqACK-CB-SpatialBundlingPUCCH-Group-r16   ENUMERATED {supported}              OPTIONAL,

-- R1 19-2: Cross Slot Scheduling

crossSlotScheduling-r16                     SEQUENCE {

non-SharedSpectrumChAccess-r16              ENUMERATED {supported}          OPTIONAL,

sharedSpectrumChAccess-r16                  ENUMERATED {supported}          OPTIONAL

}                                                                               OPTIONAL,

maxNumberSRS-PosPathLossEstimateAllServingCells-r16  ENUMERATED {n1, n4, n8, n16}         OPTIONAL,

extendedCG-Periodicities-r16                ENUMERATED {supported}              OPTIONAL,

extendedSPS-Periodicities-r16               ENUMERATED {supported}              OPTIONAL,

codebookVariantsList-r16                    CodebookVariantsList-r16            OPTIONAL,

-- R1 11-6: PUSCH repetition Type A

pusch-RepetitionTypeA-r16                   SEQUENCE {

sharedSpectrumChAccess-r16                  ENUMERATED {supported}          OPTIONAL,

non-SharedSpectrumChAccess-r16              ENUMERATED {supported}          OPTIONAL

}                                                                               OPTIONAL,

-- R1 11-4b: DL priority indication in DCI with mixed DCI formats

dci-DL-PriorityIndicator-r16                ENUMERATED {supported}              OPTIONAL,

-- R1 12-1a: UL priority indication in DCI with mixed DCI formats

dci-UL-PriorityIndicator-r16                ENUMERATED {supported}              OPTIONAL,

-- R1 16-1e: Maximum number of configured pathloss reference RSs for PUSCH/PUCCH/SRS by RRC for MAC-CE based pathloss reference RS update

maxNumberPathlossRS-Update-r16              ENUMERATED {n4, n8, n16, n32, n64}  OPTIONAL,

-- R1 18-9: Usage of the PDSCH starting time for HARQ-ACK type 2 codebook

type2-HARQ-ACK-Codebook-r16                 ENUMERATED {supported}              OPTIONAL,

-- R1 16-1g-1: Resources for beam management, pathloss measurement, BFD, RLM and new beam identification across frequency ranges

maxTotalResourcesForAcrossFreqRanges-r16    SEQUENCE {

maxNumberResWithinSlotAcrossCC-AcrossFR-r16 ENUMERATED {n2, n4, n8, n12, n16, n32, n64, n128}        OPTIONAL,

maxNumberResAcrossCC-AcrossFR-r16           ENUMERATED {n2, n4, n8, n12, n16, n32, n40, n48, n64, n72, n80, n96, n128, n256}

OPTIONAL

}                                                                               OPTIONAL,

-- R1 16-2a-4: HARQ-ACK for multi-DCI based multi-TRP – separate

harqACK-separateMultiDCI-MultiTRP-r16       SEQUENCE {

maxNumberLongPUCCHs-r16                         ENUMERATED {longAndLong, longAndShort, shortAndShort}    OPTIONAL

}                                                                               OPTIONAL,

-- R1 16-2a-4: HARQ-ACK for multi-DCI based multi-TRP – joint

harqACK-jointMultiDCI-MultiTRP-r16          ENUMERATED {supported}              OPTIONAL,

-- R4 9-1: BWP switching on multiple CCs RRM requirements

bwp-SwitchingMultiCCs-r16                   CHOICE {

type1-r16                                   ENUMERATED {us100, us200},

type2-r16                                   ENUMERATED {us200, us400, us800, us1000}

}                                                                               OPTIONAL

]],

[[

targetSMTC-SCG-r16                          ENUMERATED {supported}              OPTIONAL,

supportRepetitionZeroOffsetRV-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 11-12: in-order CBG-based re-transmission

cbg-TransInOrderPUSCH-UL-r16                ENUMERATED {supported}              OPTIONAL

]],

[[

-- R4 6-3: Dormant BWP switching on multiple CCs RRM requirements

bwp-SwitchingMultiDormancyCCs-r16           CHOICE {

type1-r16                                   ENUMERATED {us100, us200},

type2-r16                                   ENUMERATED {us200, us400, us800, us1000}

}                                                                               OPTIONAL,

-- R1 16-2a-8: Indicates that retransmission scheduled by a different CORESETPoolIndex for multi-DCI multi-TRP is not supported.

supportRetx-Diff-CoresetPool-Multi-DCI-TRP-r16               ENUMERATED {notSupported}          OPTIONAL,

-- R1 22-10: Support of pdcch-MonitoringAnyOccasionsWithSpanGap in case of cross-carrier scheduling with different SCSs

pdcch-MonitoringAnyOccasionsWithSpanGapCrossCarrierSch-r16   ENUMERATED {mode2, mode3}          OPTIONAL

]],

[[

-- R1 16-1j-1: Support of 2 port CSI-RS for new beam identification

newBeamIdentifications2PortCSI-RS-r16       ENUMERATED {supported}              OPTIONAL,

-- R1 16-1j-2: Support of 2 port CSI-RS for pathloss estimation

pathlossEstimation2PortCSI-RS-r16           ENUMERATED {supported}              OPTIONAL

]],

[[

mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16       ENUMERATED {supported}              OPTIONAL

]],

[[

srs-PeriodicityAndOffsetExt-r16				ENUMERATED {supported}				OPTIONAL

]]

}

=======================================================END OF CHANGES========================================================