3GPP TSG-RAN WG2 #119 electronic	R2-2207906

Online, 17th August – 26th August, 2022

Source:		NEC

Agenda item:		6.6.2 user plane common aspects

Title:		User plane issues for SDT

Document for:		Discussion and decision

# 1. Introduction

This contribution discusses on two user plane issues of CG-SDT, and provides TP for them.

# 2. Discussion

2.1 issue 1

At RAN2 #118e, to avoid unnecessary triggering of RA when none of the SSB is above the RSRP threshold for CG-SDT SSB selection, it was agreed that when none of the SSB is above the RSRP threshold for CG-SDT SSB selection, UE triggers legacy SR/RACH only when there is UL data available:

Agreement of RAN2 #118e

- When none of the SSB is above the RSRP threshold for CG-SDT SSB selection, UE triggers legacy SR/RACH when there is UL data available

However, the behaviour of checking whether there is UL data available is not captured in the spec:

1> else if PDCCH addressed to C-RNTI after the initial transmission of the CG-SDT with CCCH message has been received: 2> initiate Random Access procedure in clause 5.1.

Proposal 1: Capture RAN2 #118e agreement “When none of the SSB is above the RSRP threshold for CG-SDT SSB selection, UE triggers legacy SR/RACH when there is UL data available” in TS38.321 as in Annex1.

2.2 issue 2

In section 5.27.1 of TS38.321, for the determining whether SDT can be performed by MAC layer, an “A and B or C” architecture is used:

The MAC entity shall, if initiated by the upper layers for SDT procedure: 1> if the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold (A); and NOTE: For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation 1> if the RSRP of the downlink pathloss reference is higher than sdt-RSRP-Threshold (B); or 1> if sdt-RSRP-Threshold is not configured (C): 2> if the Serving Cell for SDT is configured with supplementary uplink as specified in TS 38.331 [5]; and 2> if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 3> select the SUL carrier. 2> else: 3> select the NUL carrier.

The “A and B or C” should be understand as “(A and B) or C”, since "and" and "or" have the same priority in logic description language. However the intended behaviour should be “A and (B or C)”. Readers without preliminary information can hardly understand the intended behaviour correctly based on the current spec text.

Observation: The current “A and B or C” in section 5.27.1 in TS 38.321 should be understood as “(A and B) or C”, however the intended behaviour is “A and (B or C)”.

Proposal 2: Change the “A and B or C” in section 5.27.1 of TS 38.321 to “A and (B or C)” as in Annex 2.

# 3. Conclusion

Proposal 1: Capture RAN2 #118e agreement “When none of the SSB is above the RSRP threshold for CG-SDT SSB selection, UE triggers legacy SR/RACH when there is UL data available” in TS38.321 as in Annex1.

Observation: The current “A and B or C” in section 5.27.1 in TS 38.321 should be understood as “(A and B) or C”, however the intended behaviour is “A and (B or C)”.

Proposal 2: Change the “A and B or C” in section 5.27.1 of TS 38.321 to “A and (B or C)” as in Annex 2.

# Annex 1

Proposed change to TS 38.321

### 5.8.2	Uplink

There are two types of transmission without dynamic grant:

<skipped>

For an uplink grant configured for configured grant Type 1 for CG-SDT on the selected uplink carrier as in clause 5.27, when CG-SDT is triggered and not terminated, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if, after initial transmission for CG-SDT with CCCH message has been performed according to clause 5.4.1, PDCCH addressed to the MAC entity's C-RNTI has not been received, and the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for initial transmission for CG-SDT with CCCH message (i.e., retransmission of initial transmission of CG-SDT):

2>	indicate the SSB index corresponding to the configured uplink grant to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

2>	if at least one SSB corresponding to the configured uplink grant with SS-RSRP above the cg-SDT-RSRP-ThresholdSSB is available:

3>	if this is the initial transmission of CG-SDT with CCCH message after the CG-SDT procedure is initiated as in clause 5.27 (i.e., initial transmission for CG-SDT):

4>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured grant.

3>	else if PDCCH addressed to C-RNTI has been received after the initial transmission of CG-SDT with CCCH message (i.e., subsequent new transmission for CG-SDT):

4>	if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is above cg-SDT-RSRP-ThresholdSSB and this SSB is associated with this configured grant:

5>	select this SSB.

4>	else:

5>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured grant.

3>	indicate the SSB index to the lower layer;

3>	consider this configured uplink grant as valid.

1>	else if PDCCH addressed to C-RNTI after the initial transmission of the CG-SDT with CCCH message has been received:

2> if at least one RB configured for SDT having data available for transmission:

3>	initiate Random Access procedure in clause 5.1.

# Annex 2

Proposed change to TS 38.321

### 5.27.1	General

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer. The SDT procedure can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold; and

NOTE:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than sdt-RSRP-Threshold, or

sdt-RSRP-Threshold is not configured:

2>	if the Serving Cell for SDT is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA of the configured grant Type 1 resource is valid according to clause 5.27.2; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for performing RA-SDT are selected according to clause 5.1.1b on the selected UL carrier:

3>	consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.