3GPP TSG-RAN WG2 #119 electronic	R2-2207908

Online, 17th August – 26th August, 2022

Source:		NEC

Agenda item:		8.13.3      Miscellaneous SON MDT enhancements

Title:		SON/MDT enhancements for RACH enhancements

Document for:		Discussion and decision

# 1. Introduction

At RAN #95e meeting, one new WI on further enhancements of SON/MDT was approved for Rel-18. One of the objective is to support SON/MDT enhancements for RACH report [1]:

- Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PScell change report

- NPN

- RACH report

- fast MCG recovery

In this paper, we provide our view that in Rel-18 RACH report enhancement, on which RACH enhancements features we should focus.

# 2. Discussion

In Rel-17, enhancements for random access procedure are introduced by multiple WIs:

- RA partition

To configure dedicated random access resources for feature(s), e.g. SDT and Redcap, a set of RACH resources can be associated with a feature combination, which is also known as RA partition in Rel-17. UE selects a set of RACH resources (i.e. a RA partition) based on the features applicable to the current RA procedure. And it is possible that one feature is applicable to the current RA procedure, however not associated with the set of RACH resources.

- Msg3 repetition

To support coverage enhancement, aggregation of multiple slots with TB repetition for MSG3 transmission is supported in Rel-17. The UE requests MSG3 repetition via separate RACH resources (RA partition).  In order to determine whether Msg3 repetition is applicable to the random access procedure, UE needs to check if the RSRP of the downlink pathloss reference is less than a configured threshold.

- SCG activation/deactivation

In Rel-17, SCG activation/deactivation mechanism is introduced for power saving purpose, wherein a new triggering condition for RA is introduced. Upon reception of SCG activation indication from MN, UE may need to initiate RA on the PSCell of the SCG. In order to determine whether to perform RA, the UE may need to check whether TAT timer is still running or beam failure happens.

- BFR recovery for two BFD-RS sets

In Rel-17 feMIMO, two BFD-RS sets can be configured for the beam failure detection for one serving cell. If beam failure is detected for two BFD-RS sets, Random Access procedure can be initiated for the beam failure recovery of two BFD-RS sets.

The current RA related information stored and reported by the UE does not cover the new Rel-17 RACH features. Therefore, it is proposed that RAN2 considers to store and report RA related information needed with regarding the above RACH enhancements in Rel-17.

Proposal: RAN2 considers to store and report RA related information with regarding the following RACH enhancement in Rel-17

- RACH partition

- Msg3 repetition

- SCG activation/deactivation

- BFR recovery for two BFD-RS sets

# 3. Conclusion

Proposal: RAN2 considers to store and report information with regarding the following RACH enhancement in Rel-17

- RACH partition

- Msg3 repetition

- SCG activation/deactivation

- BFR recovery for two BFD-RS sets

# Reference

[1] RP-221000, Revised WID: Further enhancement of data collection for SON (Self- Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)