3GPP TSG-RAN WG2 #119 electronic	R2-2207909

Online, 17th August – 26th August, 2022

Source:		NEC

Agenda item:		8.13.3      Miscellaneous SON MDT enhancements

Title:		Discussion on successful PSCell change report

Document for:		Discussion and decision

# 1. Introduction

At RAN #95e meeting, one new WI on further enhancements of SON/MDT was approved for Rel-18. One of the objective is to support SON/MDT enhancements for successful PSCell change report

- Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PScell change report

- NPN

- RACH report

- fast MCG recovery

In this paper, we provide our initial consideration on successful PSCell change report.

# 2. Discussion

In Rel-17, successful handover report (SHR) is introduced to report underlying issue of one successful completed handover procedure. The content of SHR can be considered as baseline for successful PSCell change report (SPCR). So we propose:

Proposal 1: the following content of should be included for SPCR:

- source PSCell ID

- target PSCell ID

- measurement results

- location information

- random access information

- cause of SPCR

For SHR, to avoid unnecessary storage and report of SHR, the UE is configured with triggering conditions to compile the SHR, hence the store of report would be triggered only if the conditions are met. This should be reused for SPCR

Proposal 2: same as SHR, network configures triggering conditions for SPCR, and UE only stores SPCR information when triggering condition is fulfilled.

The triggering condition for SHR including:

- the elapsed time of the timer T304 is greater than a threshold

- the elapsed time of the timer T310 is greater than a threshold

- the elapsed time of the timer T312 is greater than a threshold

- RLF occurred at the source PCell during the DAPS handover

Except the last one, since DAPS is not supported for PSCell change, the other three triggering conditions can be reused for SPCR.

Proposal 3: the following triggering conditions can be supported for SPCR:

- the elapsed time of the timer T304 is greater than a threshold

- the elapsed time of the timer T310 is greater than a threshold

- the elapsed time of the timer T312 is greater than a threshold

PSCell change can be categorized as MN-initiated PSCell change and SN-iniated PSCell change. For MN-initiated PSCell change, MN triggers the PSCell change procedure and decides the target PSCell; while for SN initiated PSCell change, SN triggers the PSCell change procedure and decides the target PSCell. RAN2 needs to discuss how network configures and how UE reports SPCR for the two cases.

Proposal 4: RAN2 discuss how to configure and how to report SPCR for MN-initiated PSCell change and SN-initiated PSCell change.

# 3. Conclusion

In this paper, we provide our initial consideration on successful PSCell change, and have the following proposals:

Proposal 1: the following content of should be included for SPCR.

- source PSCell ID

- target PSCell ID

- measurement results

- location information

- random access information

- cause of SPCR

Proposal 2: same as SHR, network configures triggering conditions for SPCR, and UE only stores SPCR information when triggering condition is fulfilled.

Proposal 3: the following triggering conditions can be supported for SPCR:

- the elapsed time of the timer T304 is greater than a threshold

- the elapsed time of the timer T310 is greater than a threshold

- the elapsed time of the timer T312 is greater than a threshold

Proposal 4: RAN2 discuss how to configure and how to report SPCR for MN-initiated PSCell change and SN-initiated PSCell change.

# Reference

[1] RP-221000, Revised WID: Further enhancement of data collection for SON (Self- Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)