3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2207920

Online, 17th August – 26th August, 2022

Agenda item:		8.3.2

Source:		Lenovo

Title:		Discussion on the state transition in NES

Document for:		Discussion and Decision

- Introduction

In RAN1 #109-e meeting, some agreements from the perspective of time domain were achieved, including activation request by the UE, determination for state transition, etc.:

Further study techniques and enhancements for increasing time domain energy saving opportunities by the gNB, including (but not limited to) the following aspects:

- potential methods of reducing/adapting transmission/reception of common channels/signals, e.g. SSB, SIB1, other SI, paging, PRACH, and its impact to initial access procedure, cell (re)selection, handover, synchronization and measurements performed by the idle/inactive/connected UE;

  - potential methods of reducing transmission/reception of common channels/signals can include no- or reduced-transmission/reception, increased periodicity, enablement of on-demand transmission/reception of common channels/signals, or offloading of common channels/signals to other carriers or use of light or relaxed versions of common channels /signals

- potential methods of reducing/adapting transmission/reception of periodic and semi-persistent signals and channels configuration such as CSI-RS, group-common/UE-specific PDCCH, SPS PDSCH, PUCCH carrying SR, PUCCH/PUSCH carrying CSI reports, PUCCH carrying HARQ-ACK for SPS, CG-PUSCH, SRS, positioning RS (PRS), etc.

- semi-static and/or dynamic cell on/off in one or more granularity, e.g. /subframe/slot/symbol; some examples are:

  - Cell/network node activation request by the UE, for example using signal/channel from UE for gNB’s wake-up request

  - enhancements to L1/L2 based mobility to efficiently enable a network node (e.g. TRP, repeater) on/off operation within a cell (within network energy saving SI scope)

  - signaling enhancements for indication of semi-static and/or dynamic cell/subframe/slot/symbol on/off duration

- support of periodic and/or on-demand reference signal(s) from the gNB to aid discovery of a cell;

- dynamic adaptation of UE C-DRX configurations in a UE-group or cell-specific manner

- Mechanism to utilize potential energy saving states or sleep modes and the transition between states from leveraging cell on/off opportunities

  - including studies of waking up gNB due to user traffic, or user density, or gNB receiving wake up signal

  - including technique to allow discovery and measurement of cells in sleep or dormant states

- UE assistant information facilitating BS time domain adaptation

Note: For all techniques above, study of time domain techniques is applicable for single component carrier and multi-component carrier cases. Use of UE grouping and its interaction with proposed techniques can be considered.

In this contribution, we discuss potential methods to support the state transition in NES.

- Discussion

In LTE, the eNB using cell on/off for the purpose of energy saving may adaptively turn the downlink transmission of a cell on and off. In NR, it would be useful to define different energy saving states or sleep modes for the base stations considering the difference between the eNB and gNB, also considering the diversity of the traffics in NR. Correspondingly, it would be better to support a cell to transition between any two energy saving states for the purpose of maximizing energy savings and reducing the transition time.

Proposal 1: a cell can transition among multiple different energy saving states.

Referring to the progress of RAN1, cell/network node activation request by the UE was agreed. And the requirements of the arrival traffic are identified only by the UE before the BSR or data is transmitted. Accordingly, the UE may expect one of the activation levels of a cell can satisfy the traffic requirements. For example, the UE expects the level of activation of the cell based on the data volume or whether it is the specific traffic, etc. Therefore, it is better for the UE to request the cell to transition to the suitable energy saving state. And the suitable energy saving state may be any of the energy saving states supported by the cell.

Proposal 2: the state transition to each of the potential network energy saving states can be request by the UE.

Different UE may have different criteria for the expected level of activation. It is difficult for the gNB to determine the state transition once the request from the UE is received if it is up to the UE implementation to decide which level of activation is requested. Therefore, if the UE can be configured with the threshold for requesting the activation of the cell, the network node can determine the state transition simply. Further, each configured threshold corresponds to a level of activation of the cell.

Proposal 3: the UE can be configured with different thresholds for requesting different energy saving states.

The network state transition for energy saving should be considered comprehensively including the UL and DL situations of all the camped and connected UEs. In LTE, the decision may be taken by the base station or a central controller. Accordingly, the state transition in Rel-18 NES may be decided by the gNB or CN. Then, the gNB can indicate the energy saving state transition to the UEs, for example the cell is activated to a specific activation level.

Proposal 4: the gNB indicates the energy saving state transition to the UEs.

- Conclusion

In this contribution, we discuss the potential enhancements related to the energy saving state transition in Rel-18 NR and provide the proposals:

Proposal 1: a cell can transition among multiple different energy saving states.

Proposal 2: the state transition to each of the potential network energy saving states can be request by the UE.

Proposal 3: the UE can be configured with different thresholds for requesting different energy saving states.

Proposal 4: the gNB indicates the energy saving state transition to the UEs.

References

RP-213554, “Study on network energy savings for NR”.

RAN1 #109-e meeting chairman notes.